Hanwha Vision is a global leading supplier of solutions for IP and analog video surveillance. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-effectiveness. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, https://hanwhavisionamerica.com

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: Hanwha Vision (https://hanwhavisionamerica.com)

    - Model: ACV-8081R

    - Alternates: None

  - GENERAL DESCRIPTION

    - Video – The camera shall have the following properties relating to the video signals it produces.

      - Max. 30fps (@5MP Max. 20fps)

      - Video signals are output in AHD/TVI/CVI/CVBS (selectable) format via BNC terminal.

      - Video shall be able to be transmitted up to 300 meters with 5c2v cable.

    - Camera – The camera device shall have the following physical and performance properties:

      - True day/night operation with removable IR cut filter

      - Low light level operation to 0.06 lux at F1.6, 1/30sec in color mode and 0 lux in black and white mode with IR LED on.

      - The camera shall be able to produce clear images in highly contrast scenes with digital wide dynamic range.

      - The camera shall support digital noise reduction using SSNR Ⅳ technology.

    - The camera shall be controlled by remote coax protocol.

    - IP66 and IK10 supported

  - DETAILED SPECIFICATIONS

    - Video

      - Imaging device: 1/2.8" CMOS

      - Resolution: 2592 x 1944, 2560 X 1440

      - Max. Framerate: Max. 30fps (@5MP Max. 20fps)

      - Minimum illumination: Color 0.06Lux (F1.6, 1/30sec), BW 0Lux (IR LED on)

      - Video Output: BNC (AHD / TVI / CVI / CVBS selectable)

    - Lens:

      - Focal length: 3.5mm ~ 10.5mm (3x) motorized varifocal

      - Max. Aperture Ratio: F1.6

      - Field of View:

        - H: 82° (Wide) ~ 27° (Tele)

        - V: 60° (Wide) ~ 20° (Tele)

        - D: 105° (Wide) ~ 33° (Tele)\

      - Min. Object Distance: 0.8m(2.62ft)

    - Operational Functions

      - Day/Night Setting: Auto (ICR)

      - Backlight Compensation: BLC (Off / On)

      - WDR: D-WDR

      - Digital Noise Reduction: SSNRⅣ

      - Gain Control: Off / Low / Middle / High

      - White Balance: ATW / AWC

      - Electronic Shutter Speed: Auto (1/30 sec ~ 1/50,000 sec)

      - Video Rotation: Mirror / V-Flip / Rotate

      - IR Viewable Length: 25m (82.02ft)

      - Remote Control Interface: Coaxial

      - Coaxial Protocol: ACP (AHD Coax Protocol), TCP (TVI Coax Protocol), CCP (CVI Coax Protocol), CVBS (Pelco-C Coaxitron)

      - Video Transmission Distance: 300m (984.25 ft.) (5C2V coaxial cable)

    - Electrical

      - Input Voltage:  DC 12V

      - Power Consumption: Max 7.2W, Typical 5.8W

    - Mechanical

      - Color/Material: White / Aluminum / Plastic

      - Dimensions: ø132 x 103 mm (ø5.20" x 4.06")

      - Weight: 550g (1.21 lb)

    - Environmental

      - Operating Temperature / Humidity: -30°C ~ +55°C (-22°F ~ +131°F) / Less than 90% RH

      - Storage Temperature / Humidity: -30°C ~ +60°C (-22°F ~ +140°F) / Less than 90% RH

      - Certification: IP66, IK10

    - EMC & Safety

      - FCC

        - 47 CFR Part 15 Subpart B Class A

        - ICES-3(A) / NMB-3(A)

      - UL 62368-1, CAN/CSA C22.2 NO. 62368-1, IEX 62471

END OF SECTION

10