Hanwha Techwin is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://security.hanwhatechwin.com/

- PRODUCTS

  - EQUIPMENT

    - Manufacturer : Hanwha Techwin (http://security.hanwhatechwin.com/)

    - Model : ANO-L6012R

    - Alternates :	None

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously

        - H.265 and H.264 – Max. 30fps/25fps(60Hz/50Hz)

        - MJPEG – Max. 10fps

      - The camera shall be able to configure up to 3 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - Resolution selections

1920x1080, 1280x960, 1280x720, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360

      - Simultaneous unicast access by up to 6 users

      - Multicast or unicast capable

      - Dynamic DNS (DDNS) support.

      - WiseStream and Dynamic GOV to efficiently manage bitrate of the video stream.

    - Camera – The camera device shall have the following physical and performance properties:

      - IP66 rated for protection against impacts

      - Auto day/night operation with removable IR cut filter

      - 2D and 3D digital noise reduction

      - Integral IR illumination, providing effective visibility of 30m(98.43ft) at 0 Lux when activated in Black & White mode

      - 6 privacy masking regions utilizing quadrangle

    - Intelligence and Analytics – The camera shall have a suite of integral intelligent operations and analytic functions to include:

      - Motion detection with 4 definable detection areas, minimum / maximum object size definition that ignores false alarms such as trees and waves on water.

      - Detection of logical events of specified conditions from the camera’s video input

        - Analytics events: Motion detection, Tampering, Virtual area(Intrusion/Enter/Exit), Virtual line(Crossing/Direction)

    - Interoperability – The camera shall be ONVIF Profile S / G / T compliant.

    - The camera shall possess the following further characteristics:

      - Built-in web server, accessed via standard browsers including MS Edge, Firefox, Chrome and Safari

      - Micro SD/SDHC memory card with configurable pre-alarm and post-alarm recording intervals

      - Alarms and notifications

        - alarm notification triggers:

          - video analytics

        - available notification means upon trigger:

          - file upload via FTP and e-mail

          - notification via e-mail

          - record to local storage (SD/SDHC card)

      - Pixel Counter available in the plug-in web viewer.

      - POE capable

  - DETAILED SPECIFICATIONS

    - Video

      - Image

        - Sensor : 1/2.8" 2M CMOS

          - pixels per sensor : 1920 (H) x 1080 (V)

          - scanning : progressive

        - Minimum illumination

          - Color mode : 0.03Lux(F2.0, 1/30sec)

          - Black & white mode (IR LED off) : 0.003Lux(F2.0, 1/30sec)

          - Black & white mode (IR LED on) : 0Lux

        - The following features with control settings shall be available:

          - Camera Title

            - Off / On (Displayed up to 15 characters)

          - Day/night : Auto (ICR)

          - Backlight compensation : BLC, WDR, SSDR

          - WDR : 120dB

          - Digital Noise Reduction (DNR) : SSNR

          - Motion Detection : Off / On (4ea, rectangular zones)

          - Privacy Masking : Off / On (6ea, rectangular zones)

          - Gain Control : Low / Middle / High

          - White Balance : ATW / AWC / Manual / Indoor / Outdoor

          - Lens Distortion Compensation(LDC) : Support

          - Electronic shutter speed : Minimum / Maximum / Anti flicker (2 ~ 1/12,000sec)

          - Image flip : Off / On

          - Image mirror : Off / On

          - Hallway view : Off / On (90° / 270°)

          - Analytics

            - Motion detection, Tampering, Virtual area(Intrusion/Enter/Exit), Virtual line(Crossing/Direction)

          - Alarm Triggers : Analytics

          - Alarm Events

            - File upload via FTP and e-mail

            - Notification via e-mail

            - SD/SDHC recording at Event Triggers

        - Lens : 2.8mm fixed focal

          - Max. Aperture Ratio : F2.0

          - FoV

            - H : 113.7˚ / V : 61.5˚ / D : 134.4˚

          - Min. Object Distance : 0.5m (1.64ft)

          - Focus Control : Fixed

        - DORI (EN62676-4 standard)

          - Detect : 25.1m(82.28ft)

          - Observe : 10.0m(32.91ft)

          - Recognize : 5.0m(16.46ft)

          - Identify : 2.5m(8.23ft)

      - IR Viewable Length : 30m (98.43ft)

      - Video Streams

        - The camera shall be able to produce 3 video profiles, each of which may have the following properties:

          - Encoding type : H.265, H.264, MJPEG

          - Resolution:

            - 1920x1080, 1280x960, 1280x720, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360

          - Frame rate

            - H.265 and H.264: Max. 30fps/25fps(60Hz/50Hz)

            - MJPEG: Max. 10fps

          - Smart Codec: WiseStream Ⅱ

          - Bit rate control

            - H.265 and H.264 : target bitrate level control, constant bit rate (CBR) or variable bit rate (VBR)

            - MJPEG : quality level control, variable bit rate (VBR)

      - Number of multi-streaming profiles : 3 profiles

      - Simultaneous users (total) : 6 maximum (unicast)

      - Storage and Recording

        - The camera shall have onboard SD card storage.

          - Card type : Micro SD/SDHC

          - Capacity : Up to 32GB (1 slot)

          - Image content on the card shall have the ability to be downloaded to a selected destination.

      - Interoperability - Video streams shall be capable of supporting ONVIF protocol, profiles S / G / T.

      - Single Images - The camera shall support jpg file image screenshot and export.

      - Memory : 512MB RAM, 256MB Flash

    - Network

      - Connectivity : RJ-45(10/100BASE-T)

      - Protocols supported :

        - Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

        - Configuration: Dynamic Host Configuration Protocol (DHCP)

        - Web services: Hypertext Transfer Protocol (HTTP), Secure HTTP (HTTPS)

        - Security: Secure Sockets Layer(SSL)/Transport Layer Security(TLS)

        - Network services: Address Resolution Protocol (ARP), Bonjour, Domain Name System (DNS), Internet Control Message Protocol (ICMP), Network Time Protocol (NTP), Simple Network Management Protocol (SNMP v1/2c/3 – MIB-2), Universal Plug and Play (UPnP)

        - Media: Real-Time Transport Protocol (RTP), Real-Time Control Protocol, Real-Time Streaming Protocol (RTSP)

        - Multicast: Internet Group Management Protocol (IGMP)

        - Notifications: File Transfer Protocol (FTP), Simple Mail Transfer Protocol (SMTP)

      - DDNS

        - The camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

      - Quality of Service (QoS)

        - Layer 3 DSCP

      - Security features

        - HTTPS(SSL) Login Authentication

        - Digest Login Authentication

        - IP Address Filtering

        - User access log

        - 802.1X Authentication(EAP-TLS, EAP-LEAP)

      - Application Programming Interface

        - ONVIF Profile S / G / T, SUNAPI(HTTP API)

      - Discovery

        - Manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.

    - Electrical

      - Input Voltage : PoE (IEEE802.3af, Class2)

      - Power consumption

        - PoE : Max 6.4W, Typical 5.0W

    - Mechanical And Environmental

      - Material : Plastic

      - Dimensions  : Ø 78.0 x 262.0mm (Ø 3.07 x 10.31”)

      - Weight : 390g (0.86lb)

      - Temperature And Humidity

        - Operating : -30°C ~ +55°C(-22°F ~ +131°F) / Less than 95% RH
* Start up should be done at above -20°C(-4°F)

        - Storage : -30°C ~ +55°C(-22°F ~ +131°F) / Less than 95% RH

      - Environmental Rating

        - IP66

      - RAL Code

        - RAL9003

    - EMC & Safety

      - FCC 47 CFR Part 15 Subpart B

        - ANSI C63.4a-2017 Class A

      - IC Regulation ICES-003 Issue 7

        - ANSI C63.4a-2017 Class A

      - UL listed

END OF SECTION

8