Hanwha Techwin is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://security.hanwhatechwin.com/

- PRODUCTS

  - EQUIPMENT

    - Manufacturer : Hanwha Techwin (http://security.hanwhatechwin.com/)

    - Model : PNM-7082RVD

    - Alternates :	None

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously

        - H.265 and H.264 : Max. frame rates to 30fps/25fps(60Hz/50Hz)

        - MJPEG : Max. frame rates to 30fps/25fps(60Hz/50Hz)

      - The camera shall be able to configure up to 10 independent video stream per channel profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - Resolution selections : 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

      - Simultaneous unicast access by up to 20 users

      - Multicast or unicast capable

      - Dynamic DNS (DDNS) support.

      - WiseStream and Dynamic GOV to efficiently manage bitrate of the video stream.

    - Camera – The camera device shall have the following physical and performance properties:

      - IP66, IK10, NEMA4X rated for protection against impacts

      - Auto day/night operation with removable IR cut filter

      - 2D and 3D digital noise reduction

      - Integral IR illumination, providing effective visibility of 15m(49.21ft) based on scene at 0 Lux when activated in Black & White mode

      - 32 privacy masking regions utilizing quadrangle

    - Intelligence and Analytics – The camera shall have a suite of integral intelligent operations and analytic functions to include

      - Motion detection with eight definable detection areas, minimum / maximum object size definition.

      - Detection of logical events of specified conditions from the camera’s video input

        - Analytics events: Defocus detection, Motion detection, Tampering, Fog detection, Shock detection, Virtual area (Appear / Disappear) *Support Audio detection, Sound classification via optional SPM-4210 I/O box

    - Interoperability – The camera shall be ONVIF Profile S / T compliant.

    - The camera shall possess the following further characteristics:

      - Built-in web server, accessed via standard browsers including MS Edge, Firefox, Chrome and Safari

      - Micro SD/SDHC/SDXC memory card and NAS recording options, with configurable pre-alarm and post-alarm recording intervals

      - Alarms and notifications

        - Alarm notification triggers:

          - Alarm input via optional SPM-4210 I/O box

          - Video analytics

          - Network disconnect

        - Available notification means upon trigger:

          - File upload via FTP and e-mail

          - Notification via e-mail

          - Record to local storage (SD/SDHC/SDXC card)

          - Handover(PTZ Preset, Send message by HTTP/HTTPS/TCP)

          - Alarm output via optional SPM-4210 I/O box

      - Pixel Counter available in the plug-in web viewer.

      - POE+ capable

      - NDAA Compliant

  - DETAILED SPECIFICATIONS

    - Video

      - Image

        - Sensor : 1/2.8" 2M CMOS x 2CH

          - Pixels per sensor : 1920 (H) x 1080 (V)

          - Scanning : progressive

        - Minimum illumination

          - Color mode : 0.035Lux(F2.2, 1/30sec, 30IRE)

          - Black & white mode (IR LED off) : 0.0035Lux(F2.2, 1/30sec, 30IRE)

          - Black & white mode (IR LED on) : 0Lux

        - The following features with control settings shall be available:

          - Camera Title

            - Off / On (Displayed up to 85 characters)

            - English / Numeric / Special characters

            - Multi-line (Max. 5lines), Color (Grey / Green / Red / Blue / Black / White), Transparency, Size (Small, Medium, Large)

          - Day/night : Auto (ICR)

          - Backlight compensation : BLC, WiseDR, HLC

          - WDR : extremeWDR(150dB)

          - Digital Noise Reduction (DNR) : WiseNR, SSNRⅤ

          - Defog : Support

          - Motion Detection : Off / On (8ea, 8 point polygonal zones)

          - Privacy Masking : Off / On (32ea, quadrangle zones)

            - Color : Gray / Black / White

          - Gain Control : Support

          - White Balance : ATW / AWC / Manual / Indoor / Outdoor

          - Electronic shutter speed : Minimum / Maximum / Anti flicker (1/5 ~ 1/12,000sec)

          - Image flip : Off / On

          - Image mirror : Off / On

          - Hallway view : Off / On (90° / 270°), Each channel separately

          - Analytics

            - Analytics events*: Defocus detection, Motion detection, Tampering, Virtual area (Appear / Disappear) *Support Audio detection, Sound classification via optional SPM-4210 I/O box

          - Alarm I/O : Support Alarm via optional SPM-4210 I/O box

          - Alarm Triggers : Analytics, Network disconnect, Alarm input via optional SPM-4210 I/O box

          - Alarm Events

            - File upload via FTP and e-mail

            - Notification via e-mail

            - SD/SDHC/SDXC or NAS recording at Event Triggers

            - Handover(PTZ Preset, Send message by HTTP/HTTPS/TCP)

            - Alarm output via optional SPM-4210 I/O box

          - Video Out for installation

            - USB: micro USB Type B, 1280x720

        - Lens : 3 ~ 6mm (Optical 2x) motorized varifocal

          - Max. Aperture Ratio : F2.2(Wide) ~ F3.1(Tele)

          - FoV

            - H : 107˚(Wide) ~ 56.3˚(Tele)

            - V : 57˚(Wide) ~ 31.5˚(Tele)

            - D : 126˚(Wide) ~ 64.3˚(Tele)

          - Min. Object Distance : 0.5m (1.64ft)

          - Lens Type : Fixed iris

          - Focus Control : Simple focus

        - Pan /Tilt / Rotate

          - 0 ~ 355 ˚  / 0 ~ 75 ˚  / 0 ~ 180 ˚

        - DORI (EN62676-4 standard)

          - Detect : Wide: 28.4m(93.22ft) / Tele: 71.8m(235.45ft)

          - Observe : Wide: 11.4m(37.29ft) / Tele: 28.7m(94.18ft)

          - Recognize : Wide: 5.7m(18.64ft) / Tele: 14.4m(47.09ft)

          - Identify : Wide: 2.8m(9.32ft) / Tele: 7.2m(23.55ft)

      - IR Viewable Length : 25m(65.62ft) based on scene

      - Video Streams

        - The camera shall be able to produce 5 video profiles per channel, each of which may have the following properties:

          - Encoding type : H.265, H.264, MJPEG

          - Resolution : 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

          - Frame rate

            - H.265 and H.264 : Max. 30fps/25fps(60Hz/50Hz)

            - MJPEG : Max. 30fps/25fps(60Hz/50Hz)

          - Smart Codec : Manual(5ea area), WiseStream Ⅱ

          - Bit rate control

            - H.265 and H.264 : target bitrate level control, constant bit rate (CBR) or variable bit rate (VBR)

            - MJPEG : quality level control, variable bit rate (VBR)

      - Number of multi-streaming profiles : 10 profiles per channel

      - Simultaneous users (total) : 20 maximum (unicast)

      - Storage and Recording

        - The camera shall have onboard SD card storage.

          - Card type : Micro SD/SDHC/SDXC

          - Capacity : Up to  512GB (1 slot)

          - Image content on the card shall have the ability to be downloaded to a selected destination.

        - NAS

      - Interoperability - Video streams shall be capable of supporting ONVIF protocol, profiles S / T.

      - Single Images - The camera shall support jpg file image screenshot and export.

      - Memory : 2GB RAM, 512MB Flash

    - Network

      - Connectivity : Metal shielded RJ-45(10/100/1000BASE-T)

      - Protocols supported :

        - Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

        - Configuration: Dynamic Host Configuration Protocol (DHCP)

        - Web services: Hypertext Transfer Protocol (HTTP), Secure HTTP (HTTPS)

        - Security: Secure Sockets Layer(SSL)/Transport Layer Security(TLS)

        - Network services: Address Resolution Protocol (ARP), Bonjour, Domain Name System (DNS), Internet Control Message Protocol (ICMP), Link Layer Discovery Protocol(LLDP), Network Time Protocol (NTP), Protocol Independent Multicast-Sparse Mode (PIM-SM), Simple Network Management Protocol (SNMP v1/2c/3 – MIB-2), Universal Plug and Play (UPnP)

        - Media: Real-Time Transport Protocol (RTP), Real-Time Control Protocol, Real-Time Streaming Protocol (RTSP), Secure Real-Time Transport Protocol(SRTP)

        - Multicast: Internet Group Management Protocol (IGMP)

        - Notifications: File Transfer Protocol (FTP), Simple Mail Transfer Protocol (SMTP)

      - DDNS

        - The camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

      - Quality of Service (QoS)

        - Layer 3 DSCP

      - Security features

        - TPM 2.0 (FIPS 140-2 level 2)

        - HTTPS(SSL) Login Authentication

        - Digest Login Authentication

        - IP Address Filtering

        - User access log

        - 802.1X Authentication(EAP-TLS, EAP-LEAP, EAP-PEAP MSCHAPv2)

        - Device Certificate(Hanwha Techwin Root CA, pre-installed)

        - Secure by default certificate

        - Secure OS/Boot/Storage, Verify firmware forgery

      - Application Programming Interface

        - ONVIF Profile S / T, SUNAPI(HTTP API), Wisenet open platform

      - Discovery

        - Manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.

    - Audio

      - Audio In via optional SPM-4210 I/O box

      - Audio Out via optional SPM-4210 I/O box

    - Electrical

      - Input Voltage : PoE+ (IEEE802.3at, Class4)

      - Power consumption

        - PoE+ : Max 20W, Typical 15.5W

    - Mechanical And Environmental

      - Material : Aluminum, Hard-coated dome bubble

      - Dimensions (W x D x H)  : 215 x 135 x 93.2mm (8.46 x 5.31 x 3.67”)

      - Weight : 1330g (2.86lb)

        - Temperature And Humidity1Operating : -40°C ~ +55°C(-40°F ~ +131°F) / Less than 90% RH (Non-condensing)

        - Storage : -50°C ~ +60°C(-58°F ~ +140°F) / Less than 90% RH (Non-condensing)

      - Environmental Rating

        - IP66

        - IK10

        - NEMA4X

      - RAL Code

        - RAL9003

      - Compatible Conduit hole / Gangbox

        - 3/4" (M25)

        - single, double, 4" octagon, 4" square

    - EMC & Safety

      - FCC 47 CFR Part 15 Subpart B

        - ANSI C63.4-2014 Class A

      - IC Regulation ICES-003 Issue 7

        - ANSI C63.4a-2017 Class A

      - CE EMC-Directive 2014/30/EU

        - EN 55032:2015/A11:2020

        - EN 50130-4:2011

      - UK EMC-Regulations 2016/1091

        - BS EN 55032:2015/A11:2020 Class A

        - BS EN 50130-4:2011/a1:2014

      - NEMA 250-2014

      - VCCI-CISPR 32:2016 Class A

      - AS/NZS CISPR32:2015 Class A

      - UL listed

      - CE EN 50581:2012 (RoHS)

      - BS EN 50581:2012 (RoHS)

END OF SECTION

8