Hanwha Techwin is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://security.hanwhatechwin.com/

- PRODUCTS

  - EQUIPMENT

    - Manufacturer : Hanwha Techwin (http://security.hanwhatechwin.com/)

    - Model : PNM-C9022RV

    - Alternates :	None

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously

        - H.265 and H.264 – frame rates up to 20fps (60Hz/50Hz)

        - MJPEG – frame rates up to 20fps (60Hz/50Hz)

      - The camera shall be able to configure up to 5 independent video stream per channel with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - Resolution selections

5120x1568, 3468x1152, 2560x784, 1280x392

      - Simultaneous unicast access by up to 20 users

      - Multicast or unicast capable

      - Dynamic DNS (DDNS) support.

      - WiseStreamIII to efficiently manage bitrate of the video stream.

    - Camera – The camera device shall have the following physical and performance properties:

      - IP66, IK10, NEMA 250 type 4X rated for protection against impacts

      - Auto day/night operation with removable IR cut filter

      - 2D and 3D digital noise reduction

      - Integral IR illumination, providing effective visibility of 20m(65.62ft) based on scene at 0 Lux when activated in Black & White mode

      - 10 privacy masking regions utilizing quadrangle

    - Intelligence and Analytics – The camera shall have a suite of integral intelligent operations and analytic functions to include:

      - Motion detection with eight definable detection areas, minimum / maximum object size definition and a learning algorithm that ignores false alarms such as trees and waves on water.

      - Detection of logical events of specified conditions from the camera’s video input

        - Classified object type : Person / Vehicle

        - Attributes: Not supported

        - Analytics events based on AI engine: Object detection, Virtual line (Crossing / Direction), Virtual area (Loitering / Intrusion / Enter / Exit)

        - Analytics events: Defocus detection, Motion detection, Tampering, Audio detection, Sound classification, Virtual area (Appear / Disappear)

    - Interoperability – The camera shall be ONVIF Profile S / G / T / M compliant.

    - The camera shall possess the following further characteristics:

      - Micro SD/SDHC/SDXC memory card with configurable pre-alarm and post-alarm recording intervals

      - NAS recording option with configurable pre-alarm and post-alarm recording intervals

      - Alarms and notifications

        - alarm notification triggers:

          - alarm input

          - video analytics

          - network disconnect

        - available notification means upon trigger:

          - file upload via FTP and e-mail

          - notification via e-mail

          - record to local storage (micro SD/SDHC/SDXC card)

          - Alarm output

          - Handover(PTZ Preset, Send message by HTTP/HTTPS/TCP)

          - Audio clip playback

      - Pixel Counter available in the plug-in web viewer.

      - POE+ capable

      - NDAA Compliant

  - DETAILED SPECIFICATIONS

    - Video

      - Image

        - Sensor : 1/2.8" CMOS x 4CH

          - scanning : Progressive

          - Minimum illumination

        - Minimum illumination

          - Color mode : 0.1Lux(F2.0, 1/30sec)

          - Black & white mode (IR LED off) : 0.01Lux(F2.0, 1/30sec)

          - Black & white mode (IR LED on) : 0Lux

        - The following features with control settings shall be available

          - Camera Title

            - Off / On (Displayed up to 85 characters)

            - English / Numeric / Special characters

            - Multi-line (Max. 5lines), Color (Grey / Green / Red / Blue / Black / White), Transparency, Size (Small, Medium, Large)

          - Day/night : Auto (ICR)

          - Backlight compensation : Off / BLC / WiseDR

          - WDR: extreme WDR(120dB)

          - Digital Noise Reduction : WiseNRⅡ(Based on AI engine), SSNR Ⅴ

          - Motion Detection : Off / On (8ea, 8 point polygonal zones)

          - Privacy Masking : Off / On (10ea, quadrangle zones)

            - Color : Gray / Green / Red / Blue / Black / White

          - Gain Control : Low / Middle / High

          - White Balance : ATW / AWC / Manual / Indoor / Outdoor

          - Electronic shutter speed : Minimum / Maximum / Anti flicker (2 ~ 1/12,000sec), Auto prefer shutter control (Based on AI engine)

          - Image Flip : Off / On

          - Image mirror : Off / On

          - Analytics

            - Classified object type : Person / Vehicle

            - Attributes : Not supported

            - Support Bestshot

            - Analytics events based on AI engine : Object detection, Virtual line (Crossing / Direction), Virtual area (Loitering / Intrusion / Enter / Exit)

            - Analytics events : Defocus detection, Motion detection, Tampering, Audio detection, Sound classification, Virtual area (Appear / Disappear)

          - Alarm I/O : 2 configurable I/O ports

          - Alarm Triggers : Analytics, Network disconnect, Alarm input, App event, Time schedule

          - Alarm Events

            - File(image) upload via FTP and e-mail

            - Notification via e-mail

            - Micro SD/SDHC/SDXC or NAS recording at Event Triggers

            - Alarm output

            - Handover(PTZ Preset, Send message by HTTP/HTTPS/TCP)

            - Audio clip playback

          - Video Out for installation

            - USB: micro USB Type B, 1280x720

        - Lens

          - Focal length : 2.8mm fixed focal (x4)

          - Maximum Aperture Ratio : F2.0

          - Field of View

            - H : 209˚

            - V : 60˚

          - Minimum Object Distance : 5m (16.4ft)

          - Focus Control : Fixed

        - Pan /Tilt / Rotate

          - Pan range : 0 ~ 360˚

          - Tilt range : 0 ~ 65˚

        - DORI (EN62676-4 standard)

          - Detect : 52.2m (171.25ft)

          - Observe : 20.9m (68.50ft)

          - Recognize : 10.4m (34.25ft)

          - Identify : 5.2m (17.13ft)

      - IR Viewable Length : 20m(65.62ft) based on scene, 4 individual zones with Automatic/Manual level adjustment

      - Video Streams

        - The camera shall be able to produce 5 video profiles per channel, each of which may have the following properties:

          - Encoding type : H.265, H.264, MJPEG

          - Resolution:

            - 5120x1568, 3468x1152, 2560x784, 1280x392

          - Frame rate

            - H.265 / H.264 / MJPEG : Max.20fps (60Hz/50Hz)

            - Smart Codec : Manual(5ea area), WiseStream Ⅱ, WiseStreamⅢ(Based on AI engine)

          - Bit rate control

            - H.265 and H.264 : target bitrate level control, constant bit rate (CBR) or variable bit rate (VBR)

            - MJPEG : quality level control, variable bit rate (VBR)

      - Streaming Capability : Unicast(20 users) / Multicast, Multiple streaming(up to 3 profiles, 2 virtual channel support)

      - Simultaneous users (total) : 20 maximum (Unicast)

      - Storage and Recording

        - The camera shall have onboard SD card storage.

          - Card type : Micro SD/SDHC/SDXC

          - Capacity : Up to  1024 TB (512GB x 2 slots)

          - Image content on the card shall have the ability to be downloaded to a selected destination.

        - NAS

      - Interoperability - Video streams shall be capable of supporting ONVIF protocol, profiles S/G/T/M.

      - Single Images - The camera shall support jpg file image screenshot and export.

      - Memory : 4GB RAM, 512MB Flash

    - Network

      - Connectivity : Metal shielded RJ-45(10/100/1000BASE-T)

      - Protocols supported :

        - Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

        - Configuration: Dynamic Host Configuration Protocol (DHCP)

        - Web services: Hypertext Transfer Protocol (HTTP), Secure HTTP (HTTPS)

        - Security: Secure Sockets Layer(SSL)/Transport Layer Security(TLS)

        - Network services: Address Resolution Protocol (ARP), Bonjour, Domain Name System (DNS), Internet Control Message Protocol (ICMP), Link Layer Discovery Protocol(LLDP), Network Time Protocol (NTP), Protocol Independent Multicast-Sparse Mode (PIM-SM), Simple Network Management Protocol (SNMP v1/2c/3 – MIB-2), Universal Plug and Play (UPnP)

        - Media: Real-Time Transport Protocol (RTP), Real-Time Control Protocol, Real-Time Streaming Protocol (RTSP), Secure Real-Time Transport Protocol(SRTP)

        - Multicast: Internet Group Management Protocol (IGMP)

        - Notifications: File Transfer Protocol (FTP), Simple Mail Transfer Protocol (SMTP)

      - DDNS

        - The camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

      - Quality of Service (QoS)

        - Layer 3 DSCP

      - Security features

        - TPM 2.0 (FIPS 140-2 level 2)

        - HTTPS(SSL) Login Authentication

        - Digest Login Authentication

        - IP Address Filtering

        - User access log

        - 802.1X Authentication(EAP-TLS, EAP-LEAP, EAP-PEAP MSCHAPv2)

        - Device Certificate(Hanwha Techwin Root CA, pre-installed)

        - Secure by default certificate

        - Secure OS/Boot/Storage, Verify firmware forgery

      - Discovery

        - Manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.

    - Audio

      - Audio In

        - Selectable( MIC in / Line in)

        - Supply voltage : 2.5V DC (4mA)

        - Input impedance : 2K Ohm

      - Audio Out

        - Line out

        - Max. output level 1Vrms

      - Audio Compression

        - G.711 u-law / G.726 Selectable

        - G.726(ADPCM) 8KHz, G.711 8KHz

        - G.726 16KHz, 24KHz, 32KHz, 40KHz

        - AAC-LC : 48Kbps at 16KHz

    - Electrical

      - Input Voltage : PoE+ (IEEE802.3at, Class4)

      - Power consumption

        - PoE+ : Max 24.5W, Typical 11.3W

    - Mechanical And Environmental

      - Material : Aluminum, Hard-coated dome bubble

      - Dimensions (diameter x height) : 253 x 170 mm (9.96 x 6.69 inch)

      - Weight : 2900g (6.39lb)

      - Temperature And Humidity

        - Operating : -40°C ~ +55°C(-40°F ~ +131°F) / Less than 95% RH (Non-condensing)

        - Storage : -30°C ~ +60°C(-22°F ~ +140°F) / Less than 90% RH

      - Environmental Rating

        - IEC/EN 63000

        - IEC/EN 60529:2013 IP66

        - IEC/EN 62262:2005 IK10

        - NEMA 250 type 4X

      - RAL Code

        - RAL9003

      - Compatible Conduit hole / Gangbox

        - 3/4" (M25)

        - single, double, 4" octagon, 4" square

    - EMC & Safety

      - FCC 47 CFR Part 15 Subpart B

        - ANSI C63.4a-2017 Class A

      - IC Regulation ICES-003 Issue 7

        - ANSI C63.4a-2017 Class A

      - CE

        - EMC-Directive 2014/30/EU

          - EN 55032:2015/A11:2020

          - EN 50130-4:2011

          - EN 61000-3-2: 2014

          - EN 61000-3-3: 2013

        - RoHS-Directive 2011/65/EU

          - EN IEC 63000:2018

      - UKCA

        - EMC

          - S.I. 2016:1091 : EMC(Electromagnetic Compatibility) Regulations

            - BS EN 55032:2015/A11:2020 Class A

            - BS EN 50130-4:2011

            - BS EN 61000-3-2: 2014

            - BS EN 61000-3-3: 2013

        - RoHS

          - S.I. 2012:3032 : The Restriction of the Use of Certain Hazardous Substances in Electrical and Electronic Equipment Regulations 2012

            - BS EN IEC 63000:2018

      - NEMA 250-2014

      - VCCI-CISPR 32:2016 Class A

      - AS/NZS CISPR32:2015 Class A

      - UL listed

        - UL 62368-1, 3rd Ed. Issue Date:2019-12-13, Revision Date: 2021-10-22

        - CAN/CSA 222.2

END OF SECTION

8