Hanwha Vision is a global leading supplier of solutions for IP and analog video surveillance. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-effectiveness. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: Hanwha Vision(https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com)

    - Model: PNV-A6081R-E2T

    - Alternates: None

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264, and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously.

        - H.265 and H.264 – Max. 120/100fps (60Hz/50Hz) at all resolution

        - MJPEG – Max. 30fps

      - The camera shall be able to configure up to 10 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - The camera shall be able to configure various resolution selections.

        - 16:9 aspect ratio: 1920 x 1080, 1280 x 720, 800 x 448, 640 x 360

        - 4:3 aspect ratio: 1280 x 960, 1024 x 768, 800 x 600, 640 x 480

        - 5:4 aspect ratio: 1280 x 1024, 720 x 576

        - 3:2 aspect ratio: 720 x 480

      - The camera shall support unicast video streaming for up to 20 users.

      - The camera shall support multicast video streaming.

      - The camera shall support multiple video streaming up to 5 profiles

      - The camera shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

      - The camera shall provide WiseStreamⅡ, WiseStreamⅢ, Dynamic GOV, and Dynamic FPS to efficiently manage bit rate of the video stream and reduce storage.

    - Camera – The camera device shall have the following physical and performance properties:

      - IK10 rated for protection against impacts.

      - True day/night operation with removable IR cut filter

        - Low light level operation to 0.008 lux at F1.3 in color mode and 0.0008 lux at F1.3 in black and white mode.

        - Black and White mode operation to 0 lux with IR LED on.

      - The camera shall be able to produce clear images in high contrast scenes with multi-exposure wide dynamic range up to 120dB.

      - The camera shall support WiseNR II to reduce digital noise using both 2D and 3D noise reduction technology.

      - The camera shall be able to configure 6 privacy masking areas with rectangles.

      - The camera shall provide a video display on the smartphone (iPhone, Android) to adjust the viewing angle, rotation, and focus.

    - Intelligence and Analytics – The camera shall have a suite of intelligent analytic functions.

      - Analytics events

        - Object detection: Person / Face / Vehicle(Car/Truck/Bus/Bicycle/Motorcycle) / License plate, Bestshot

        - IVA: Virtual line/Area, Enter/Exit, Loitering, direction, intrusion

        - Face mask detection

        - Social distancing detection

      - Attributes

        - Person

          - Gender: Female / Male

          - Upper/Lower (color): Black / Gray / White / Red / Orange / Yellow / Green / Blue / Purple (up to 2 colors at the same time)

          - Bag: Wear Bag or Not

        - Vehicle

          - Type: Car(sedan/SUV/Van) / Bus / Truck / Motorcycle / Bicycle

          - Color: Black / Gray / White / Red / Orange / Yellow / Green / Blue / Purple (up to 2 colors at the same time)

        - Face

          - Gender: Female / Male

          - Age: Child / Young / Middle / Old

          - Glasses: Wear Glasses or Not

        - License plate

      - Statistics events

        - People counting

        - Heatmap

        - Queue management

      - Motion detection with 8 definable detection areas with 8 points polygonal zones, and minimum/maximum object size.

      - Tampering detection that event can be created when the screen is blocked or the camera position is changed

      - Defocus detection that event can be created when defocus of the camera lens is detected

      - Audio detection that event can be created when audio above a set level is detected

      - Classification of the following sound.

        - Scream

        - Gunshot

        - Explosion

        - Crashing glass

    - Interoperability – The camera shall be ONVIF Profile S / G and T compliant.

    - The camera shall possess the following further characteristics:

      - Built-in web server, accessed via non-plugin browsers including Google Chrome, MS Edge, Mozilla Firefox and Apple Safari.

      - NAS recording option with configurable pre-alarm and post-alarm recording intervals

      - Alarms and notifications

        - alarm notification triggers:

          - Alarm input

          - Analytics

          - Network disconnect

          - App event

          - Time schedule

          - MQTT subscription

        - available notification means upon trigger:

          - File upload(image): e-mail/FTP

          - Notification: e-mail

          - Alarm output

          - Handover: PTZ preset, Send message by HTTP/HTTPS/TCP

          - Audio clip playback

          - MQTT: publication

      - Pixel Counter available in the web viewer.

      - IP66/IP67/IP6K9K, IK10, NEMA4X

      - This device has been verified using STP cable. The use of appropriate GND grounding and STP cable is recommended to effectively protect your product and property from transient voltage, thunder stroke, and communication interruption.

  - DETAILED SPECIFICATIONS

    - Video

      - Imaging device: 1/2" CMOS

      - Scanning: Progressive

      - Minimum Illumination

        - Color: 0.008Lux(F1.3, 1/30sec, 30IRE) 30/25fps

        - B/W: 0.0008Lux(F1.3, 1/30sec, 30IRE) 30/25fps

        - Color: 0.016Lux(F1.3, 1/60sec, 30IRE) 60/50fps

        - B/W: 0.0016Lux(F1.3, 1/60sec, 30IRE) 60/50fps

        - Color: 0.032Lux(F1.3, 1/120sec,30IRE) 120/100fps

        - B/W: 0.0032Lux(F1.3, 1/120sec, 30IRE) 120/100fps

        - 0Lux(IR LED on)

    - Lens

      - Focal length: 4.38~9.33mm(2.13x) motorized varifocal

      - Max. Aperture Ratio: F1.3(Wide)~2.15(Tele)

      - Field of View

        - [Wide] H: 104˚,  V: 55°, D: 124°

        - [Tele]  H:   44˚, V: 24°, D:  51°

      - Min. Object Distance: 0.5m (1.64ft)

      - Focus Control: Simple focus

      - Lens Type: P-Iris(IR corrected)

    - Pan & Tilt & Rotate

      - Range:	0˚~360˚ / -45˚~85˚ / 0˚~355˚

    - IR Viewable Length: 40m (131.23ft), Wise IR

    - Operational Functions

      - Camera Title: Off / On (Displayed up to 85 characters)

      - Day/Night Setting: Auto (ICR) / Color / B/W / External / Schedule

      - Backlight Compensation: Off / BLC / WDR / SSDR

      - WDR: 120dB

      - Contrast Enhancement:	Off / On (SSDR)

      - Digital Noise Reduction:	 Off / On (SSNR, WiseNR II )

      - Digital Image Stabilization: Off / On

      - Motion Detection: Off / On (8ea, 8-point polygonal)

      - Privacy Masking: Off / On (6 zones, rectangle)

        - Color: Grey / Black / White

      - Gain Control: Off / Low / Middle / High

      - White Balance: ATW / AWC / Manual / Indoor / Outdoor

      - Electronic Shutter Speed: Min / Max / Anti-flicker (1/5 ~ 1/12,000sec)

        - Auto prefer shutter control

      - Digital PTZ: Support (Preset, Group)

      - Image Rotation

        - Flip: Off / On

        - Mirror: Off / On

        - Hallway view: 0˚ / 90˚ / 270˚

      - Alarm I/O: Input 1ea / Output 1ea / DC 12V Power(Max. 50mA) 1ea

      - Alarm Triggers: 	Alarm Input, Analytics, Network Disconnection

      - Alarm Events

        - File Upload via FTP and E-mail

        - Notification via E-mail

        - SD/SDHC/SDXC or NAS recording at event triggers

        - Alarm output

        - Handover

        - Audio playback

        - MQTT: publication

      - Pixel Counter: Support

      - Edge Storage

        - 2TB SSD included(WAVE pre-installed)

* For WAVE-related A&E documents, please visit the U.S. website (https://hanwhavisionamerica.com)

        - Possible to register the maximum number of camera

1) Up to 6 cameras (Host + 5 cameras)

2) Total bandwidth: up to 33Mbps

      - Intelligent Analytics

        - Object detection(Person/Face/Vehicle/License plate with attributes, BestShot per object),

        - Face mask detection

        - Social distancing detection

        - Virtual line: Directional detection

        - Virtual area: Enter/Exit, Loitering

        - Digital auto tracking,

        - Business Intelligence : People counting, Queue management, Heatmap

      - Intelligent Analytics

        - Defocus detection

        - Motion detection

        - Appear/Disappear,

        - Tampering

        - Audio detection

        - Sound classification

        - Shock detection

      - Video Out (Installation)

        - CVBS: 1.0 Vp-p / 75Ω composite, 720x480(N), 720x576(P)

      - Memory: 4096MB RAM, 512MB Flash

    - Video Streams

      - Video compression: H.265, H.264, MJPEG

      - Resolution: 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768,800x600, 800x448, 720x576, 720x480, 640x480, 640x360

      - Maximum Framerate

        - H.265 / H.264

          - Max. 60/50fps(60Hz/50Hz) at all resolutions

          - Max. 120/100fps(60Hz/50Hz) at all resolutions

        - MJPEG: Max. 30/25fps

      - Smart Codec: Manual Mode (area-based : 5EA)

      - WiseStream: WiseStreamⅡ, WiseStreamⅢ

      - Bitrate Control Method

        - H.265 / H.264: CBR or VBR

        - MJPEG: VBR

      - Streaming Capability: Multiple streaming (Up to 5 profiles)

      - Streaming method: Unicast / Multicast

      - Simultaneous Users: 6 maximum (Unicast)

      - Interoperability:	ONVIF Profile S / G / T, SUNAPI(HTTP API), Open Platform

    - Audio

      - Audio In

        - Mic-in / Line-in

        - Supply voltage: 2.5V DC(4mA), Input impedance: 2K Ohm

      - Audio Out

        - Line out

        - Max. output level 1Vrms

      - Audio Compression

        - G.711 u-law /G.726 Selectable

        - G.726 (ADPCM) 8KHz, G.711 8KHz

        - G.726: 16Kbps, 24Kbps, 32Kbps, 40Kbps

        - AAC-LC: 48Kbps at 16KHz

    - Network

      - Connectivity – Metal Shield RJ-45(10/100/1000BASE-T)

      - Protocol

        - IP v4 / v6, TCP, UDP

        - Configuration: DHCP, LLDP

        - Web service: HTTP, HTTPS

        - Network Service: ARP, Bonjour, DNS, ICMP, NTP, SNMP v1/2c/3 – MIB-2, UPnP

        - Media: RTP, RTCP, RTSP

        - Multicast: IGMP

        - Notifications: FTP, SMTP, MQTT

      - DDNS – The camera shall support DDNS services offered by the manufacturer and others publicly available service offerings

      - QoS – Layer 3 DSCP

      - Security Feature

        - User password protection

        - The device shall not provide a manufacture default password. Default password change shall be required to access the camera.

        - A minimal level of password complexity shall be required by the camera.

        - The camera shall not have a manufacture back-door password.

        - The manufacturer shall provide a tool that provides the ability to make password changes to multiple cameras at the same time.

        - HTTPS(SSL) Login Authentication

        - Digest Login Authentication

        - IP Address Filtering

        - User access log

        - 802.1X Authentication(EAP-TLS, EAP-LEAP)

        - Device Certificate(Hanwha Techwin Root CA)

        - TPM with FIPS 140-2 level2

        - Secure boot, Verify firmware forgery

      - Discovery – The manufacturer shall offer a discovery program to identify all devices of them on the network.

      - Configuration – The manufacturer shall offer a configuration program that remotely allows users to change settings on multiple cameras simultaneously.

      - Firmware upgrade – The manufacturer shall offer a program capable of upgrading multiple cameras at the same time (not requiring access to individual cameras).

      - Camera backup setting – The manufacturer shall provide a program that provides the ability to save multiple camera settings to a file and restore these camera settings if needed.

      - Reporting – The manufacturer shall provide a tool that can generate a report including thumbnail view, MAC address, IP address, serial number and other camera settings.

    - Electrical

      - Power

        - Input Voltage / Current: PoE+ (IEEE 802.3at), DC 12V

        - Power Consumption

          - PoE+: Max 19.2W, typical 14.4W

          - 12VDC: Max 17.5W, typical 12.7W

          - Power redundancy failover

    - Mechanical And Environmental

      - Color/Material: White / Aluminum, Hard-coated dome bubble

      - RAL code: RAL9003

      - Dimensions(W x H): Ø180 x 130mm (7.09" x 5.12")

      - Weight:	1.90kg (4.19lb)

      - Compatible Conduit Hole: 19.1mm(3/4")(M25)

      - Gangbox: single, double, 4” octagon, 4” square

      - Hanging Mount(Dome): SBP-187HMW

      - Temperature

        - Operating: -30°C ~ +55°C (-22°F ~ +131°F) / 0~90%(non-condensing)

* Start-up should be done at above -20°C

        - Storage: -50°C ~ +60°C (-58°F ~ +140°F)

      - Humidity: Less than 90% RH

      - Ingression Protection: IP66/IP67/IP6K9K, NEMA4X

      - Vandal Resistance: IK10

    - DORI (EN62676-4 standard)

      - Detect(25PPM/ 8PPF): Wide: 30.0m(98.43ft) / Tele: 95.0m(311.82ft)

      - Observe(63PPM/ 19PPF): Wide: 12.0m(39.37ft) / Tele: 38.0m(124.73ft)

      - Recognize(125PPM/ 38PPF): Wide: 6.0m(19.69ft) / Tele: 19.0m(62.36ft)

      - Identify(250PPM/ 76PPF): Wide: 3.0m(9.84ft) / Tele: 9.5m(31.18ft)

    - EMC

      - FCC 47 CFR 15 Subpart B Class A

      - ICES-3(A)/NMB-3(A)

      - CE/UKCA

- EN 55032 Class A, EN 50130-4, EN 61000-3-2, EN 61000-3-3

      - VCCI CISPR 32 Class A

      - RCM AS/NZS CISPR 32 Class A

      - KS C 9832 Class A , KS C 9835CE EN 50581:2012 (hazardous substances)

    - Safety

      - UL 62368-1, CAN/CSA C22.2 NO. 62368-1

      - IEC/EN 62471

    - Environment

      - IEC/EN 63000

      - IEC/EN 60529 IP66/IP67, ISO 20653 IP6K9K, IEC/EN 62262 IK10

      - NEMA 250 type 4X

END OF SECTION

10