Hanwha Techwin is a global leading supplier of solutions for IP and analog video surveillance. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-effectiveness. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://www.hanwha-security.com/

2M NETWORK INTERCOM

# DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented in colored text.

Important:  See further notes on the following page.

# Important Note to Security Systems Specifiers

# CSI MasterFormat 2018 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although adoption of MasterFormat 2018 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

# Primary Specification Area:

MasterFormat 2016:

# 28 10 00	Access Control

# 28 15 00	Access Control Hardware Devices

# 28 15 23	Intercom Entry Systems

# 28 15 23.17	Audio Video Intercom (IP)

# MasterFormat 2018:

# 28 10 00	Access Control

# 28 15 00	Access Control Hardware Devices

# 28 15 23	Intercom Entry Systems

# 28 15 23.17	Audio-Video Intercom

2M NETWORK INTERCOM

- GENERAL

  - SUMMARY

    - Intercoms shall be IP-based and comply with established network and video standards.

    - Product - A 2M IP video intercom with multi-streaming (H.265, H.264 and MJPEG) capability with IR illuminator.

    - Related Requirements

Refer to MasterFormat notes at the beginning of this document to select requirements specific to the MasterFormat version being used in the specification.

  - REFERENCES

    - Abbreviations

      - AGC		Auto Gain Control

      - AES		Advanced Encryption Standard

      - API		Application Programming Interface

      - ARP		Address Resolution Protocol

      - AWB		Auto White Balance

      - BLC		Back light compensation

      - CBR		Constant Bit Rate

      - CVBS		Composite Video Blanking and Sync

      - DHCP		Dynamic Host Configuration Protocol

      - DNR		Digital Noise Reduction

      - DNS		Domain Name Server

      - DDNS		Dynamic Domain Name Server

      - DSCP		Differentiated Services Code Point

      - FPS		frames per second

      - FTP		File Transfer Protocol

      - GOV		Group of Video

      - GUI		Graphical User Interface

      - HD		High Definition

      - HTTP		Hypertext Transfer Protocol

      - HTTPS		Secure HTTP

      - ICMP		Internet Control Message Protocol

      - IGMP		Internet Group Management Protocol

      - IP		Internet Protocol

      - IR		Infrared

      - JPEG		Joint Photographic Experts Group

      - LAN 		Local Area Network

      - LED		Light Emitting Diode

      - LDC		Lens Distortion Correction

      - LLDP		Link Layer Discovery Protocol

      - LPR		License Plate Recognition

      - MJPEG		Motion JPEG

      - MP		Megapixel

      - MPEG		Moving Pictures Experts Group

      - NAS		Network Attached Storage

      - NTP		Network Time Protocol

      - NVR		Network Video Recorder

      - ONVIF		Global standard for the interface of IP-based physical security products

      - PIM-SM	Protocol Independent Multicast-Sparse Mode

      - PoE		Power over Ethernet

      - PPPoE		Point to Point Protocol over Ethernet

      - QoS		Quality of Service

      - RTP		Real-Time Transport Protocol

      - RTCP		Real-Time Control Protocol

      - RTSP		Real-Time Streaming Protocol

      - SDK		Software Development Kit

      - SFP		Small Form factor Pluggable

      - SIP		Session Initiation Protocol

      - SMTP		Simple Mail Transfer Protocol

      - SNMP		Simple Network Management Protocol

      - SSDR		Super Smart Dynamic Range

      - SSNR		Super Smart Noise Reduction

      - SSL		Secure Sockets Layer

      - TCP		Transmission Control Protocol

      - UDP		User Datagram Protocol

      - UPnP		Universal Plug and Play

      - VBR		Variable Bit Rate

      - VMS		Video Management System

      - WDR		Wide Dynamic Range

    - Reference Standards

      - Network - IEEE

        - 802.3 Ethernet Standards

        - 802.1x  Port-based Network Access Control

        - IPv4 IP addressing version 4

        - IPv6 IP addressing version 6

        - QoS Quality of Service

      - Video

        - ISO / IEC 23008-2:2013, MPEG-H Part2 (ITU H.265, HEVC)

        - ISO / IEC 14496–10, MPEG-4 Part 10 ( ITU H.264)

        - ISO / IEC 10918 – JPEG

        - ONVIF – Profile S

      - EMC & Safety

        - FCC 47 CFR Part 15 Subpart B

          - ANSI C63.4-2017 Class A

        - IC Regulation ICES-003:2016

          - CAN/CSA CISPR 32:17 Class A

        - CE EMC-Directive 2014/30/EU

          - EN 55032:2012/AC:2013 Class A

          - EN 50130-4:2011

          - EN 610000-3-2:2014

          - EN 610000-3-3:2014

        - BS CE EMC-Regulations 2016/1091

          - BS EN 55032:2012/AC:2013 Class A

          - BS EN 50130-4:2011

          - BS EN 610000-3-2:2014

          - BS EN 610000-3-3:2014

        - VCCI-CISPR 32:2016 Class A

        - AS/NZS CISPR32:2015 Class A

      - Safety

        - UL listed

        - CE EN 50581:2012 (hazardous substances)

        - BS EN 50581:2012 (hazardous substances)

      - Ingress Protection and Vandal Resistance

        - IEC 60529:2006 – Degrees of Protection Provided by Enclosures – IP65

        - IEC EN 62262 - Degrees of protection provided by enclosures for electrical equipment against external mechanical impacts : IK08

    - Definitions

      - GOV (Group of Video object planes) – A set of video frames for H.264 and H.265 compression, indicating a collection of frames from the initial I-Frame (key frame) to the next I-Frame. GOV consists of two kinds of frames in video surveillance setup: I-Frame and P-Frame.

      - Dynamic GOV – Dynamic assignment of GOV length based on the complexity of the scene to efficiently manage bitrate of the video stream and reduce the storage required.

      - Multi-exposure wide dynamic range – Operation which automatically adjusts shutter speed to provide a wide range between dark and light areas visible at the same time, preventing backlighting issues. Long exposure is used for dark areas and a short exposure is used in bright areas.

      - Dynamic FPS – Dynamic assignment of FPS (frames per seconds) based on the movement of object(s) in the scene to efficiently manage bitrate of the video stream and reduce the storage required.

      - Smart Codec – Smart Codec that controls quantization parameter and dynamic FPS in H.265 and H.264 to efficiently manage bitrate of the video stream and reduce the storage required.

      - WiseStream – Technology that controls quantization parameter, FPS, and GOV length in H.265 and H.264 to efficiently manage bitrate of the video stream and reduce the storage required.

      - DORI (Detect, Observe, Recognize, Identify) – A standard system (EN-62676-4) for defining the ability of a camera to distinguish persons or objects within a covered area.

        - Detect: 25PPM / 8PPF

        - Observe: 63PPM / 19PPF

        - Recognize: 125PPM / 38PPF

        - Identify: 250PPM / 76PPF

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic data sheets

      - Manufacturer’s installation and operation manuals

      - Warranty documentation

  - QUALIFICATIONS

    - Manufacturer shall have a minimum of five years’ experience in producing IP video equipment.

    - Installers shall be trained and authorized by the Manufacturer to install, integrate.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver the intercom in the manufacturer’s original, unopened, undamaged container with identification labels intact.

    - Store the intercom in a temperature environment specified in section 2.04 Detailed Specification, protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY, LICENSING AND SUPPORT

    - Manufacturer shall provide at least a limited 3 year warranty for the product to be free of defects in material and workmanship.

    - Manufacturer shall provide embedded intercom video analytics free of license charges.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Hanwha Techwin

http://www.hanwha-security.com/

    - Model		TID-600R

    - Alternates: 		None

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The intercom shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously.

        - H.265 and H.264 – maximum of 60/50FPS(60Hz/50Hz) at all resolution

        - MJPEG – maximum of 30fps/25fps(60Hz/50Hz)

      - The intercom shall be able to configure up to 10 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - The intercom shall be able to configure various resolution selections.

        - 16:9 aspect ratio : 	1920x1080, 1280x720, 800x448,

640x360

        - 4:3 aspect ratio : 	1280x960, 1024x768, 800x600, 640x480, 320x240

        - 5:4 aspect ratio : 	1280x1024, 720x576

        - 3:2 aspect ratio : 	720x480

      - The camera shall support unicast video streaming up to 20 users.

      - The camera shall support multicast video streaming.

      - The intercom shall support multicast video streaming up to 10 profiles.

      - The intercom shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

      - The intercom shall provide WiseStream, Dynamic GOV and Dynamic FPS to efficiently manage bit rate of the video stream and reduce storage.

    - Camera – The intercom device shall have the following physical and performance properties:

      - True day/night operation with scheduling and options for external devices.

        - Low light level operation to 0.018 lux at F1.6 in color mode.

        - Black and white mode operation to 0 lux with IR LED on.

      - The intercom shall be able to produce clear images in highly contrast scenes with multi-exposure wide dynamic range up to 150dB.

      - The intercom shall support digital noise reduction using both 2D and 3D noise reduction technology.

      - The intercom shall be able to configure 32 privacy masking areas with polygonal zones.

      - The intercom shall have the defog feature to remove fogginess of scene which can be triggered automatically from the fog detection event.

    - Intelligence and Analytics – The intercom shall have a suite of intelligent analytic functions.

      - Analytics event

        - IVA

          - Directional detection, Appear/Disappear, Virtual line, Enter/Exit, Loitering, intrusion

        - Motion detection

          - Motion detection with 8 definable detection areas with 8 point polygonal zones, and minimum/maximum object size.

        - Tampering

        - Fog detection

        - Audio detection

        - Shock detection

        - Sound classification

          - Detection and classification of the following sound.

            - Scream

            - Gunshot

            - Explosion

            - Crashing glass

    - Interoperability – The intercom shall be ONVIF Profile S.

    - The intercom shall possess the following further characteristics:

      - Micro SD/SDHC/SDXC memory card with configurable pre-alarm and post-alarm recording intervals

      - NAS recording option with configurable pre-alarm and post-alarm recording intervals

      - Alarms and notifications

        - alarm notification triggers:

          - Analytics

          - Network disconnect

          - Alarm input

          - App event

          - Time schedule

        - available notification means upon trigger:

          - File upload(image) : e-mail/FTP

          - Notification : e-mail

          - Recording : SD/SDHC/SDXC or NAS recording at event triggers

          - Alarm output

          - Handover(PTZ preset, Send message by HTTP/HTTPS)

          - Audio clip playback

      - Pixel Counter available in the web viewer.

      - IP65/IK08/NEMA4x

      - This device has been verified using STP cable. The use of appropriate GND grounding and STP cable is recommended to effectively protect your product and property from transient voltage, thunderstroke, communication interruption.

    - Call functionality

      - The intercom shall support SIP for integration with VoIP, peer-to-peer or integrated into SIP/PBX.

      - The intercom shall support touchless call.

      - Call group will

    - Access control functionality

      - The intercom shall support remote control using DTMF codes.

      - DTMF code can be entered up to 4 digits (from 0 to 9)

      - Account can be allowed to add up to 4 user using Register.

  - INTERCOM SOFTWARE

    - The intercom shall have a built in web server.

    - The web viewer shall provide a monitoring screen which displays live intercom video and simultaneously provides same-screen access to the following functions:

      - Live view window size

      - Resolution setting

      - Image (snapshot) capture

      - Manual recording to SD or NAS

      - Audio/microphone control

      - Access recorded data playback and intercom configuration menus

    - The web viewer shall provide a playback screen which provides access to the following functions:

      - Recorded data search using date and time range

      - Recorded data search using event type

      - Play a recorded video by event triggering

      - Set playback speed

      - Play audio if present

      - Generate a backup copy of saved video data

    - The web viewer shall provide a setup screen which provides access to the following configuration settings and functions in the intercom:

      - Digital video profile to include compression type, maximum or target bit rate, frame rate, multicast parameters, and crop encoding area

      - User profile to include password, access level, authentication

      - Date and time

      - Network settings and IP version

        - DDNS

        - IP filtering

        - SSL/TLS, including certificate management

        - 802.1x authentication

        - Quality of Service settings

        - SNMP to include version selection and settings

        - Auto IP configuration

      - Video setup

        - Flip / mirror mode

        - Video output type

        - Privacy zone

      - Audio setup to include source, audio codec type, gain and bit rate.

      - Intercom settings to configure image preset, sensor frame capture, dynamic range, white balance, back light, exposure, day/night operation, on-screen display, sharpness, contrast, color level and lens distortion correction.

      - Event detection setup to configure notification parameters, recording rules, time schedule, tamper protection, motion detection and event triggers

      - System function to control reboot, upgrade, check system and event logs and application (SDK) management

      - View profile information

  - DETAILED SPECIFICATIONS

    - Video

      - Imaging device			1/2.8" CMOS

      - Scanning			Progressive

      - Minimum Illumination		Color: 0.018Lux (F1.6, 1/30sec),

B/W: 0Lux(IR LED On)

      - Video out			CVBS : 1.0 Vpp/75Ω composite, 720x480(N), 720x576(P)

for installation

    - Lens

      - Focal length			1.6mm fixed focal

      - Max. Aperture Ratio		F1.6

      - Field of View			H : 108°, V : 114°, D : 180°

      - Min. Object Distance		0.3m(0.98ft)

      - Focus Control			Fixed

    - Operational Functions

      - IR Viewable Length		5m(16.4ft)

      - Camera Title 			Off / On (Displayed up to 85 characters)

      - Day/Night Setting		Auto (ICR) / Color / B/W / Schedule

      - Backlight Compensation		Off / BLC / HLC / WDR(150dB)

      - Digital Noise Reduction (DNR)	SSNRⅤ

      - Digital Image Stabilization (DIS)	Support

      - Defog				Support

      - Motion Detection		Off / On (8ea, 8-point polygonal)

      - Privacy Masking		Off / On (32ea, Poligonal zones)

- Color: Grey / Green / Red / Blue / Black / White

- Mosaic

      - Gain Control			Off / Low / Middle / High

      - White Balance 			ATW / AWC / Manual / Indoor / Outdoor

      - Electronic Shutter Speed 	Min / Max / Anti-flicker (2 ~ 1/12,000sec)

      - Image Rotation			Flip: Off / On

Mirror: Off / On

      - Alarm I/O			Input 2ea / Output 1ea,

Relay out 1ea (DC 12V Max. 550mA with 2A adapter)

Tampering Switch, Touchless call

      - Alarm Triggers			Analytics, Network disconnect, Alarm input

      - Alarm Events			File upload(image) : e-mail/FTP

Notification : e-mail

Recording : SD/SDHC/SDXC or NAS recording at event triggers

Alarm output

Handover(PTZ preset, Send message by HTTP/HTTPS/TCP)

Audio clip playback

      - Audio Streaming		Two-way/One-way selectable, Full duplex,

Echo cancellation and noise reduction

      - Audio in			Built-in MIC

      - Audio out			Built-in speaker, 85dB at 0.5m

      - Pixel Counter			Support

      - Storage				Micro SD/SDHC/SDXC 1slot 256GB

      - Analytics			Directional detection, Motion detection, Appear/Disappear

Enter/Exit, Loitering, Tampering, Virtual line, Audio detection

Shock detection, Sound classification

      - Memory			2GB RAM

    - Video Streams

      - Video compression		H.265, H.264, MJPEG

      - Resolution			1920x1080, 1280x1024,1280x960, 1280x720,1024x768,

800x600, 800x448, 720x576, 720x480, 640x360, 320x240

      - Maximum Framerate

        - H.265 / H.264		H.265/H.264: Max. 60fps/50fps(60Hz/50Hz)

        - MJPEG 			Max. 30fps/25fps(60Hz/50Hz)

      - Smart Codec			Manual(5ea area), Low/Middium/High

      - WiseStream			Off/Low/Medium/High

      - Bitrate Control Method		H.265 / H.264: CBR or VBR

MJPEG: VBR

      - Streaming Capability		Unicast(20 users) / Multicast

Multiple streaming(Up to 10 profiles)

      - Streaming method		Unicast / Multicast

      - Simultaneous Users		20 maximum (Unicast)

      - Profile set			Max. 10 ea

      - Interoperability			ONVIF Profile S, SUNAPI(HTTP API),

Wisenet Open Platform

    - Network

      - Connectivity 			Metal shielded RJ-45(10/100BASE-T)

      - Protocol

        - IP v4 / v6, TCP, UDP

        - Configuration: DHCP, LLDP

        - Web service: HTTP, HTTPS

        - Network Service: ARP, Bonjour, DNS, ICMP, NTP, PIM-SM, SNMPv1/v2c/v3(MIB-2), UPnP

        - Media: RTP, RTCP, RTSP, SIP 2.0

        - Unicast: SRTP

        - Multicast: IGMP, PIM-SM

        - Notifications: FTP, SMTP

      - DDNS – The intercom shall support DDNS services offered by the manufacturer and others publicly available service offerings

      - QoS – Layer 3 DSCP

      - Security Feature

        - User password protection

        - The device shall not provide a manufacture default password. Default password change shall be required to access the intercom.

        - A minimal level of password complexity shall be required by the intercom.

        - The intercom shall not have a manufacture back-door password.

        - The manufacturer shall provide a tool that provides the ability to make password changes to multiple intercoms at the same time.

        - IP address filtering – List of allowed or blocked IP addresses

        - HTTPS(SSL/TLS) login authentication

        - HTTPS(SSL/TLS) secured communication

        - Digest login authentication

        - User access log

        - 802.1X Authentication(EAP-TLS, EAP-LEAP)

        - Device Certificate(Hanwha Techwin Root CA, pre-installed)

      - Discovery – The manufacturer shall offer a discovery program to identify all devices of them on the network.

      - Configuration – The manufacturer shall offer a configuration program that remotely allows users to change settings on multiple intercoms simultaneously.

      - Firmware upgrade – The manufacturer shall offer a program capable of upgrading multiple intercoms at the same time (not requiring access to individual intercoms).

      - Intercom backup setting – The manufacturer shall provide a program that provides the ability to save multiple intercom settings to a file and restore these intercom settings if needed.

      - Reporting – The manufacturer shall provide a tool that can generate a report including thumbnail view, MAC address, IP address, serial number and other intercom settings.

    - Electrical

      - Power

        - Input Voltage		PoE, 12VDC

        - Power Consumption	PoE: Max 12.95W, typical 9.1W

12VDC: Max 17.6W, typical 7W

※12VDC power consumption includes 12VDC/550mA relay out

    - Mechanical And Environmental

      - Color/Material			Dark Gray / Aluminum

      - RAL Code			RAL7022

      - Dimensions (W x H x D)	Ø49x165x48mm(1.93x6.50x1.89")

      - Weight 				490g(1.08 lb)

      - Skin Cover			SBC-165W

      - Temperature

        - Operating 			-30 °C ~ 55 °C(-22°F ~ +131°F)

        - Storage  			-50°C ~ +55°C(-58°F ~ +131°F)

      - Humidity

        - Operating 			Less than 95% RH (non-condensing)

        - Storage			Less than 90% RH (non-condensing)

      - Ingression Protection		IP65, NEMA4x

      - Vandal Resistance		IK08

    - DORI

      - Detect (25PPM/ 8PPF) 		25.4m(83.33ft)

      - Observe (63PPM/ 19PPF) 	10.02m(32.87ft)

      - Recognize (125PPM/ 38PPF) 	5.01m(16.43ft)

      - Identify (250PPM/ 76PPF) 	2.5m(8.2ft)

END OF SECTION

- EXECUTION

  - INSTALLERS

            - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

            - The network design and configuration shall be verified for compatibility and performance with the intercom(s).

            - Network configuration shall be tested and qualified by the Contractor prior to intercom installation.

            - All firmware found in products shall be the latest and the most up-to-date provided by the manufacturer, or of a version as specified by the provider of the VMS or NVR.

            - All equipment requiring users to log on using a password shall be configured with user/site-specific password/passwords. No system/product default passwords shall be allowed.

  - INSTALLATION

            - The contractor shall carefully follow instructions in documentation provided by the manufacturer to insure all steps have been taken to provide a reliable, easy-to-operate system.

            - All equipment shall be tested and configured in accordance with instructions provided by the manufacturer prior to installation.

            - Before permanent installation of the system, the contractor shall test the system in conditions simulating the final installed environment.

  - STORAGE

            - The hardware shall be stored in an environment where temperature and humidity are in the range specified by the manufacturer.

END OF SECTION

10