Hanwha Vision is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://hanwhavision.com/

- PRODUCTS

  - EQUIPMENT

    - Manufacturer : Hanwha Vision (http://hanwhavision.com/)

    - Model : TNO-7180RLP

    - Alternates :	TNO-7180RLP

  - GENERAL DESCRIPTION

    - Thermal VGA camera shall have a metal housing which can prevent any inside sparks caused by the electronic components from igniting gases and conductive dust and causing explosion in a hazardous area.

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously

        - H.265 : Max. frame rates to 55fps @ 3M and 60fps @ 2M

        - H.264 : Max. frame rates to 50fps @ 3M and 50fps @ 2M

        - MJPEG : Max. frame rates to 5fps @ 3M and 2M

      - The camera shall be able to configure up to 10 independent video stream per channel with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - Resolution selections :

        - 3M: 2048x1536, 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

        - 2M: 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

      - Simultaneous unicast access by up to 20 users

      - Multicast or unicast capable

      - Dynamic DNS (DDNS) support.

      - WiseStream Ⅱ, Dynamic GOV to efficiently manage bitrate of the video stream.

    - Camera – The camera device shall have the following physical and performance properties:

      - IP66, IK10, NEMA4X rated for protection against impacts

      - Provide video display on smart phone (iPhone, Android) to adjust viewing angle.

      - 32 privacy masking regions utilizing quadrangle

    - Intelligence and Analytics – The camera shall have a suite of integral intelligent operations and analytic functions to include

      - Motion detection with eight definable detection areas, minimum / maximum object size definition.

      - Detection of logical events of specified conditions from the camera’s video input

    - Analytics events: Directional detection, Motion detection, Appear/Disappear, Enter/Exit, Loitering, Tampering, Virtual line, Audio detection, Temperature detection, Sound classification, Shock detection

    - Interoperability – The camera shall be ONVIF Profile S / G / T compliant.

    - The camera shall possess the following further characteristics:

      - Micro SD/SDHC/SDXC memory card and NAS recording options, with configurable pre-alarm and post-alarm recording intervals

      - Alarms and notifications

        - Alarm notification triggers:

          - Alarm input

          - Analytics events

          - Network disconnect

        - Available notification means upon trigger:

          - File upload via FTP and e-mail

          - Notification via e-mail

          - Record to local storage (Micro SD/SDHC/SDXC card or NAS)

          - Alarm output

      - POE capable

  - DETAILED SPECIFICATIONS

    - Video

      - Image

        - Sensor : Global Shutter 1/1.8” CMOS

        - The following features with control settings shall be available:

          - Camera Title

            - Off / On (Displayed up to 85 characters)

          - Digital Image Stabilization : Support (Built-in gyro sensor)

          - Motion Detection : Off / On (8ea, 8 point polygonal zones)

          - Privacy Masking : Off / On (6ea, Rectangle zones)

            - Color : Gray / Green / Red / Blue / Black / White

          - Video Rotation : Flip, Mirror

          - Alarm I/O : 4 configuratble I/O ports

          - Alarm Triggers : Analytics, Network disconnect, Alarm input

          - Alarm Events

            - File upload via FTP and e-mail

            - Notification via e-mail

            - Micro SD/SDHC/SDXC or NAS recording at Event Triggers

            - Alarm output

          - Pixel Counter : Support

          - Video Out for installation

            - BNC: CVBS, 1.0 Vp-p / 75Ω, 720 x 480(N), 720 x 576(P)

            - Micro USB Type B, 1280 x 720

        - Lens : 6.8~120mm(18x) motorized varifocal

          - Max. Aperture Ratio : F1.6(Wide)~F4.13(Tele)

          - FoV : H: 54.5º(Wide)~3.4º(Tele), V: 42.3º(Wide)~2.5º(Tele)

          - Min. Object Distance : 2m (6.5ft)

          - Lens Type : DC auto iris

          - Focus Control : Simple focus, Focus save

      - Video Streams

        - The camera shall be able to produce 10 video profiles, which may have the following properties:

          - Encoding type : H.265, H.264, MJPEG

          - Resolution :

            - 3M: 2048x1536, 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

            - 2M: 1920x1080, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

          - Frame rate

            - H.265 : Max. frame rates to 55fps @ 3M and 60fps @ 2M

            - H.264 : Max. frame rates to 50fps @ 3M and 50fps @ 2m

            - MJPEG : Max. frame rates to 5fps @ 3M and 2M

          - Smart Codec : Manual(5ea area), WiseStream Ⅱ

          - Bit rate control

            - H.265 and H.264 : target bitrate level control, constant bit rate (CBR) or variable bit rate (VBR)

            - MJPEG : quality level control, variable bit rate (VBR)

          - Number of multi-streaming profiles : Up to 10 profiles

          - Streaming method : Unicast / Multicast

          - Simultaneous users (total) : 20 maximum (unicast)

      - Storage and Recording

        - The camera shall have onboard SD card storage.

          - Card type : Micro SD/SDHC/SDXC

          - Capacity : Up to  32GB (1 slot)

          - Image content on the card shall have the ability to be downloaded to a selected destination.

      - Interoperability - Video streams shall be capable of supporting ONVIF protocol, profiles S / G / T.

      - Memory : 2048MB RAM, 256MB Flash

    - Network

      - Connectivity : RJ-45(10/100BASE-T)

      - Protocols supported :

        - Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

        - Configuration: Dynamic Host Configuration Protocol (DHCP)

        - Web services: Hypertext Transfer Protocol (HTTP), Secure HTTP (HTTPS)

        - Security: Secure Sockets Layer(SSL)/Transport Layer Security(TLS)

        - Network services: Address Resolution Protocol (ARP), Bonjour, Domain Name System (DNS), Internet Control Message Protocol (ICMP), Link Layer Discovery Protocol(LLDP), Network Time Protocol (NTP), Protocol Independent Multicast-Sparse Mode (PIM-SM), Simple Network Management Protocol (SNMP v1/2c/3 – MIB-2), Universal Plug and Play (UPnP)

        - Media: Real-Time Transport Protocol (RTP), Real-Time Control Protocol, Real-Time Streaming Protocol (RTSP), Secure Real-Time Transport Protocol(SRTP)

        - Multicast: Internet Group Management Protocol (IGMP)

        - Notifications: File Transfer Protocol (FTP), Simple Mail Transfer Protocol (SMTP)

        - Remote Access: PPPoE

      - DDNS

        - The camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

      - Quality of Service (QoS)

        - Layer 3 DSCP

      - Security features

        - HTTPS(SSL) Login Authentication

        - Digest Login Authentication

        - IP Address Filtering

        - User access log

        - 802.1X Authentication(EAP-TLS, EAP-LEAP)

        - Device Certificate(Hanwha Techwin Root CA, pre-installed)

      - Application Programming Interface

        - ONVIF Profile S / G / T, SUNAPI(HTTP API), Wisenet open platform

      - Discovery

        - Manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.

    - Audio

      - Audio In

        - Selectable (Microphone IN/Line IN)

      - Audio Out : Line out

    - Electrical

      - Input Voltage : PoE (IEEE 802.3af, Class3), 24VAC, 12VDC

      - Power consumption

        - PoE : Max 50W, Typical 27W

        - 12VDC : Max 47.5W, Typical 25W24VAC : Max 10.5W, typical 8.9W

    - Mechanical And Environmental

      - Material : White / Aluminum

      - Dimensions (W x H)  : W186.9 x D293.7 x H259.3 (mm) / 4.8kg (10.5 lb)

      - Weight : 4.8kg(10.5 lb)

      - Temperature And Humidity

        - Normal : -40°C~+55°(-40°F ~ +131°F) / Intermittent : -40°C~+60°C(-40°F ~ +140°F)

        - Cold start : -40°C

        - Maximum Temperature based on NEMA-TS 2(2.2.7) : +74°C(+165°F) Less than 95% RH(Non-condensing)

      - Environmental Rating

        - IP66

        - IK10

        - NEMA4X

      - RAL Code

        - RAL9003

    - Wisenet Road AI LPR

      - Solution

          - Speed Description

            - City Traffic Observation : Regular Speed

            - Highways : High Speed

          - Lane Coverage

            - City Traffic Observation and Highways : Up to 2 lanes

          - Speed Limit

            - City Traffic Observation : Up to 120kmh(75mph) for 2 lanes

            - Highways : Up to 200kmh(125mph) for 2 lanes

          - Min. Forward Distance

            - City Traffic Observation : 16m(52ft)

            - Highways : 27m(90ft)

          - Max. Forward Distance

            - City Traffic Observation and Highways : 46m (150ft)

            - HIghways : 46m (150ft)

          - Max Horizontal Angle

            - City Traffic Observation : 25°

            - Highways : 15°

          - Max Vertical Angle

            - City Traffic Observation : 25°

            - Highways : 15°

          - Horizontal Offset

            - City Traffic Observation : Up to 7m (24ft)

            - Highways : Up to 4m(12ft)

          - Camera Height

            - City Traffic Observaction : Up to 7m(24ft)

            - Highways : Up to 7m(24ft)

          - Vehicle Recognition

            - City Traffic Observation and Highways

              - Make : 70+

              - Model : 600+

              - Color : 10

          - Available Countries

            - City Traffic Observation and Highways

              - Albania, Armenia, Austria, Belarus, Belgium, Bosnia and Herzegovina, Bulgaria, Cyprus, Czechia, Denmark, Estonia, Finland, France, Greece, Germany, Hungary, Ireland, Iceland, Italy, Lithuania, Luxembourg, Latvia, Montenegro, North Macedonia, Malta, Netherlands, Norway, Poland, Portugal, Romania, Serbia, Slovenia, Slovakia, Spain, Switzerland, Turkey, United Kingdom, United States of America, Canada, Mexico

END OF SECTION

8