Hanwha Vision is a global leading supplier of solutions for IP and analog video surveillance. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-effectiveness. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit https://www.hanwhavision.com

- PRODUCTS

  - EQUIPMENT

    - Manufacturer : Hanwha Vision

    - Model : TNV-C7013RC

    - Alternates :	None

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously.

        - H.265 and H.264 – Max. 30fps/25fps(60Hz/50Hz)

        - MJPEG – Max. 30fps(@3MP Max. 5fps)

      - The camera shall be able to configure various resolution selections.

        - 2048x1536, 1920x1080, 1600x1200, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

      - The camera shall support multiple video streaming up to 10 profiles.

      - The camera shall support unicast video streaming up to 20 users.

      - The camera shall support multicast video streaming.

      - The camera shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

      - The camera shall provide WiseStreamⅡ, WiseStreamⅢ based on AI engine, Dynamic GOV and FPS to efficiently manage bit rate of the video stream and reduce storage.

      - The camera shall provide WiseNRⅡ that working based on AI engine for reduce noise and blur on image.

    - Camera – The camera device shall have the following physical and performance properties:

      - IP66, IP6K9K, IK10+ rated for protection against impacts

      - Auto day/night operation with removable IR cut filter

      - 2D and 3D digital noise reduction

      - Integral IR illumination, providing effective visibility of 15m(49.21ft) at 0 Lux when activated in Black & White mode

      - 32 privacy masking regions utilizing 4 points quadrangle

    - Intelligence and Analytics – The camera shall have a suite of integral intelligent operations and analytic functions to include:

      - Analytics

        - Defocus detection, Motion detection, Tampering, Audio detection, Sound classification, Virtual area (Appear / Disappear), Clothes color detection ( Orange / Black / Red, @400lux)

        - Motion detection with 8 definable detection areas with 8 point polygonal zones, and minimum/maximum object size.

        - Audio detection and sound classification (Scream / Gunshot / Explosion / Crashing glass)

      - Analytics based on AI engine

        - Object detection : Person / Face / Vehicle (car/bus/truck/motorcycle/bicycle) / License plate

        - IVA : Virtual line (Crossing / Direction), Virtual area (Loitering / Intrusion / Enter / Exit)

        - Support DetectionShot

    - Interoperability – The camera shall be ONVIF Profile S / G / T / M compliant.

    - The camera shall possess the following further characteristics:

      - Built-in web server, accessed via standard browsers including MS Edge, Firefox, Chrome and Safari

      - Micro SD/SDHC/SDXC memory card and NAS recording options, with configurable pre-alarm and post-alarm recording intervals

      - Bi-directional audio

      - Alarms and notifications

        - Alarm notification triggers : Analytics, Network disconnect, Alarm input (via optional SPM-4210 I/O box), App event, Time schedule, MQTT subscription

        - Available notification means upon trigger : File upload via FTP and e-mail, Notification via e-mail, SD/SDHC/SDXC or NAS recording, Alarm output (via optional SPM-4210 I/O box), Handover, Audio clip playback, MQTT publication

      - Pixel Counter available in the plug-in web viewer

      - POE capable

      - NDAA  Compliant

  - DETAILED SPECIFICATIONS

    - Video

      - Image Sensor : 1/1.8" CMOS

      - Scanning : progressive

      - Minimum illumination

        - Color mode : 0.04Lux(F2.0, 1/30sec, 30IRE)

        - Black & white mode (IR LED off) : 0.04Lux(F2.0, 1/30sec, 30IRE)

        - Black & white mode (IR LED on) : 0Lux

    - Lens

      - Focal length : 2.39mm fixed focal

      - Max. Aperture Ratio : F2.0

      - FoV : H 128˚ / V 100˚

      - Min. Object Distance : 0.5m (1.64ft)

      - Focus Control : Manual

      - Lens Type : Fixed

    - Pan /Tilt / Rotate

      - Range : - / -  / -

    - Operational Functions

      - Camera Title

        - Off / On (Displayed up to 85 characters)

        - English / Numeric / Special characters

      - Day/night : Auto (ICR)

      - IR Viewable Length : 15m (49.21ft), 940nm

      - Backlight compensation : BLC, HLC, WDR, SSDR

      - WDR : extremeWDR (120dB)

      - Digital Noise Reduction (DNR) : WiseNRⅡ(Based on AI engine), SSNRⅤ

      - Digital Image Stabilization : Support

      - Motion Detection : Off / On (8ea, 8 points polygonal zones)

      - Privacy Masking : Off / On (32ea, 4 points quadrangle zones)

      - Gain Control : Off / Max Gain / Manual

      - White Balance : ATW / Narrow ATW / AWC / Manual / Indoor / Outdoor

      - Lens Distortion Compensation : None

      - Electronic shutter speed

        - Minimum / Maximum / Anti flicker (2 ~ 1/12,000sec)

        - Prefer shutter control(Based on AI engine)

      - Digital PTZ : Support (Preset, Group)

      - Video Rotation : Flip, Mirror, Hallway view (90° / 270°)

      - Analytics

        - Defocus detection, Motion detection, Tampering, Audio detection, Sound classification, Virtual area (Appear / Disappear), Clothes color detection (Orange / Black / Red, @400lux)

        - Motion detection with 8 definable detection areas with 8 point polygonal zones, and minimum/maximum object size.

        - Audio Detection and sound classification (Scream / Gunshot / Explosion / Crashing glass)

      - Analytics based on AI engine

        - Object detection : Person / Face / Vehicle (car/bus/truck/motorcycle/bicycle) / License plate

        - IVA : Virtual line (Crossing / Direction), Virtual area (Loitering / Intrusion / Enter / Exit)

        - Support DetectionShot

      - Alarm I/O : 2 configurable I/O ports * Support Alarm via optional SPM-4210 I/O box

      - Alarm Triggers

        - Analytics

        - Network disconnect

        - Alarm input (via optional SPM-4210 I/O box)

        - App event

        - Time schedule

        - MQTT subscription

      - Alarm Events

        - File upload via FTP and e-mail

        - Notification via e-mail

        - SD/SDHC/SDXC or NAS recording at Event Triggers

        - Alarm output (via optional SPM-4210 I/O box)

        - Handover : PTZ preset, send message by HTTP / HTTPS / TCP

        - Audio clip playback

        - MQTT publication

      - Memory : 4GB RAM, 512MB Flash

    - Video Streams

      - Video compression

        - H.265/H.264 : Main/Baseline/High

        - MJPEG

      - Resolution : 2048x1536, 1920x1080, 1600x1200, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

      - Frame rate

        - H.265 and H.264 : Max. 30fps/25fps(60Hz/50Hz)

        - MJPEG : Max. 30fps(@3MP Max. 5fps)

      - Smart Codec : Manual(5ea area), WiseStream Ⅱ, WiseStreamⅢ(Based on AI engine)

      - Bit rate control

        - H.265 and H.264 : CBR or VBR

        - MJPEG : VBR

      - Streaming Capability : Multiple streaming (Up to 10 profiles)

      - Simultaneous users (total)

        - 20 maximum (unicast)

        - 128 maximum (multicast)

      - Storage and Recording

        - The camera shall have onboard SD card storage.

          - Card type : Micro SD/SDHC/SDXC

          - Capacity : Up to  1TB (1 slot)

        - NAS

      - Interoperability : ONVIF protocol, profiles S / G / T / M.

      - Single Images - The camera shall support jpg file image screenshot and export.

    - Network

      - Connectivity : RJ-45(10/100BASE-T)

      - Protocols supported :

        - IPv4 / IPv6, TCP, UDP

        - Configuration: DHCP, LLDP

        - Web service: HTTP, HTTPS

        - Network Service: ARP, Bonjour, DNS, ICMP, NTP, SNMP v1/v2c/v3(MIB-2), UPnP

        - Media: RTP, RTCP, RTSP

        - Unicast: SRTP

        - Multicast: IGMP

        - Notifications: FTP, SMTP

        - NTCIP

        - MQTT

      - DDNS - The camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

      - QoS - Layer 3 DSCP

      - Security features

        - TPM 2.0 (FIPS 140-2 level 2)

        - User password protection

        - The device shall not provide a manufacture default password. Initial password setting shall be required to access the camera.

        - A minimal level of password complexity shall be required by the camera.

        - The camera shall not have a manufacture back-door password.

        - The manufacturer shall provide a tool that provides the ability to make password changes to multiple cameras at the same time.

        - IP address filtering – List of allowed or blocked IP addresses.

        - SRTP secured video communication for multicast.

        - Digest login authentication.

        - User access log.

        - 802.1X Authentication (EAP-TLS, EAP-LEAP)

        - Device Certificate (Hanwha Vision Root CA)

        - HTTPS(SSL/TLS) login authentication and secured video communication with TLS v1.3

        - Disk encryption on SD card recording (AES-256)

        - File encryption on local disk recording via web browser (AES-256)

        - Mutual Authentication (Client Authentication) for secure communication

        - Secure OS and Security Boot

      - Application Programming Interface

        - ONVIF Profile S / G / T / M, SUNAPI(HTTP API), Wisenet open platform

      - Discovery – The manufacturer shall offer a discovery program to identify all devices of them on the network.

      - Configuration – The manufacturer shall offer a configuration program that remotely allows users to change settings on multiple cameras simultaneously.

      - Firmware upgrade – The manufacturer shall offer a program capable of upgrading multiple cameras at the same time (not requiring access to individual cameras).

      - Camera backup setting – The manufacturer shall provide a program that provides the ability to save multiple camera settings to a file and restore these camera settings if needed.

      - Reporting – The manufacturer shall provide a tool that can generate a report including thumbnail view, MAC address, IP address, serial number and other camera settings.

    - Audio

      - Audio In

        - Selectable (mic in / line in / built-in MIC)

        - Supply voltage: 2.5VDC(4mA), Input impedance: 2K Ohm * Support Audio via optional SPM-4210 I/O box

      - Audio Out

        - Line out, Max. output level: 1Vrms

      - Audio Compression

        - G.711 u-law /  G.726 Selectable,

        - G.726(ADPCM) 8KHz, G.711 8KHz

        - G.726: 16Kbps, 24Kbps, 32Kbps, 40Kbps,

        - AAC-LC: 48Kbps at 16KHz

    - Electrical

      - Input Voltage : PoE (IEEE802.3af, Class2), 12VDC

      - Power consumption

        - PoE : Max 12.95W, Typical 9.7W

        - 12VDC : Max. 12.7W, Typical 9.0W

    - Mechanical And Environmental

      - Color / Material : White / Aluminum

      - RAL Code : RAL9003

      - Dimensions  : 198.3(W)x104.8(H)x198.6(D)mm(7.87"x4.13"x7.82")

      - Weight : 1.935kg (4.27lb)

      - Temperature And Humidity

        - Operating : -10°C ~ +55°C(+14°F ~ +131°F) / Less than 90% RH (Non-condensing)

        - Storage : -50°C ~ +60°C(-58°F ~ +140°F) / Less than 90% RH (Non-condensing)

      - Environmental Rating : IP66, IP6K9K, IK10+

    - DORI (EN62676-4 standard)

      - Detect : 35.1m(115.162ft)

      - Observe : 14m(45.93ft)

      - Recognize : 7.0m(22.97ft)

      - Identify : 3.5m(11.48ft)

    - EMC & Safety

      - FCC

        - 47 CFR Part 15 Subpart B(ANSI C63.4a-2017 Class A)

        - ICES-003 Issue 7(ANSI C63.4a-2017 Class A)

      - CE

        - EMC-Directive 2014/30/EU(EN 55032:2015/A11:2020 Class A, EN 50130-4:2011)

      - UKCA

        - Regulations 2016(EN 55032:2015/A11:2020 Class A, EN 50130-4:2011)

      - RCM AS/NZS CISPR 32:2015 AMD 1:2020 Class A

      - VCCI CISPR 32:2016 Class A

      - KS C 9832 Class A, KS C9835

      - UL 62368-1, CSA C22.2 62638-1:19, IEC 62471

      - CE EN IEC 63000:2018 (hazardous substances)

END OF SECTION

8