Hanwha Techwin is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://security.hanwhatechwin.com/

- PRODUCTS

  - EQUIPMENT

    - Manufacturer : Hanwha Techwin (http://security.hanwhatechwin.com/)

    - Model : XNO-6123R

    - Alternates :	None

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The 2M camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously

        - H.265 and H.264 – frame rates to 120fps

          - Maximum frame rates available at selected WDR turn on/off:

          - WDR off : Max. 120fps/100fps(60Hz/50Hz)

          - WDR on : Max. 60fps/50fps(60Hz/50Hz)

        - MJPEG – frame rates to 30fps/25fps(60Hz/50Hz)

      - The 2M camera shall be able to configure up to 10 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - Resolution selections

1920x1080, 1600x1200, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

      - Simultaneous unicast access by up to 20 users

      - Multicast or unicast capable

      - Dynamic DNS (DDNS) support.

      - WiseStream and Dynamic GOV to efficiently manage bitrate of the video stream.

    - Camera – The 2M camera device shall have the following physical and performance properties:

      - IP66/67, IK10, NEMA4X, NEMA-TS 2(2.2.8, 2.2.9) rated for protection against impacts.

      - Auto day/night operation with removable IR cut filter

        - 30/25fps : Low light level operation to 0.03 lux (color), 0.003 lux (black and white) and 0 lux(IR LED on)

        - 60/50fps : Low light level operation to 0.06 lux (color), 0.006 lux (black and white) and 0 lux(IR LED on)

        - 120/100fps : Low light level operation to 0.12 lux (color), 0.012 lux (black and white) and 0 lux(IR LED on)

      - 2D and 3D digital noise reduction

      - Integral IR illumination, providing effective visibility of 90m at 0 Lux when activated in Black & White mode.

      - 32 privacy masking regions utilizing polygons

    - Intelligence and Analytics – The 2M camera shall have a suite of integral intelligent operations and analytic functions to include:

      - Motion detection with eight definable detection areas, minimum / maximum object size definition and a learning algorithm that ignores false alarms such as trees and waves on water.

      - Detection of logical events of specified conditions from the camera’s video input

        - Classified object type : Person/Face/Vehicle/License plate

        - Attributes : Vehicle(Type:car/bus/truck/motorcycle/bicycle)

        - Analytics events based on AI engine

          - Object detection, Virtual line*(Crossing/Direction),

Virtual area(Loitering/Intrusion/Enter/Exit)Analytics events

          - Defocus detection, Motion detection, Tampering, Fog detection, Audio detection, Sound classification, Shock detection,

        - Virtual area(Appear/Disappear)

    - Interoperability – The 2M camera shall be ONVIF Profile S / G / T / M compliant.

    - The 2M camera shall possess the following further characteristics:

      - Built-in web server, accessed via standard browsers including, MS Edge, Firefox, Chrome and Safari

      - Micro SD/SDHC/SDXC memory card and NAS recording options, with configurable pre-alarm and post-alarm recording intervals

      - Bi-directional audio

      - Alarms and notifications

        - alarm notification triggers:

          - alarm input

          - motion detection

          - video analytics

          - audio detection

          - network disconnect

        - available notification means upon trigger:

          - file upload via FTP and e-mail

          - notification via e-mail

          - record to local storage (SD/SDHC/SDXC card)

          - NAS recording at event triggers

          - external output

      - Pixel Counter available in the plug-in web viewer.

      - POE capable

  - DETAILED SPECIFICATIONS

    - Video

      - Image

        - Sensor : 1/2.8" 2M CMOS

          - pixels per sensor : 1920 (H) x 1080 (V)

          - scanning : progressive

        - Minimum illumination

          - Color mode : 0.03Lux(F1.6, 30IRE, 30/25fps), 0.06Lux(F1.6, 30IRE, 60/50fps), 0.03Lux(F1.6, 30IRE, 120/100fps)

          - Black & white mode (IR LED on) : 0.03Lux(F1.6, 30IRE, 30/25fps), 0.06Lux(F1.6, 30IRE, 60/50fps), 0.03Lux(F1.6, 30IRE, 120/100fps)

          - Black & white mode (IR LED on) : 0Lux

        - The following features with control settings shall be available:

          - Camera Title

            - Off / On (Displayed up to 85 characters per line)

            - English / Numeric / Special characters

            - Multi-line (Max. 5lines), Color (Grey / Green / Red / Blue / Black / White), Transparency, Auto scale by resolution

          - Day/night : Auto (ICR)

          - Backlight compensation : BLC, HLC, WiseDR

          - WDR : extremeWDR 150dB

          - Contrast Enhancement : Off / On (SSNR V)

          - Digital Image Stabilization: Support(built-in gyro sensor)

          - Defog : Support

          - Motion Detection : Off / On (8ea polygonal zones)

          - Privacy Masking : Off / On (32ea Quadrangle zones)

            - Color : Gray / Green / Red / Blue / Black / White

            - Mosaic

          - Gain Control : Support

          - White Balance : ATW / Narrow ATW / AWC / Manual / Indoor / Outdoor

          - Electronic shutter speed

            - Settings : Minimum / Maximum / Anti flicker (2 ~ 1/12,000sec)

            - prefer shutter control(Based on AI engine)

          - LDC: Support (Fill/stretch mode)

          - Image flip : Off / On

          - Image mirror : Off / On

          - Hallway view : Off / On (90°, 270°)

          - Alarm I/O : 2 configurable I/O ports, DC 12V output(Max. 50mA)

          - Alarm Triggers : Analytics, Network disconnect, Alarm input, App event, Time schedule

          - Alarm Events :

            - File upload(image) : e-mail/FTP

            - Notification : e-mail

            - Recording : SD/SDHC/SDXC or NAS recording at event triggers

            - Alarm output

            - Handover(PTZ preset, Send message by HTTP/HTTPS/TCP)

            - Audio clip playback

          - Pixel Counter : Available in the plug-in web viewer.

          - Video Out for installation

            - CVBS : 1.0 Vp-p / 75Ω composite, 720x480(N), 720x576(P), for installation

            - USB : Micro USB type B, 1280x720 for installation

        - Lens : 5.2~62.4mm(12x) zoom (digital 32x, total 384x zoom)

            - Max. Aperture Ratio: F1.6(Wide) - F3.0(Tele)

          - FoV

            - H: 54.58˚(Wide) ~ 5.30˚(Tele)  / V : 32.19˚(Wide) ~ 3.00˚(Tele)

            - Min. Object Distance: 1.5 m (4.92ft)

            - Lens Type: DC auto iris

            - Focus Control: Oneshot AF, Focus save

      - IR Viewable Length : 90m (295.28ft)

      - DORI (EN62676-4 standard)

        - Detect (25PPM/ 8PPF): Wide: 74.4m(244.19ft) / Tele: 829.7m(2721.97ft)

        - Observe (63PPM/ 19PPF): Wide: 29.8m(97.68ft) / Tele: 331.9m(1088.79ft)

        - Recognize (125PPM/ 38PPF): Wide: 14.9m(48.84ft) / Tele: 165.9m(544.39ft)

        - Identify (250PPM/ 76PPF): Wide: 7.4m(24.42ft) / Tele: 83.0m(272.20ft)

      - Video Streams

        - The 2M camera shall be able to produce 10 video profiles, each of which may have the following properties:

          - Encoding type:

            - H.265

            - H.264

            - MJPEG

          - Resolution:

            - 1920x1080, 1600x1200, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

          - Maximum frame rate:

            - H.265 and H.264 : 120fps

            - MJPEG : 30fps

          - Smart Codec : Manual(5ea area), WiseStreamⅡ, WiseStreamⅢ(Based on AI engine)

          - Bit rate control method :

            - H.265 and H.264 : target bitrate level control, constant bit rate (CBR) or variable bit rate (VBR)

            - MJPEG : quality level control, variable bit rate (VBR)

      - Number of multi-streaming profiles : 10 maximum

      - Simultaneous users (total) : 20 maximum (unicast)

      - Storage and Recording

        - The 2M camera shall have onboard SD card storage.

          - Card type : Micro SD/SDHC/SDXC

          - Capacity : Up to 1 TB (512GB x 2)

          - Image content on the card shall have the ability to be downloaded to a selected destination.

        - NAS

      - Interoperability - Video streams shall be capable of supporting ONVIF protocol, profiles S/G/T/M.

      - Single Images - The 2M camera shall support jpg file image screenshot and export.

      - Memory - 2GB RAM, 512MB Flash

    - Network

      - Connectivity : Metal shielded RJ-45(10/100/1000BASE-T)

      - Protocols supported :

        - Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

        - Configuration: Dynamic Host Configuration Protocol (DHCP)

        - Web services: Hypertext Transfer Protocol (HTTP), Secure HTTP (HTTPS)

        - Security: Secure Sockets Layer(SSL)/Transport Layer Security(TLS)

        - Network services: Address Resolution Protocol (ARP), Bonjour, Domain Name System (DNS), Internet Control Message Protocol (ICMP), Link Layer Discovery Protocol(LLDP), Network Time Protocol (NTP), Protocol Independent Multicast-Sparse Mode (PIM-SM), Simple Network Management Protocol (SNMP v1/2c/3 – MIB-2), Universal Plug and Play (UPnP)

        - Media: Real-Time Transport Protocol (RTP), Real-Time Control Protocol, Real-Time Streaming Protocol (RTSP), Secure Real-Time Transport Protocol(SRTP)

        - Multicast: Internet Group Management Protocol (IGMP)

        - Notifications: File Transfer Protocol (FTP), Simple Mail Transfer Protocol (SMTP)

        - Remote Access: Point-to-Point Protocol over Ethernet) (PPPoE)

      - DDNS

        - The 2M camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

      - Quality of Service (QoS)

        - Layer 3 DSCP

      - Security features

        - TPM 2.0 (FIPS 140-2 level 2)

        - HTTPS(SSL) Login Authentication

        - Digest Login Authentication

        - IP Address Filtering

        - User access log

        - 802.1X Authentication(EAP-TLS, EAP-LEAP, EAP-PEAP MSCHAPv2)

        - Device Certificate(Hanwha Techwin Root CA, pre-installed)

        - Secure by default certificate

        - Secure OS/Boot/Storage, Verify firmware forgery

      - Application Programming Interface

        - ONVIF Profile S/G/T, SUNAPI(HTTP API), Wisenet open platform

      - Discovery

        - Manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.

    - Audio

        - Direction: bi-directional

      - Audio In

        - Selectable (Mic in / Line in), Supply voltage: 2.5V DC (4mA)

        - Input impedance: approx. 2K Ohm

      - Audio Out

        - Line out (3.5mm mono jack), Max output level: 1Vrms

      - Compression

        - G.726 (ADPCM) 8KHz

        - G.711 8KHz

        - G.726: 16Kbps, 24Kbps, 32Kbps, 40Kbps

        - AAC-LC: 48Kbps at 16KHz

    - Electrical

        - Input Voltage: PoE+(IEEE802.3at, Class4), 12VDC

      - Power consumption

        - PoE+: Max 19.5W, typical 15.5W

        - 12VDC: Max 16.5W, typical 13.1W

        - Power redundancy failover

    - Mechanical And Environmental

      - Material

        - White / Aluminum

        - Hard-coated window

      - Dimensions (W x H)

        - Ø93.4x298.4mm(3.68"x11.75")

      - Weight

        - 2300g(5.07 lb)

      - Temperature/Humidity:

          - Operating: -40°C~+60°C(-40°F ~ +140°F) / Less than 95% RH(Non-condensing)

          - Storage: -50°C~+60°C (-58°F~+140°F) / Less than 95% RH(Non-condensing)

      - Environmental Rating

        - IP66/67

        - IK10

        - NEMA4X

        - NEMA-TS 2(2.2.8, 2.2.9)

      - RAL Code: RAL9003

      - Compatible Conduit hole / Gangbox

        - 3/4" (M25)

        - single, double, 4" octagon, 4" square

    - EMC & Safety

      - 47 CFR Part 15, Subpart B

        - ANSI C63.4a-2017 Class A

      - IC Regulation ICES-003:2016

        - ANSI C63.4a-2017  Class A

      - CE EMC-Directive 2014/30/EU and  Regulations 2016/1091

        - EN 55032:2015/A11:2020 Class A

        - BS EN 55032:2015/A11:2020 Class A

        - EN 50130-4:2011

        - BS EN 50130-4:2011/A1:2014

        - EN 61000-3-2:2014

        - BS EN 61000-3-2:2014

        - EN 61000-3-3:2013

        - BS EN 61000-3-3:2013

        - EN 50581:2012 (RoHS)

      - VCCI-CISPR 32:2016 Class A

      - AS/NZS CISPR32:2015 Class A

      - UL listed

      - KS C IEC 60529:2013 (IP66/67)

      - NEMA 250-2014 Enclosures for Electrical Equipment (1000 Volts Maximum)

      - KS C IEC 62262: 2005 (IK10)

END OF SECTION

8