Hanwha Techwin is a global leading supplier of solutions for IP and analog video surveillance. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-effectiveness. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://www.hanwha-security.com/

- PRODUCTS

  - EQUIPMENT

    - Manufacturer : Hanwha Techwin(http://www.hanwha-security.com/)

    - Model : XNV-6083RZ

    - Alternates : None

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously.

        - H.265/H.264 : Max. 120/100fps(60Hz/50Hz, WDR off), 120/100fps(60Hz/50Hz, WDR on)

        - MJPEG : Max. 30/25fps(60Hz/50Hz)

      - The camera shall be able to configure various resolution selections.

        - 16:9 aspect ratio : 1920x1080, 1280x720, 800x448,  640x360

        - 4:3 aspect ratio : 1280x960, 1024x768, 800x600, 640x480, 320x240

        - 5:4 aspect ratio : 1280x1024, 720x576

        - 3:2 aspect ratio : 720 x 480

      - The camera shall support unicast video streaming up to 20 users.

      - The camera shall support multicast video streaming.

      - The camera shall support multiple video streaming up to 10 profiles.

      - The camera shall support 3 virtual channel support

      - The camera shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

      - The camera shall provide WiseStream Ⅱ, WiseStreamⅢ based on AI engine, Dynamic GOV and FPS to efficiently manage bit rate of the video stream and reduce storage.

      - The camera shall provide WiseNRⅡ that working based on AI engine for reduce noise and blur on image

    - Camera – The camera device shall have the following physical and performance properties:

      - IK10+ rated for protection against impacts.

      - True day/night operation with removable IR cut filter

        - Low light level operation to 0.01 lux at F1.4 in color mode and 0.001 lux at F1.4 in black and white mode. (F1.4, 1/30sec, 30IRE), (30/25fps)

        - Black and White mode operation to 0 lux with IR LED on.

      - The camera shall be able to produce clear images in highly contrast scenes with multi-exposure wide dynamic range up to 120dB.

      - The camera shall support digital noise reduction using both 2D and 3D noise reduction technology.

      - The camera shall be able to configure 32 privacy masking areas with Quadrangle zones.

      - The camera shall have the defog feature to remove fogginess of scene which can be triggered automatically from the fog detection event.

      - The camera shall provide video display on smart phone (iPhone, Android) to adjust viewing angle, pan, tilt, rotation and focus.

    - Intelligence and Analytics – The camera shall have a suite of intelligent analytic functions.

      - Analytics events based on AI engine

        - Object detection : Person/Face/Vehicle(car/truck/bus/bicycle/motorcycle)/License plate

        - Support DetectionShot

        - Virtual line(Crossing/Direction), Virtual Area(Loitering/intrusion/Enter/Exit)

      - Analytics events : Defocus detection, Motion detection, Tampering, Fog detection, Shock detection, Virtual area(Appear/Disappear), *Audio detection/Sound classification (with I/O box SPM-4210)

        - Motion detection with 8 definable detection areas with 8 point polygonal zones, and minimum/maximum object size.

        - Detection and classification of the following sound.

          - Scream

          - Gunshot

          - Explosion

          - Crashing glass

    - Interoperability – The camera shall be ONVIF Profile S / G and T compliant.

    - The camera shall possess the following further characteristics:

      - Micro SD/SDHC/SDXC memory card with configurable pre-alarm and post-alarm recording intervals

      - NAS recording option with configurable pre-alarm and post-alarm recording intervals

      - Alarms and notifications

        - alarm notification triggers

          - Analytics

          - Network disconnect

          - Alarm input *with NW I/O Box SPM-4210

          - App event

          - Time schedule

        - available notification means upon trigger

          - File upload(image) : e-mail/FTP

          - Notification : e-mail

          - Recording : SD/SDHC/SDXC or NAS recording at event triggers

          - Alarm output *with NW I/O Box SPM-4210

          - Handover(PTZ preset, Send message by HTTP/HTTPS/TCP)

      - Pixel Counter available in the web viewer.

      - IP66/IP67/IP6K9K, IK10+, NEMA4X

      - This device has been verified using STP cable. The use of appropriate GND grounding and STP cable is recommended to effectively protect your product and property from transient voltage, thunder stroke, communication interruption.

      - NDAA Compliant

  - DETAILED SPECIFICATIONS

    - Video

      - Imaging device : 1/2.8" CMOS

      - Scanning : Progressive

      - Minimum Illumination

        - Color : 0.01Lux(F1.4, 1/30sec, 30IRE)(30/25fps), 0.02Lux(F1.4, 1/60sec, 30IRE)(60/50fps), 0.04Lux (F1.4, 1/120sec, 30IRE)(120/100fps)

        - B/W : 0.001Lux(F1.4, 1/30sec, 30IRE)(30/25fps), 0.002Lux(F1.4, 1/60sec, 30IRE)(60/50fps), 0.004Lux (F1.4, 1/120sec, 30IRE)(120/100fps), 0Lux (IR LED on)

    - Lens

      - Focal length : 2.8~12m(4.3x) motorized varifocal

      - Max. Aperture Ratio : F1.4(Wide)~3.6(Tele)

      - Field of View

        - H : 120°(Wide)~27°(Tele)

        - V : 63°(Wide)~15°(Tele)

        - D : 143°(Wide)~32°(Tele)

      - Min. Object Distance : 0.5m(1.64ft)

      - Focus Control : Simple focus, Manual

      - Lens Type : DC auto iris with hall sensor (IR corrected)

    - Pan/Tilt/Rotate Range

      - Pan/Tilt/Rotate Rage : 0°~350° / 0°~85° / 0°~340° Remote adjustment (Max. 200cycles)

    - Operational Functions

      - IR Viewable Length : IR 15m (49.21ft), 30m(98.43ft) based on scene

      - Camera Title

        - Off / On (Displayed up to 85 characters)

        - English / Numeric / Special characters

        - Multi-line (Max. 5lines), Color (Grey / Green / Red / Blue / Black / White), Transparency, Auto scale by resolution

      - Day/Night Setting : Auto (ICR) / Color / B/W / External / Schedule

      - Backlight Compensation : Off / BLC / HLC / extremeWDR(120dB)

      - Contrast Enhancement : Off / On (SSDR)

      - Digital Noise Reduction : Off / On (SSNRⅤ, WiseNR Ⅱ(using AI engine))

      - Digital Image Stabilization : Off / On

      - Defog : Off / Auto / Manual

      - Motion Detection : Off / On (8ea, 8-point polygonal zones)

      - Privacy Masking : Off / On (32ea, Quadrangle zones)

        - Color: Gray, Green, Red, Blue, Black, White

        - Mosaic

      - Gain Control : Off / Manual / Max Gain

      - White Balance : ATW / Narrow ATW / AWC / Manual / Indoor / Outdoor

      - LDC : Support (Fill/stretch mode)

      - Electronic Shutter Speed : Min / Max / Anti-flicker (2 ~ 1/12,000sec)

        - Auto Prefer shutter control based on AI engine

      - Image Rotation

        - Flip : Off / On

        - Mirror : Off / On

        - Hallway view (90˚ / 270˚)

      - Alarm Triggers

        - Analytics

        - Network disconnect

        - Alarm input *with NW I/O Box SPM-4210

        - App event

        - Time schedule

      - Alarm Events

        - File upload(image) : e-mail/FTP

        - Notification : e-mail

        - Recording : SD/SDHC/SDXC or NAS recording at event triggers

        - Alarm output *with NW I/O Box SPM-4210

        - Handover (PTZ preset, Send message by HTTP/HTTPS/TCP)

      - Pixel Counter : Support

      - Storage : Micro SD/SDHC/SDXC 2slot Max. 1TB (512GB x 2)

      - AI Intelligent Analytics

        - Object detection : Person, Face, License plate, Vehicle (car/truck/bus/bicycle/motorcycle)

        - IVA : Virtual line(Crossing/Direction), Virtual Area(Loitering/intrusion/Enter/Exit)

      - Intelligent Analytics : Defocus detection, Motion detection, Tampering, Fog detection, Shock detection, Appear/Disappear, *Audio detection/Sound classification (with I/O box SPM-4210)

      - Memory : 2GB RAM, 512MB Flash

    - Video Streams

      - Video compression : H.265, H.264, MJPEG

      - Resolution : 1920x1080, 1600x1200, 1280x1024, 1280x960, 1280x720, 1024x768, 800x600, 800x448, 720x576, 720x480, 640x480, 640x360, 320x240

      - Maximum Framerate

        - H.265 / H.264 : Max. 120/100fps(60Hz/50Hz, WDR off), 120/100fps(60Hz/50Hz, WDR on)

        - MJPEG : Max. 30/25fps(60Hz/50Hz)

      - Smart Codec

        - Manual(5ea area)

        - WiseStreamⅡ

        - WiseStreamⅢ(Based on AI engine),

      - Bitrate Control Method

        - H.265 / H.264 : CBR or VBR

        - MJPEG : VBR

      - Streaming Capability : Multiple streaming (Up to 10profiles , 3 virtual channel support)

      - Streaming method : Unicast / Multicast

      - Simultaneous Users : 20 maximum (Unicast)

      - Interoperability

        - ONVIF Profile S / G / T

        - SUNAPI(HTTP API)

        - Wisenet Open Platform

    - Network

      - Connectivity : Metal Shield RJ-45(10/100/1000BASE-T)

      - Protocol

        - IP v4 / v6, TCP, UDP

        - Configuration: DHCP, LLDP

        - Web service: HTTP, HTTPS

        - Network Service: ARP, Bonjour, DNS, ICMP, NTP, PIM-SM, SNMP v1/v2c/v3 – MIB-2, UPnP

        - Media: RTP, RTCP, RTSP

        - Unicast: SRTP

        - Multicast: IGMP, PIM-SM

        - Notifications: FTP, SMTP

      - DDNS – The camera shall support DDNS services offered by the manufacturer and others publicly available service offerings

      - QoS – Layer 3 DSCP

      - Security Feature

        - TPM 2.0 (FIPS 140-2 level 2) Validated

        - User password protection

        - The device shall not provide a manufacture default password. Initial password setting shall be required to access the camera.

        - A minimal level of password complexity shall be required by the camera.

        - The camera shall not have a manufacture back-door password.

        - The manufacturer shall provide a tool that provides the ability to make password changes to multiple cameras at the same time.

        - IP address filtering – List of allowed or blocked IP addresses.

        - SRTP secured video communication for multicast.

        - Digest login authentication.

        - User access log.

        - 802.1X Authentication (EAP-TLS, EAP-LEAP)

        - Device Certificate (Hanwha Techwin Root CA)

        - HTTPS(SSL/TLS) login authentication and secured video communication with TLS v1.3

        - Disk encryption on SD card recording (AES-256)

        - File encryption on local disk recording via web browser (AES-256)

        - Mutual Authentication (Client Authentication) for secure communication

        - Secure OS and Security Boot

      - Discovery – The manufacturer shall offer a discovery program to identify all devices of them on the network.

      - Configuration – The manufacturer shall offer a configuration program that remotely allows users to change settings on multiple cameras simultaneously.

      - Firmware upgrade – The manufacturer shall offer a program capable of upgrading multiple cameras at the same time (not requiring access to individual cameras).

      - Camera backup setting – The manufacturer shall provide a program that provides the ability to save multiple camera settings to a file and restore these camera settings if needed.

      - Reporting – The manufacturer shall provide a tool that can generate a report including thumbnail view, MAC address, IP address, serial number and other camera settings.

    - Electrical

      - Power

        - Input Voltage / Current

          - PoE+ (IEEE 802.3at type2, Class4)

        - Power Consumption

          - PoE+: Max 22.5W, typical 11.0W

    - Mechanical And Environmental

      - Color/Material : White / Aluminum, Hard-coated dome bubble

      - RAL code : RAL9003

      - Dimensions (W x H) : Ø180x135mm(7.09x5.31")

      - Weight : 2Kg (4.41lb)

      - Temperature

        - Operating : -50°C ~ +55°C (-58°F ~ +131°F)

          - NEMA TS-2(2.2.7): 74°C(165°F )

          - Start up should be done at above -30°C

        - Storage : -50°C ~ +60°C (-58°F ~ +140°F)

      - Humidity

        - Operating : Less than 95% RH(non-condensing)

          - Humidity control /w GORE vent

        - Storage : Less than 90% RH

      - Ingression Protection : IP66/IP67/IP6K9K, NEMA4X

      - Vandal Resistance : IK10+

      - Compatible Conduit hole/Gangbox

        - 19.1mm(3/4”)(M25)

        - Single, double, 4” octagon, 4” square

    - DORI

      - Detect (25PPM/ 8PPF) : Wide: 22.2m(72.74ft) / Tele: 159.9m(524.76ft)

      - Observe (63PPM/ 19PPF) : Wide: 8.9m(29.09ft) / Tele: 64.0m(209.90ft)

      - Recognize (125PPM/ 38PPF) : Wide: 4.4m(14.55ft) / Tele: 32.0m(104.95ft)

      - Identify (250PPM/ 76PPF) : Wide: 2.2m(7.27ft) / Tele: 16.0m(52.48ft)

    - EMC & Safety

      - FCC

        - 47 CFR Part 15 Subpart B(ANSI C63.4a-2017 Class A)

        - ICES-003 Issue 7(ANSI C63.4a-2017 Class A)

      - CE

        - EMC-Directive 2014/30/EU(EN 50121-4:2016)

      - UKCA

        - EMC-Directive 2014/30/EU(EN 55032:2015/A11:2020 Class A, EN 50130-4:2011)

        - Regulations 2016(EN 55032:2015/A11:2020 Class A, EN 50130-4:2011)

      - RCM AS/NZS CISPR 32:2015 AMD 1:2020 Class A

      - VCCI CISPR 32:2016 Class A

      - KS C 9832 Class A, KS C9835

      - UL 62368-1, CSA C22.2 62638-1:19, IEC 62471

      - CE EN 50581:2012 (hazardous substances)

END OF SECTION

10