April 2025

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2016 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

Section 28 21 00: Video Security

Section 28 21 13: IP Cameras

4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV

- – GENERAL

  - SUMMARY

    - Section Includes

      - Section 28 21 17: Video Security – Security Cameras – Camera Housings

      - Section 28 21 19: Video Security – Security Cameras – Camera Mounts

      - Section 28 21 21: Video Security – Security Cameras – Illuminators

      - Section 28 27 00: Video Security – Video Security Sensors

    - Related Sections

      - [Section 28 33 15: Security Detection, Alarm and Monitoring – Security Monitoring and Control – Security Monitoring and Control Software].

**********Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

    - HD standards

      - Complies with the SMPTE 274M-2008 Standard in:

        - Resolution: 1920x1080

        - Scan: Progressive

        - Color representation: complies with ITU-R BT.709

        - Aspect ratio: 16:9

        - Frame rate: 25 and 30 frames/s

      - Complies with the 296M-2001 Standard in:

        - Resolution: 1280x720

        - Scan: Progressive

        - Color representation: complies with ITU-R BT.709

        - Aspect ratio: 16:9

        - Frame rate: 25, 30, 50 and 60 frames/s

        - Interference-Causing Equipment Standards

  - SYSTEM DESCRIPTION

    - Section Includes

      - Video Security – Security Cameras – IP Cameras

    - Performance Requirements

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall be a full-featured network bullet camera designed for discrete video security applications in indoor and outdoor environments.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall use one (1) high performance 1/1.8-in. 4MP Progressive-scan CMOS sensor.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer four (4) dual-core (IR and white light) LEDs to produce color images in total darkness.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer advanced analytics that detect and categorize between human and vehicular objects and technology that reduces the number of false alarms.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer active alarm functionality to respond automatically to a triggered alarm.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall include a built-in microphone.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer an Audio Out for an external speaker to be added.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall provide direct network connection using Smart Encoding H.265+, H.265, Smart Encoding H.264+, H.264, MJPEG (Sub Stream Only) compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall support the following dual, redundant power options:

        - 12 VDC

        - PoE (802.3at)

        - LumiPower

        - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall default to use power from the PoE power supply, if connected.

        - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall reboot and switch to the 12 VDC power supply if power from the PoE power supply is lost.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer Wide Dynamic Range (130 dB) for clear images in extreme high-contrast environments.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer three (3) separate and configurable streams with individually configurable 2688 × 1520, D1, and 1080p streams.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall have a motorized vari-focal lens with a lens length of 2.7 to 12 mm.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall combine temperature-tolerant components with a waterproof enclosure to ensure flawless operation in temperatures as low as –30° C (–22° F).

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV housing shall conform to the IP67 and IK10 standard.

  - SUBMITTALS

Submit under provisions of Section [01 33 00.]

    - Product Data:

      - Manufacturer’s data, user and installation manuals for all equipment and software programs including computer equipment and other equipment required for complete video management system.

    - Dimensional Drawings; include

      - Overall device dimensions.

      - Dimensions specific for installation.

    - Closeout Submittals

      - User manual.

      - Parts list.

      - Maintenance requirements.

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [10] years of experience in manufacture and design Video Security Devices.

    - Video Security System:

      - List certifying bodies (UL, CSA, etc.)

      - Provide evidence of compliance upon request.

    - Installer:

      - Minimum of [5] years of experience installing Video Security System.

  - DELIVERY, STORAGE AND HANDLING

    - Comply with requirements of Section 01 60 00.

    - Deliver materials in manufacture’s original, unopened, undamaged containers; and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users.

    - Provide factory direct technical support via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - [Acceptable Manufacturer:

Luminys Systems Corporation

15245 Alton Pkwy, #100, Irvine, CA, 92618

Tel: (949) 679-7777

Fax: (949) 679-5760

Email: sales@luminyscorp.com]

    - Substitutions: [Not permitted.] [Under provisions of Division 1.]

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

  - PRODUCT SPECIFICATIONS

    - General Characteristics:

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall be a full-featured network dome camera designed for discrete video security applications in indoor and outdoor environments.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall use one (1) high performance 1/1.8-in. 4MP Progressive-scan CMOS sensor.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer four (4) dual-core (IR and white light) LEDs to produce color images in total darkness.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer advanced analytics that detect and categorize between human and vehicular objects and technology that reduces the number of false alarms.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer active alarm functionality to respond automatically to a triggered alarm.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall include a built-in microphone.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer an Audio Out for an external speaker to be added.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall provide direct network connection using Smart Encoding H.265+, H.265, Smart Encoding H.264+, H.264, MJPEG (Sub Stream Only) compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall support the following dual, redundant power options:

        - 12 VDC

        - PoE (802.3at)

        - LumiPower

        - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall default to use power from the PoE power supply, if connected.

        - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall reboot and switch to the 12 VDC power supply if power from the PoE power supply is lost.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer Wide Dynamic Range (130 dB) for clear images in extreme high-contrast environments.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer three (3) separate and configurable streams with individually configurable 2688 × 1520, D1, and 1080p streams.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall have a motorized vari-focal lens with a lens length of 2.7 to 12 mm.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall combine temperature-tolerant components with a waterproof enclosure to ensure flawless operation in temperatures as low as –30° C (–22° F).

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV housing shall conform to the IP67 and IK10 standard.

    - Imaging

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall use one (1) high performance 1/1.8-in. 4MP Progressive-scan CMOS sensor.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer an effective number of pixels of 2688 × 1520 effective picture elements.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer a motorized vari-focal lens with a lens length of 2.7 to 12 mm

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall have a horizontal angle of 114° to 48°, a vertical angle of 59° to 27°, and a diagonal angle of 143° to 56°.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer a maximum aperture of F1.2.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall produce a color image with a minimum scene illumination of 0.0006 Lux at F1.2.

    - Illumination

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall have four (4) dual-core (IR and white light) LEDs.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer an IR and white-light illumination distance of up to 40.0 m (131.0 ft).

    - Video Characteristics

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer CBR/VBR bit rate control.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer the following video compression protocols:

        - H.264: 32 kbps to 16,384 kbps

        - H.265: 12 kbps to 11,776 kbps

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer Smart Encoding H.265+, H.265, Smart Encoding H.264+, H.264, MJPEG (Sub Stream Only) video compression protocols.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer WDR (130 dB) for backlight compensation.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer 3D NR noise reduction.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer four (4) zones of ROI.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer four (4) privacy masking areas.

    - Streaming Capability

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall generate 2688 × 1520 at 30 fps resolution using Smart Encoding H.265+ compression.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer the following resolutions: 4MP (2688 × 1520), 3.6MP (2560 × 1440), 3MP (2048 × 1536), 3MP (2304 × 1296), 1080p (1920 × 1080), 1.3MP (1280 × 960), 720p (1280 × 720), D1 (704 × 576/704 × 480), VGA (640 × 480), CIF (352 × 288/352 × 240).

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall generate three (3) streams at the following maximum resolutions:

        - Main Stream: 2688 x 1520 at 30 fps

        - Sub Stream 1: D1 at 30 fps

        - Sub Stream 2: 1080p at 30 fps

    - IP Connectivity

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall allow full camera control and configuration capabilities via a TCP/IP network.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall deliver 4MP video at rates up to 30 frames per second via TCP/IP over an RJ-45 (10/100 Base-T) connection.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall conform to the ONVIF S, G, and T profiles.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer Quality of Service (QoS) configuration options.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall support the IPv6 internet-layer protocol for packet switched internetworking across multiple IP networks.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer local and network storage options that include: MicroSD and recording to a local PC for instant recording.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall support the following protocols: IPv4, IPv6, HTTP, TCP, RTSP, RTMP, SNMP, FTP, NAS, DHCP, DDNS, QoS, UPnP, NTP, Multicast, NFS, PPPoE, and SNMP.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall support the LumiViewer and LumiViewer mobile app.

    - Analytics

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV offer the following built-in analytics functions to provide advanced analytics for any scene:

        - Video Content Analytics: Line Crossing, Intrusion, Loitering Detection, Parking Detection

        - Intelligent Motion Detection (iMD): Supported with Alarms for Vehicles and Humans

        - Face Detection: Capture Snapshots and Facial Attributes

        - Heat Map

        - People Counting: Tripwire and In-Area People Counting, Queue Management With Report

        - LumiSearch+: Supported When Used With LumiSearch+-Enabled NVRs

        - LumiAnalytics

        - Object Monitoring

    - Audio

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer the following audio compression: G.711a, G.711Mu, PCM.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall have the following audio input and output channels:

        - Input: Built-in Microphone and 1-channel RCA Input

        - Output: Built-in Speaker and 1-channel RCA Output

    - Alarm

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall have the following alarm input and output channels:

        - Input: One (1) Channel (Max. 5 VDC, 5 mA)

        - Output: One (1) Channel (Max. 12 VDC, 300 mA)

    - Installation Requirements

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall be capable of operating in an outdoor environment within a temperature range of –30° C to +60° C (–22° F to +140° F).

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall accept power, transmit video, and accept control via an Ethernet connection.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall support the following dual, redundant power options:

        - 12 VDC

        - PoE (802.3at)

        - LumiPower

        - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall default to use power from the PoE power supply, if connected.

    - Housing Options

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall be offered in a metal housing.

      - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV housing shall conform to the IP67 and IK10 standard.

  - ACCESSORIES

    - The 4MP Network LumiLuxSmart Dome Camera (Vari-Focal) - N5D-4LAV shall offer the optional accessories that can be found at https://luminyscorp.com/products/accessories.

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

  - INSTALLATION

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Perform installation with qualified service personnel.

    - Install devices in accordance with the National Electrical Code or applicable local codes.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to video system devices for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of camera settings to comply with specific customer’s need.

  - DEMOSTRATION

    - Demonstrate at final inspection that video management system and devices functions properly.

END OF SECTION