March 2025

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2016 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

Section 28 21 00: Video Security

Section 28 21 13: IP Cameras

2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2

- – GENERAL

  - SUMMARY

    - Section Includes

      - Section 28 21 17: Video Security – Security Cameras – Camera Housings

      - Section 28 21 19: Video Security – Security Cameras – Camera Mounts

      - Section 28 21 21: Video Security – Security Cameras – Illuminators

      - Section 28 27 00: Video Security – Video Security Sensors

    - Related Sections

      - [Section 28 33 15: Security Detection, Alarm and Monitoring – Security Monitoring and Control – Security Monitoring and Control Software].

**********Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

    - HD standards

      - Complies with the SMPTE 274M-2008 Standard in:

        - Resolution: 1920x1080

        - Scan: Progressive

        - Color representation: complies with ITU-R BT.709

        - Aspect ratio: 16:9

        - Frame rate: 25 and 30 frames/s

      - Complies with the 296M-2001 Standard in:

        - Resolution: 1280x720

        - Scan: Progressive

        - Color representation: complies with ITU-R BT.709

        - Aspect ratio: 16:9

        - Frame rate: 25, 30, 50 and 60 frames/s

        - Interference-Causing Equipment Standards

  - SYSTEM DESCRIPTION

    - Section Includes

      - Video Security – Security Cameras – IP Cameras

    - Performance Requirements

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall be a full-featured network turret camera designed for discrete video security applications in indoor and outdoor environments.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall use two (2) high performance 1/2.7-in. 4MP Progressive-scan CMOS sensor.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer two (2) IR and two (2) white-light LEDs to produce color images in total darkness.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer advanced analytics that detect and categorize between human and vehicular objects and technology that reduces the number of false alarms.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer active alarm functionality to respond automatically to a triggered alarm.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall include a built-in dual microphone.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall include a built-in speaker.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall provide direct network connection using Smart Encoding H.265+, H.265, Smart Encoding H.264+, H.264, MJPEG (Sub Stream Only) compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall support the following dual, redundant power options:

        - 12 VDC

        - PoE (802.3af)

        - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall default to use power from the PoE power supply, if connected.

        - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall reboot and switch to the 12 VDC power supply if power from the PoE power supply is lost.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer Wide Dynamic Range (120 dB) for clear images in extreme high-contrast environments.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer three (3) separate and configurable streams with individually configurable 4096 × 1860, 1200 × 530, and 1920 × 1080 streams.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall have a fixed lens with a lens length of 2.8 mm.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall combine temperature-tolerant components with a waterproof enclosure to ensure flawless operation in temperatures as low as –40° C (–40° F).

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 housing shall conform to the IP67 standard.

  - SUBMITTALS

Submit under provisions of Section [01 33 00.]

    - Product Data:

      - Manufacturer’s data, user and installation manuals for all equipment and software programs including computer equipment and other equipment required for complete video management system.

    - Dimensional Drawings; include

      - Overall device dimensions.

      - Dimensions specific for installation.

    - Closeout Submittals

      - User manual.

      - Parts list.

      - Maintenance requirements.

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [10] years of experience in manufacture and design Video Security Devices.

    - Video Security System:

      - List certifying bodies (UL, CSA, etc.)

      - Provide evidence of compliance upon request.

    - Installer:

      - Minimum of [5] years of experience installing Video Security System.

  - DELIVERY, STORAGE AND HANDLING

    - Comply with requirements of Section 01 60 00.

    - Deliver materials in manufacture’s original, unopened, undamaged containers; and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users.

    - Provide factory direct technical support via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - [Acceptable Manufacturer:

Luminys Systems Corporation

15245 Alton Pkwy, #100, Irvine, CA, 92618

Tel: (949) 679-7777

Fax: (949) 679-5760

Email: sales@luminyscorp.com]

    - Substitutions: [Not permitted.] [Under provisions of Division 1.]

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

  - PRODUCT SPECIFICATIONS

    - General Characteristics:

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall be a full-featured network bullet camera designed for discrete video security applications in indoor and outdoor environments.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall use two (2) high performance 1/2.7-in. 4MP Progressive-scan CMOS sensor.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer two (2) IR and two (2) white-light LEDs to produce color images in total darkness.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer advanced analytics that detect and categorize between human and vehicular objects and technology that reduces the number of false alarms.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer active alarm functionality to respond automatically to a triggered alarm.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall include a built-in microphone.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall include a built-in speaker.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall provide direct network connection using Smart Encoding H.265+, H.265, Smart Encoding H.264+, H.264, MJPEG (Sub Stream Only) compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall support the following dual, redundant power options:

        - 12 VDC

        - PoE (802.3af)

        - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall default to use power from the PoE power supply, if connected.

        - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall reboot and switch to the 12 VDC power supply if power from the PoE power supply is lost.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer Wide Dynamic Range (120 dB) for clear images in extreme high-contrast environments.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer three (3) separate and configurable streams with individually  4096 × 1860, 1200 × 530, and 1920 × 1080 streams.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall have a fixed lens with a lens length of 2.8 mm.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall combine temperature-tolerant components with a waterproof enclosure to ensure flawless operation in temperatures as low as –40° C (–40° F).

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 housing shall conform to the IP67 standard.

    - Imaging

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall use two (2) high performance 1/2.7-in. 4MP Progressive-scan CMOS sensor.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer an effective number of pixels of 3840 × 2160 effective picture elements.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer a fixed lens with a lens length of 2.8 mm

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall have a horizontal angle of 180° and a vertical angle of 86°.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer a maximum aperture of F1.4.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall produce a color image with a minimum scene illumination of 0.002 Lux at F1.4.

    - Illumination

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall have two (2) IR and two (2) white-light LEDs.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer an IR illumination distance of up to 25.0 m (82.0 ft) and white-light illumination distance of up to 20.0 m (66.0 ft).

    - Video Characteristics

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer CBR/VBR bit rate control.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer the following video compression protocols:

        - H.265: 6 to 16384 kbps

        - H.264: 6 to 16384 kbps

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer Smart Encoding H.265+, H.265, Smart Encoding H.264+, H.264, MJPEG (Sub Stream Only) video compression protocols.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer WDR (120 dB) for backlight compensation.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer 3D NR noise reduction.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer four (4) zones of ROI.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer four (4) privacy masking areas.

    - Streaming Capability

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall generate 2688 × 1520 at 30 fps resolution using Smart Encoding H.265+ compression.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer the following resolutions: 8MP (4096 × 1860), 6MP (3632 x 1632), 4MP (3040 x 1368), 1.6MP (1920 x 860), 1200 x 530, 960 x 432.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall generate three (3) streams at the following maximum resolutions:

        - Main Stream: 4096 × 1860 at 20 fps

        - Sub Stream 1: 1200 × 530 at 20 fps

        - Sub Stream 2: 1920 × 1080 at 20 fps

    - IP Connectivity

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall allow full camera control and configuration capabilities via a TCP/IP network.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall deliver 4096 × 1860 video at rates up to 20 frames per second via TCP/IP over an RJ-45 (10/100 Base-T) connection.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall conform to the ONVIF S, G, and T profiles.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer Quality of Service (QoS) configuration options.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall support the IPv6 internet-layer protocol for packet switched internetworking across multiple IP networks.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer local and network storage options that include: FTP, SFTP, NAS, MicroSD card, and recording to a local PC for instant recording.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall support the following protocols: IPv4, IPv6, HTTP, HTTPS, TCP, UDP, ARP, RTP , RTSP, RTCP, RTMP, SMTP, FTP, SFTP, DHCP, DNS, DDNS, QoS, UPnP; NTP, Multicast, ICMP, IGMP, NFS, PPPoE, 802.1x.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall support the LumiCenter VMS, LumiViewer, and LumiViewer mobile app.

    - Analytics

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 offer the following built-in analytics functions to provide advanced analytics for any scene:

        - Video Content Analytics+: Tripwire, Intrusion, Loitering Detection

        - Intelligent Motion Detection (iMD): Supported with Alarms for Vehicles and Humans

        - LumiTracking-E: Zoom and Track Multiple Humans/Vehicles and Trigger Alarms Automatically

    - Audio

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer the following audio compression: PCM, G.711a, G.711Mu.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall have the following audio input and output channels:

        - Input: Built-in Dual Microphones and One (1) RCA Port

        - Output: Built-in Speaker and One (1) RCA Port

    - Installation Requirements

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall be capable of operating in an outdoor environment within a temperature range of –40° C to +60° C (–40° F to +140° F).

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall accept power, transmit video, and accept control via an Ethernet connection.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall support the following dual, redundant power options:

        - 12 VDC

        - PoE (802.3af)

        - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall default to use power from the PoE power supply, if connected.

        - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall reboot and switch to the 12 VDC power supply if power from the PoE power supply is lost.

    - Housing Options

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall be offered in a metal housing.

      - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 housing shall conform to the IP67 standard.

  - ACCESSORIES

    - The 2 x 4MP Network LumiDeterrent Stitched Turret Camera (2.8 mm) - N5P-8DA2 shall offer the optional accessories that can be found at https://luminyscorp.com/products/accessories.

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

  - INSTALLATION

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Perform installation with qualified service personnel.

    - Install devices in accordance with the National Electrical Code or applicable local codes.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to video system devices for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of camera settings to comply with specific customer’s need.

  - DEMOSTRATION

    - Demonstrate at final inspection that video management system and devices functions properly.

END OF SECTION