March 2025

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2016 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

Section 28 21 00: Video Security

Section 28 21 13: IP Cameras

8MP Network Fisheye 360° Camera - N9F-8RA1

- – GENERAL

  - SUMMARY

    - Section Includes

      - Section 28 21 17: Video Security – Security Cameras – Camera Housings

      - Section 28 21 19: Video Security – Security Cameras – Camera Mounts

      - Section 28 21 21: Video Security – Security Cameras – Illuminators

      - Section 28 27 00: Video Security – Video Security Sensors

    - Related Sections

      - [Section 28 33 15: Security Detection, Alarm and Monitoring – Security Monitoring and Control – Security Monitoring and Control Software].

**********Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

    - HD standards

      - Complies with the SMPTE 274M-2008 Standard in:

        - Resolution: 1920x1080

        - Scan: Progressive

        - Color representation: complies with ITU-R BT.709

        - Aspect ratio: 16:9

        - Frame rate: 25 and 30 frames/s

      - Complies with the 296M-2001 Standard in:

        - Resolution: 1280x720

        - Scan: Progressive

        - Color representation: complies with ITU-R BT.709

        - Aspect ratio: 16:9

        - Frame rate: 25, 30, 50 and 60 frames/s

        - Interference-Causing Equipment Standards

  - SYSTEM DESCRIPTION

    - Section Includes

      - Video Security – Security Cameras – IP Cameras

    - Performance Requirements

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall be a full-featured network fisheye camera designed for discrete video security applications in indoor and outdoor environments.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall use one (1) high performance 1/1.7-in. 8MP Progressive-scan CMOS sensor.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer three (3) IR LEDs to produce color images in total darkness.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer advanced analytics that detect and categorize between human and vehicular objects and technology that reduces the number of false alarms.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer active alarm functionality to respond automatically to a triggered alarm.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall include a built-in dual microphone.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer a built-in speaker.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall provide direct network connection using Smart Encoding H.265+, H.265, Smart Encoding H.264+, H.264, MJPEG (Sub Stream Only) compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall support the following dual, redundant power options:

        - 12 VDC

        - PoE+ (< 13.9 W)

        - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall default to use power from the PoE power supply, if connected.

        - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall reboot and switch to the 12 VDC power supply if power from the PoE power supply is lost.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer Digital Wide Dynamic Range (DWDR) for clear images in extreme high-contrast environments.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer three (3) separate and configurable streams with individually configurable 3280 × 2480, 704 x 480, and 1920 x 1080 streams.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall have a fixed lens with a lens length of 1.85 mm.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall combine temperature-tolerant components with a waterproof enclosure to ensure flawless operation in temperatures as low as –40° C (–40° F).

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 housing shall conform to the IP67 and IK10 standards.

  - SUBMITTALS

Submit under provisions of Section [01 33 00.]

    - Product Data:

      - Manufacturer’s data, user and installation manuals for all equipment and software programs including computer equipment and other equipment required for complete video management system.

    - Dimensional Drawings; include

      - Overall device dimensions.

      - Dimensions specific for installation.

    - Closeout Submittals

      - User manual.

      - Parts list.

      - Maintenance requirements.

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [10] years of experience in manufacture and design Video Security Devices.

    - Video Security System:

      - List certifying bodies (UL, CSA, etc.)

      - Provide evidence of compliance upon request.

    - Installer:

      - Minimum of [5] years of experience installing Video Security System.

  - DELIVERY, STORAGE AND HANDLING

    - Comply with requirements of Section 01 60 00.

    - Deliver materials in manufacture’s original, unopened, undamaged containers; and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users.

    - Provide factory direct technical support via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - [Acceptable Manufacturer:

Luminys Systems Corporation

15245 Alton Pkwy, #100, Irvine, CA, 92618

Tel: (949) 679-7777

Fax: (949) 679-5760

Email: sales@luminyscorp.com]

    - Substitutions: [Not permitted.] [Under provisions of Division 1.]

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

  - PRODUCT SPECIFICATIONS

    - General Characteristics:

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall be a full-featured network bullet camera designed for discrete video security applications in indoor and outdoor environments.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall use one (1) high performance 1/1.7-in. 8MP Progressive-scan CMOS sensor.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer three (3) IR LEDs to produce color images in total darkness.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer advanced analytics that detect and categorize between human and vehicular objects and technology that reduces the number of false alarms.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer active alarm functionality to respond automatically to a triggered alarm.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall include a built-in dual microphone.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer a built-in speaker.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall provide direct network connection using Smart Encoding H.265+, H.265, Smart Encoding H.264+, H.264, MJPEG (Sub Stream Only) compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall support the following dual, redundant power options:

        - 12 VDC

        - PoE+ (< 13.9 W)

        - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall default to use power from the PoE power supply, if connected.

        - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall reboot and switch to the 12 VDC power supply if power from the PoE power supply is lost.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer Digital Wide Dynamic Range (DWDR) for clear images in extreme high-contrast environments.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer three (3) separate and configurable streams with individually configurable 3280 × 2480, 704 x 480, and 1920 x 1080 streams.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall have a fixed lens with a lens length of 1.85 mm.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall combine temperature-tolerant components with a waterproof enclosure to ensure flawless operation in temperatures as low as –40° C (–40° F).

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 housing shall conform to the IP67 and IK10 standard.

    - Imaging

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall use one (1) high performance 1/1.8-in. 12MP Progressive-scan CMOS sensor.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer an effective number of pixels of 3280 × 2480 effective picture elements.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer a fixed lens with a lens length of 1.85 mm

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall have a horizontal angle of 185° and a vertical angle of 180°.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer a maximum aperture of F2.0.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall produce a color image with a minimum scene illumination of 0.01 Lux at F2.0.

    - Illumination

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall have three (3) IR LEDs.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer an IR and white-light illumination distance of up to 10.0 m (32.8 ft).

    - Video Characteristics

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer CBR/VBR bit rate control.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer the following video compression protocols:

        - H.265: 3 kbps to 12288 kbps

        - H.264: 8 kbps to 16384 kbps

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer Smart Encoding H.265+, H.265, Smart Encoding H.264+, H.264, MJPEG (Sub Stream Only) video compression protocols.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer Digital Wide Dynamic Range (DWDR) for backlight compensation.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer 3D NR noise reduction.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer four (4) zones of ROI.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer four (4) privacy masking areas.

    - Streaming Capability

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall generate 4512 × 2512 at 1–15 fps resolution using Smart Encoding H.265+ compression.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer the following resolutions: 3280 × 2480 (8MP), 2880 × 2160 (6MP), 2048 x 1536 (3MP), 704 x 480 (D1), 352 x 240 (CIF).

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall generate three (3) streams at the following maximum resolutions:

        - Mainstream: 3280 × 2480 at 30 fps

        - Substream 1: 704 x 480 at 30 fps

        - Substream 2: 1920 x 1080 at 30 fps

        - Fisheye Mode: 1O (3280 × 2480), 1P (4096 × 1024), 2P (2432 × 2432), 1R (1216 × 1216), 2R (2432 × 2432), 4R (2432 × 2432), 1O+3R (2432 × 2432)

    - IP Connectivity

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall allow full camera control and configuration capabilities via a TCP/IP network.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall deliver 4000 x 3000 video at rates up to 25 frames per second via TCP/IP over an RJ-45 (10/100 Base-T) connection.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall conform to the ONVIF S, G, and T profiles.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer Quality of Service (QoS) configuration options.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall support the IPv6 internet-layer protocol for packet switched internetworking across multiple IP networks.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer local and network storage options that include: FTP, SFTP, NAS, MicroSD Card (Max. 256 GB), and recording to a local PC for instant recording.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall support the following protocols: IPv4, IPv6, TCP, UDP, RTP, RTCP, RTSP, RTMP, NTP, HTTP, ARP, SMTP, FTP, SFTP. DHCP, DNS, DDNS, QoS, UPnP, Multicast, ICMP, IGMP, NFS, PPPoE, SNMP, P2P.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall support the LumiCenter VMS, LumiViewer, and LumiViewer mobile app.

    - Analytics

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 offer the following built-in analytics functions to provide advanced analytics for any scene:

        - Video Content Analytics: Line Crossing, Intrusion, Loitering Detection, Parking Detection

        - Heat Map

        - People Counting: Tripwire and In-Area People Counting, Queue Management With Report

    - Audio

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer the following audio compression: G.711a, G.711Mu, PCM.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall have the following audio input and output channels:

        - Input: Built-In Dual Microphone and 1-channel RCA Port

        - Output: Built-In Speaker and 1-Channel RCA Port

    - Alarm

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall have the following alarm input and output channels:

        - Input: Two (2) Channels 5mA 3V-5VDC

        - Output: Two (2) Channels 1,000mA 30VDC/500mA 50VDC

    - Installation Requirements

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall be capable of operating in an outdoor environment within a temperature range of –40° C to +60° C (–40° F to +140° F).

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall accept power, transmit video, and accept control via an Ethernet connection.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall support the following dual, redundant power options:

        - 12 VDC

        - PoE+ (< 13.9 W)

        - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall default to use power from the PoE power supply, if connected.

        - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall reboot and switch to the 12 VDC power supply if power from the PoE power supply is lost.

    - Housing Options

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall be offered in a metal housing.

      - The 8MP Network Fisheye 360° Camera - N9F-8RA1 housing shall conform to the IP67 and IK10 standards.

  - ACCESSORIES

    - The 8MP Network Fisheye 360° Camera - N9F-8RA1 shall offer the optional accessories that can be found at https://luminyscorp.com/products/accessories.

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

  - INSTALLATION

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Perform installation with qualified service personnel.

    - Install devices in accordance with the National Electrical Code or applicable local codes.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to video system devices for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of camera settings to comply with specific customer’s need.

  - DEMOSTRATION

    - Demonstrate at final inspection that video management system and devices functions properly.

END OF SECTION