July 2025

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2016 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

Section 28 21 00: Video Security

Section 28 21 13: IP Cameras

4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV

- – GENERAL

  - SUMMARY

    - Section Includes

      - Section 28 21 17: Video Security – Security Cameras – Camera Housings

      - Section 28 21 19: Video Security – Security Cameras – Camera Mounts

      - Section 28 21 21: Video Security – Security Cameras – Illuminators

      - Section 28 27 00: Video Security – Video Security Sensors

    - Related Sections

      - [Section 28 33 15: Security Detection, Alarm and Monitoring – Security Monitoring and Control – Security Monitoring and Control Software].

**********Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

    - HD standards

      - Complies with the SMPTE 274M-2008 Standard in:

        - Resolution: 1920x1080

        - Scan: Progressive

        - Color representation: complies with ITU-R BT.709

        - Aspect ratio: 16:9

        - Frame rate: 25 and 30 frames/s

      - Complies with the 296M-2001 Standard in:

        - Resolution: 1280x720

        - Scan: Progressive

        - Color representation: complies with ITU-R BT.709

        - Aspect ratio: 16:9

        - Frame rate: 25, 30, 50 and 60 frames/s

        - Interference-Causing Equipment Standards

  - SYSTEM DESCRIPTION

    - Section Includes

      - Video Security – Security Cameras – IP Cameras

    - Performance Requirements

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall be a full-featured network camera designed for discrete video security applications in indoor and outdoor environments.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall use four (4) high performance 1/2.7-in. 5MP Progressive-scan CMOS sensor.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer eight (8) IR LEDs to produce color images in total darkness.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer advanced analytics that detect and categorize between human and vehicular objects and technology that reduces the number of false alarms.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer active alarm functionality to respond automatically to a triggered alarm.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall provide direct network connection using Smart Encoding H.265+, H.265, Smart Encoding H.264+, H.264, MJPEG (Sub Stream Only) compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall support the following dual, redundant power options:

        - 12 VDC

        - PoE+ (802.3at)

        - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall default to use power from the PoE power supply, if connected.

        - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall reboot and switch to the 12 VDC power supply if power from the PoE power supply is lost.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer Digital Wide Dynamic Range (DWDR) for clear images in extreme high-contrast environments.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer three (3) separate and configurable streams with individually configurable 5MP, D1, and 1080p streams for single streaming and 2MP, D1, and 8MP for consolidated streaming.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall have motorized vari-focal lenses with a lens length of 2.8 mm to 12.0 mm.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall combine temperature-tolerant components with a waterproof enclosure to ensure flawless operation in temperatures as low as –30° C (–22° F).

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV housing shall conform to the IP67 and IK10 standard.

  - SUBMITTALS

Submit under provisions of Section [01 33 00.]

    - Product Data:

      - Manufacturer’s data, user and installation manuals for all equipment and software programs including computer equipment and other equipment required for complete video management system.

    - Dimensional Drawings; include

      - Overall device dimensions.

      - Dimensions specific for installation.

    - Closeout Submittals

      - User manual.

      - Parts list.

      - Maintenance requirements.

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [10] years of experience in manufacture and design Video Security Devices.

    - Video Security System:

      - List certifying bodies (UL, CSA, etc.)

      - Provide evidence of compliance upon request.

    - Installer:

      - Minimum of [5] years of experience installing Video Security System.

  - DELIVERY, STORAGE AND HANDLING

    - Comply with requirements of Section 01 60 00.

    - Deliver materials in manufacture’s original, unopened, undamaged containers; and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users.

    - Provide factory direct technical support via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - [Acceptable Manufacturer:

Luminys Systems Corporation

15245 Alton Pkwy, #100, Irvine, CA, 92618

Tel: (949) 679-7777

Fax: (949) 679-5760

Email: sales@luminyscorp.com]

    - Substitutions: [Not permitted.] [Under provisions of Division 1.]

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

  - PRODUCT SPECIFICATIONS

    - General Characteristics:

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall be a full-featured network camera designed for discrete video security applications in indoor and outdoor environments.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall use four (4) high performance 1/2.7-in. 5MP Progressive-scan CMOS sensosr.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer eight (8) IR LEDs to produce color images in total darkness.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer advanced analytics that detect and categorize between human and vehicular objects and technology that reduces the number of false alarms.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer active alarm functionality to respond automatically to a triggered alarm.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall provide direct network connection using Smart Encoding H.265+, H.265, Smart Encoding H.264+, H.264, MJPEG (Sub Stream Only) compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall support the following dual, redundant power options:

        - 12 VDC

        - PoE+ (802.3at)

        - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall default to use power from the PoE power supply, if connected.

        - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall reboot and switch to the 12 VDC power supply if power from the PoE power supply is lost.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer Digital Wide Dynamic Range (DWDR) for clear images in extreme high-contrast environments.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer three (3) separate and configurable streams with individually configurable 5MP, D1, and 1080p streams for single streaming and 2MP, D1, and 8MP for consolidated streaming.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall have motorized vari-focal lenses with a lens length of 2.8 mm to 12.0 mm.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall combine temperature-tolerant components with a waterproof enclosure to ensure flawless operation in temperatures as low as –30° C (–22° F).

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV housing shall conform to the IP67 and IK10 standard.

    - Imaging

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall use four (4) high performance 1/2.7-in. 5MP Progressive-scan CMOS sensors.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer an effective number of pixels of 2592 × 1944 effective picture elements per sensor.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer motorized vari-focal lenses with a lens length of 2.8 mm to 12.0 mm.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall have a horizontal angle of 100° to 42°, a diagonal angle of 133° to 53°, and a vertical angle of 74° to 32°.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer a maximum aperture of F1.7.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall produce a color image with a minimum scene illumination of 0.005 Lux at F1.7.

    - Illumination

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall have eight (8) IR LEDs.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer an IR illumination distance of up to 30.0 m (98.4 ft).

    - Video Characteristics

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer CBR/VBR bit rate control.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer the following video compression protocols:

        - H.265: 3 kbps to 19,712 kbps

        - H.264: 3 kbps to 19,712 kbps

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer Smart Encoding H.265+, H.265, Smart Encoding H.264+, H.264, MJPEG (Sub Stream Only) video compression protocols.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer Digital Wide Dynamic Range (DWDR) for backlight compensation.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer 3D NR noise reduction.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer four (4) zones of ROI.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer eight (8) privacy masking areas.

    - Streaming Capability

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall generate 5MP at 30 fps (single stream) or 8MP at 30 fps (consolidated stream) resolution using Smart Encoding H.265+ compression.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer the following resolutions:

        - Single Stream: 5MP (2592 × 1944), 4MPP (2560 × 1440), 3MP (2304 × 1296/2048 × 1536), 1080p (1920 × 1080), 1.3MP (1280 × 960), 720p (1280 × 720), D1 (704 × 480), VGA (640 × 480), CIF (352 × 240)

        - Consolidated Stream: 8MP (3296 × 2472), 5MP (2592 × 1944), 1080p (1920 × 1080)

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall generate three (3) streams at the following maximum resolutions:

        - Single Stream

          - Main Stream: 5MP at 30 fps

          - Sub Stream 1: D1 at 30 fps

          - Sub Stream 2: 1080p 30 fps

        - Consolidated Stream

          - Main Stream: 2MP at 30 fps

          - Sub Stream 1: D1 at 30 fps

          - Sub Stream 2: 8MP at 30 fps

    - IP Connectivity

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall allow full camera control and configuration capabilities via a TCP/IP network.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall deliver 4096 × 1860 video at rates up to 20 frames per second via TCP/IP over an RJ-45 (10/100 Base-T) connection.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall conform to the ONVIF S, G, and T profiles.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer Quality of Service (QoS) configuration options.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall support the IPv6 internet-layer protocol for packet switched internetworking across multiple IP networks.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer local and network storage options that include: FTP, SFTP, NAS, MicroSD card, and recording to a local PC for instant recording.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall support the following protocols: IPv4, IPv6, HTTP, HTTPS, TCP, UDP, ARP, RTP , RTSP, RTCP, RTMP, SMTP, FTP, SFTP, DHCP, DNS, DDNS, QoS, UPnP; NTP, Multicast, ICMP, IGMP, NFS, PPPoE, 802.1x.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall support the LumiCenter VMS, LumiViewer, and LumiViewer mobile app.

    - Analytics

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV offer the following built-in analytics functions to provide advanced analytics for any scene:

        - Video Content Analytics+: Tripwire, Intrusion, Loitering Detection

        - Intelligent Motion Detection (iMD): Supported with Alarms for Vehicles and Humans

        - LumiAnalytics: Face Detection, People Counting, Smart Object Detection

          - Heat map and queue management are not supported.

        - People Counting: Tripwire and In-Area People Counting, Queue Management With Report

    - Audio

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer the following audio compression: PCM, G.711, G.726.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall have the following audio input and output channels:

        - Input: One (1) RCA Port

        - Output: One (1) RCA Port

    - Alarm

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall support the following alarm events: No SD Card, Disk Full, SD Card Error, Offline, IP Conflict, Illegal Access, Motion Detection, Video Tampering, Voltage Detection, Security Exception, Line Crossing, Intrusion, Abandoned Object, Missing Object, Loitering Detection, People Gathering, Parking Detection, Scene Changing, Audio Detection, External Alarm, Face Detection, People Counting In Area, Abnormal Number of People Detection, People Counting, People Stay Detection.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall have the following alarm input and output channels:

        - Input: Three (3) Channels (Max. 5 VDC, 5 mA)

        - Output: Two (2) Channels (Max. 12 VDC, 300 mA)

    - Installation Requirements

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall be capable of operating in an outdoor environment within a temperature range of -30 °C to 60 °C (-22°F to 140°F).

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall accept power, transmit video, and accept control via an Ethernet connection.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall support the following dual, redundant power options:

        - 12 VDC

        - PoE+ (802.3at)

        - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall default to use power from the PoE power supply, if connected.

        - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall reboot and switch to the 12 VDC power supply if power from the PoE power supply is lost.

    - Housing Options

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall be offered in a metal housing.

      - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV housing shall conform to the IP67 and IK10 standards.

  - ACCESSORIES

    - The 4 x 5MP LumiFlex Multi-Directional Network Camera - N9P-20RAV shall offer the optional accessories that can be found at https://luminyscorp.com/products/accessories.

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

  - INSTALLATION

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Perform installation with qualified service personnel.

    - Install devices in accordance with the National Electrical Code or applicable local codes.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to video system devices for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of camera settings to comply with specific customer’s need.

  - DEMOSTRATION

    - Demonstrate at final inspection that video management system and devices functions properly.

END OF SECTION