Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2016 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

Section 28 21 00: Video Surveillance

Section 28 21 13: IP Cameras

All-in-One Enforcement Camera

- – GENERAL

  - SUMMARY

    - Section Includes

      - Section 28 21 17: Video Surveillance – Surveillance Cameras – Camera Housings

      - Section 28 21 19: Video Surveillance – Surveillance Cameras – Camera Mounts

      - Section 28 27 00: Video Surveillance – Video Surveillance Sensors

    - Related Sections

      - [Section 28 33 15: Security Detection, Alarm and Monitoring – Security Monitoring and Control – Security Monitoring and Control Software].

********** Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

| STANDARD | CE: E234884-A6183-IT-1/4789603556-1 FCC: 4789603556-2 UL: E234884-A6183-X3 |
| --- | --- |

  - SYSTEM DESCRIPTION

    - Section Includes

      - Video Surveillance – Surveillance Cameras –All-in-One Enforcement Camera

  - SUBMITTALS

    - Product Data:

      - Manufacturer’s data, user and installation manuals for all equipment and software programs including computer equipment and other equipment required for complete video management system.

    - Dimensional Drawings; include

      - Overall device dimensions.

      - Dimensions specific for installation.

    - Closeout Submittals

      - User manual.

      - Parts list.

      - Maintenance requirements.

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [10] years of experience in manufacture and design Video Surveillance Devices.

    - Video Surveillance System:

      - List certifying bodies (UL, etc.)

      - Provide evidence of compliance upon request.

    - Installer:

      - Minimum of [5] years of experience installing Video Surveillance System.

  - DELIVERY, STORAGE AND HANDLING

    - Comply with requirements of Section 01 60 00.

    - Deliver materials in manufacture’s original, unopened, undamaged containers; and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

  - WARRANTY

    - Provide manufacturer’s warranty covering [3] years for replacement and repair of defective equipment. Warranty varies country to country.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users.

    - Provide factory direct technical support via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:

Zhejiang Dahua Vision Technology Co., Ltd.

Address: No.1199 Bin’an Road, Binjiang District, Hangzhou, China

Tel: +86-571-87688883

Fax: +86-571-87688815

Email:overseas@dahuasecurity.com

    - Substitutions:

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

  - All-in-One Enforcement Camera | DHI-ITC431-RW1F-IRL8-C2

| Camera | Image Sensor | 1/1.8" CMOS |
| --- | --- | --- |
| Camera | Shutter Mode | Single shutter |
| Camera | Electronic Shutter Speed | Auto/Manual 1/50s～1/10,000s |
| Camera | Noise Reduction | 3D NR |
| Camera | S/N Ratio | > 56 dB |
| Camera | WDR | 90 dB |
| Camera | Min. Illumination | 0.0094 lux |
| Camera | Day/Night | Supports ICR auto switch: IR cut-off filter (IRCF) with the polarizing filter is used during the daytime, and switches to the common IR cut-off filter at night |
| Illuminator | Illuminator Number | 4 illuminators (830 nm LED illuminators, adjustable brightness) |
| Illuminator | IR Light | Yes |
| Illuminator | Illumination Distance | 23 m–30 m (75.46 ft–98.43 ft) (adjustable brightness) |
| Illuminator | Lane Coverage | 1–2 lanes |
| Lens | Focal Length | 10 mm–50 mm |
| Lens | Max. Aperture | Max. F1.33 |
| Lens | Field of View | Horizontal: 9.4°–40.8° Vertical: 5.4°–22.9° Diagonal: 10.7°–46.9° |
| Lens | Exposure Mode | Auto; Manual (Select from shutter values or customize shutter range) |
| Function | Trigger Mode | Video detection/radar |
| Function | OSD Overlay | Time, address, device SN, lane No., plate number, plate color, vehicle logo, vehicle type, vehicle color, vehicle size, vehicle speed, country/region, event (violation name), and motorcycle attributes (type, number of people, and helmet) |
| Function | Alarm Event | Storage full, storage error, external alarm, no storage space, license plate blocklist, illegal access, and security fault |
| Function | Automatic Network Replenishment (ANR) | Platform and FTP (TF card is required) |
| Function | Auto Registration | Yes |
| Intelligence | Target Detection | Motor vehicle; motorcycle |
| Intelligence | ANPR | Adopts Dahua developed algorithms to recognize license plate numbers and letters |
| Intelligence | Vehicle Type Recognition | Vehicle head: Large bus, heavy truck, medium truck, sedan, van, light truck, medium bus, SUV, MPV, and pickup Vehicle tail: SUV, large bus, sedan, light truck, pickup, medium truck, van, and heavy truck |
| Intelligence | Vehicle Color Recognition | White, pink, black, red, yellow, gray, blue, green, dark orange, purple, brown, and silver gray (color recognition is not supported during nighttime) |
| Intelligence | Vehicle Brand/Logo Recognition | Acura, Alfaromeo, Ashokleyland, Astonmartin, Audi, Baic, Bently, Benz, BMW, Buick, BYD, Cadillac, Chery, Chevrolet, Chrysler, Citroen, Dacia, Daihatsu, Datsun, Dodge, DS, Ferrari, Fiat, Force, Ford, Foton, Geely, GMC, Greatwall, Hino, Honda, Hyundai, Infiniti, Isuzu, Iveco, Jac, Jaguar, Jeep, Kia, Kinglong, Land, Lexus, Lifan, Lincoln, Mahindra, MAN, Maserati, Mazda, Mercury, MG, Mini, Mitsubishi, Nissan, Opel, Peugeot, Porsche, Renault, Rollsroyce, Saab, Scania, Seat, Skoda, Smart, Subaru, Suzuki, Tata, Tesla, Toyota, UD, Volkswagen, Volvo |
| Intelligence | Motor Vehicle Violation Capture | Speeding; driving slow; wrong-way driving; illegal lane change |
| Intelligence | Motorcycle Violation Capture | Overload; not wearing helmet |
| Intelligence | Traffic Flow Detection | Statistics on vehicle flow, average speed, vehicle type, lane occupancy, average time headway, average queue length, and road status |
| Intelligence | Traffic Event | Motor vehicle illegal stopping on the roadway; traffic congestion |
| Video | Video Compression | H.265;H.264M;H.264H;MJPEG |
| Video | Video Resolution | 4M (2688 × 1520); 1080p (1920 × 1080); UXGA (1600 × 1200); 720p (1280 × 720); D1 (704 × 576); CIF (352 × 288) |
| Video | Video Frame Rate | Max. 25 fps; main stream (2688 × 1520@25 fps), sub stream (1600 × 1200@25 fps) |
| Video | Video Bit Rate | H.264: 32 kbps–32767 kbps H.265: 32 kbps–32767 kbps MJPEG: 512 kbps–32767 kbps |
| Video | Bit Rate Control | CBR/VBR |
| Video | White Balance | Auto;Manual;Outdoor;Natural;Street Lamp;Area white balance |
| Video | Edge Enhancement | Yes |
| Video | HLC | Yes |
| Video | BLC | Yes |
| Video | Bad Pixel Correction | Yes |
| Video | Gain Range | 0-100 |
| Image | Composite Image | Supports combining up to 3 source images, and 1 close-up image into a composite image. |
| Image | Image Resolution | 2688 (H) × 1520 (V) (OSD black background is not calculated in the pixels) |
| Image | Image Encoding Format | JPEG |
| Image | Image Tampering Prevention | Verifies watermarks in videos and images |
| Network | Network Port | 1 × RJ-45 Ethernet port, 10/100/1000 M network transmission |
| Network | SDK and API | Yes |
| Network | Security | Authorized username and password, MAC address binding, HTTPS encryption, and network access control |
| Network | Browser | IE: IE 9–11 Chrome: Chrome 41 and earlier Firefox: Firefox 49 and earlier For Win 10 users, run the browser as administrator |
| Network | Positioning | GPS |
| Network | Network Status Monitoring | NTP; GPS |
| Port | Storage | 1, supports maximum 256 GB TF card local storage |
| Port | RS-485 | 2, connects to devices such as radars |
| Port | RS-232 | 2, con is used for serial debugging and R T G is for connecting to radars |
| Port | Audio Input | 1 channel |
| Port | Audio Output | 1 channel |
| Port | Alarm Input | 3 channels |
| Port | Alarm Output | 2 channels. 1 for relay, and 1 for optocoupler |
| General | Power Supply | 12–36 VDC, PoE |
| General | Power Consumption | ≤ 13 W |
| General | Operating Temperature | –40 °C to +65 °C (–40 °F to +149 °F) |
| General | Storage Temperature | –40 °C to +65 °C (–40 °F to +149 °F) |
| General | Operating Humidity | 10%–90% (RH) |
| General | Storage Humidity | 10%–90% (RH) |
| General | Protection | IP67 |
| General | Product Dimensions | 466.4 mm × 135.8 mm × 134.1 mm (18.36'' × 5.35'' × 5.28'') (L × W × H) |
| General | Net Weight | 3.9 kg (8.59 lb) |
| General | Gross Weight | 4.8 kg (10.58 lb) |
| General | Installation | Installs with universal mounting brackets, or as a side-mount with mounting brackets. |

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

  - INSTALLATION

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Perform installation with qualified service personnel.

    - Install devices in accordance with the National Electrical Code or applicable local codes.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to video system devices for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of camera settings to comply with specific customer’s need.

  - DEMOSTRATION

    - Demonstrate at final inspection that video management system and devices functions properly.

END OF SECTION