Hanwha Vision is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: Hanwha Vision (https://www.hanwhavision.com, https://hanwhavisionamerica.com)

    - Model: NHP-P200

    - Alternates: none

  - GENERAL DESCRIPTION

    - The device shall possess the following key specifications:

      - Capable of integrating with ACaaS, 3rd party ACS platforms, and operating in “Standalone Mode”: First “Open Architecture” IP Controller with “Direct to Cloud” capabilities

      - Supports current open standards (ONVIF, SUNAPI, OSDP)

      - Capable of supporting OpenPlatform

      - Power over Ethernet (PoE++) and Networkable

      - Operates in an “all-in-one” package that does not need external accessories (like enclosure, PS, etc.)

      - Highly reliable, cyber-secure, easy to install, and easy to configure

      - Meets regulatory requirements (UL294)

      - Cost effective, designed only for what customers need

      - Aesthetic and advanced mechanical/industrial design

    - Interoperability – The camera shall be ONVIF Profile A / C / D compliant.

  - DETAILED SPECIFICATIONS

    - Hardware

      - Readers: 2 OSDP/Wiegand Reader Port (capable of supporting 4 OSDP Readers or 2 Wiegand Readers)

      - Reader Port Specs

        - OSDP Protocol (v2.2), Legacy Wiegand Support

        - 300mA @ 12vDC

      - General Purpose Inputs: 2 (Door Position Switches, 2 REX Sensors, 1 aux input)+ 1 additional input if OSDP is used

      - Input Specs

        - 2 pin, supervised input port

        - 0.025W (5mA sink, 5V nominal) 0 to +5VCD Ref

      - General Purpose Outputs: 2 (Door Lock)

      - Output Specs

        - Dry/12vDC/24vDC Field selectable, 2A @ 30vDC DRY

        - 500mA @ 12vDC WET, 350mA @ 24vDC WET"

      - Dedicated Inputs

        - Housing tamper (built-in; removal of housing cover, removal from wall)

        - 1 FAI for Power Drop"

      - Aux Port: 12vDC 50mA Port (for PIR Power)

      - Terminal Blocks: Color coded by typical purpose

    - Access

      - Doors: 2 Door

      - Credentials: 50,000

      - Access Levels: 50,000

      - Event History: 250,000 (offline cache event)

    - Operational

      - Panel Title: Displayed up to 85 characters

      - Operation Mode: Server/Cloud Controlled, Online Mode, Offline Mode, Standalone Mode

    - Network

      - Ethernet: RJ-45(10/100BASE-T)

      - Protocol: IPv4, IPv6, TCP/IP, UDP/IP, NTP, HTTP, HTTPS, SSL/TLS, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, PIM-SM, UPnP, Bonjour

      - Network Security

        - HTTPS(SSL) login authentication

        - Digest login authentication

        - IP address filtering

        - User access log

        - 802.1X authentication(EAP-TLS, EAP-LEAP)

        - SNMP v3, TLS 1.2/1.3

        - Device certificate(Hanwha Private Root CA)"

      - Data Security

        - Data encryption at rest

        - SecureOS/Boot/Storage

        - Firmware verification

        - TPM with FIPS 140-2 Level 2

    - System Integration

      - Supported Accessories: Surface Mount Backbox, DIN Rail Mount

      - Application Programming Interface

        - ONVIF Profile A

        - ONVIF Profile C

        - ONVIF Profile D

        - SUNAPI (HTTP API)

        - Wisenet OpenPlatform

    - Environmental & Electrical

      - Operating Temperature / Humidity: 0°C~+50°C / 20-85% RH (non-condensing)

      - Storage Temperature / Humidity: -50°C~+55°C / Less than 95% RH

      - Certification: Safety(UL), EMC(FCC, CE)

      - Input Voltage: PoE+, 12vDC IN

      - Power Consumption: TBD

    - Mechanical

      - Color / Material: White / Aluminum

      - RAL Code: TBD

      - Product dimensions / weight: TBD

      - Conduit hole: None (cable entry through base of panel)

      - Mounting holes: Single and Double Gang, 4"" Octagon, Square 5.5"" x 5"" (127mm x 139.7mm) at 1/4""

      - Diagnostic lights: Host connection, Internet connection, Board health, Reader active, Input active, Output active

    - General

      - Webpage Language: English, Korean, French, Spanish, Portuguese, Polish

      - Included Accessories: Installation guide, terminal blocks

      - Memory: TBD

      - Certifications UL 294, UL 2043, FCC Part 15, CE, California Prop 65

END OF SECTION

8