DC-D3C33HRX

Architectural and Engineering Specifications

Version 1.2

(Aug. 29, 2022)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Section 28 23 29 – Video Surveillance Remote Devices and Sensors

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 463-400, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

General

Product Description

DC-D3C33HRX is a Dome type IP Camera designed and manufactured by IDIS. This camera provides 12MP (4000x3000) resolution at 15ips (images per second) or 8MP (3840x2160) resolution at 30ips with H.265, H.264, and M-JPEG compression. This camera is equipped with Motorized Vari-focal lens, IR LEDs, True Day/Night, PoE (IEEE 802.3af Class 3), Audio I/O, Alarm I/O, microSD/SDHC/SDXC card backup, Vandal-proof dome enclosure design, IK10 / IP67 rated and heater to provides continuous operation in subzero temperature.

General Specification

- The IP camera shall be equipped with 12 Megapixel 1/1.7” CMOS Sensor.

- The IP camera shall be equipped with 4.5mm - 10mm motorized vari-focal lens, F1.6 - F2.6.

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance.
The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have wide dynamic range compensation (True WDR) for improved video quality in high-contrast situations (120 dB).

- The IP camera shall have 6 IR LEDs looming 30m (98.4 ft).

- The IP camera shall support P-Iris.

- The IP camera shall utilize configurable 2DNR/3DNR (Dynamic Noise Reduction) technology to reduce the bitrate and storage requirements by removing noise artifacts.

- The IP camera shall be Vandal-Proof IK10 and IP rating 67 complied dome enclosure design.

- The IP camera shall be equipped with 10/100/1000 Base-T, auto-sensing, half/full duplex, RJ-45 Ethernet connection.

- The IP camera shall support industry standard Power over Ethernet (PoE) IEEE 802.3af, Class 3 to supply power to the camera over the network and 12VDC input.

- The IP camera shall have video out feature (NTSC/PAL).

- The IP camera shall have built-in heater for continued use in subzero temperature conditions and utilize 12 VDC input to provide power.

- The IP camera shall have on board microSD/SDHC/SDXC card backup storage slot as a safeguard against data loss during network interruptions.

- Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.

- The IP camera shall deliver maximum video resolution of 4000x3000 at rates up to 15ips (images per second) or 3840x2160 at rates up to 30ips.

- The IP camera shall provide direct network connection using H.265, H.264 and MJPEG compression.

- The IP camera shall support quadruple streams

- The IP camera shall conform to the ONVIF Profile S Ver.2.4.0 standard.

- The IP camera shall be equipped with embedded web server which works independently using a Web Browser with ActiveX plug-in.

- The IP camera shall SSL Encryption, Multi-user Authority, IEEE 802.1x IP Filtering and HTTPS for greater security.

- The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.

- The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Intelligent Video Analysis (VA): Video Motion Detection, Active Tampering Alarm, Trip Zone.

Protocol Specification: DirectIP 2.0

- The IP camera shall have DirectIP 2.0 mode.

- DirectIP 2.0 protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

- DirectIP 2.0 shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

- DirectIP 2.0 shall support camera can be linked to IDIS software solution such ad IDIS Center and IDIS Solution Suite, or 3rd party solution while it is being connected to a DirectIP NVR.

- DirectIP 2.0 camera shall be compatible with DirectIP 1.0 NVR as well as DirectIP 2.0 NVR.

- DirectIP 2.0 camera shall be unavailable for No-password login when connecting to DirectIP 2.0 NVR and IDIS Software Solutions.

- DirectIP 2.0 protocol shall provide Quadruple streams.

- DirectIP 2.0 protocol shall support H.264 and H.265 and MJPEG compression.

Technical Specification

Video Specification

- Image Sensor: 1/1.7” CMOS

- Maximum Resolution: 4000x3000

- Scanning Mode: Progressive Scan

- Lens Type: Motorized Vari-focal (f=4.5 - 10mm, F1.6 - 2.6)

- Iris Control: P-Iris

- Angular Field of View (H: Horizontal, V: Vertical, D:Diagonal):

[8MP Mode]

  - Wide: 90.2º(H), 48.5º(V), 106.2º(D)

  - Tele: 41.6º(H), 23.3º(V), 47.8º(D)

[12MP Mode]

  - Wide: 94.6º(H), 68.5º(V), 125.2º(D)

  - Tele: 43.4º(H), 32.4º(V), 54º(D)

- Pan/Tilt Range: Pan: -176° ~ 176°, Tilt: 0° ~ 65°, Rotate: -90° ~ 90°

- Minimum Illumination:

  - COLOR : 0.3 lux @ F1.6

  - B/W: 0 lux (IR LED ON)

- S/N Ratio: more than 48 dB

- Maximum Frame Rate:

  - 30fps : 3840x2160 (WDR)

  - 15fps : 4000x3000 (WDR)

- Video Resolution:

  - 12MP mode : 4000x3000, 2592x1944, 1280x960, 640x480, 320x240

  - 8MP mode : 3840 x 2160 , 1920x1080, 1280x720, 640x360, 320x180

- Video Compression : H.265, H.264 and MJPEG

- Video Compression Level: Basic, Standard, High, Very High

- Multi-Video Streaming: Quadruple streams

- Dynamic Range: 120 dB (True WDR)

- True Day & Night: Yes (ICR)

- IR Distance (The number of LEDs, IR wavelength): 30m / 98.4 ft (6ea)

- Intelligent Video Analytic: Video Motion Detection, Active Tampering Alarm, Trip Zone

- Analog Video Output: 1 BNC (Accessory)

Audio Specification

- Audio Compression Algorithm: ADPCM 16K, G.726, G.711 u-Law, G.711 a-Law

- Audio Input / Output: Line-in 1ea / Line-out 1ea

- Two-way Audio Communication: Yes

- Pre-recorded Voice Alert: Yes

Network Specification

- Port: RJ-45 10/100/1000 Base-T 1 port

- Network Protocols: DirectIP 2.0 Protocol, IPv4, IPv6, RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP,RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, uPNP, SNMPv2

- Remote Access Client: DirectIP NVR Connection, IDIS Web, IDIS Mobile, IDIS Solution Suite

- Recording Session Buffer (NLTSrec): Up to 60MB

- Edge Storage: micro SD/SDHC/SDXC, Smart Failover (Up to 256GB)

Security Specification

- Multi-user Authority, IEEE 802.1x, IP Filtering, HTTPS, SSL Encryption

- Maximum User Access: 10 (Live), 1 (Recording), 1 (Search), 2 (Admin)

Alarm and Event Specification

- Alarm Input / Output: 1 / 1

  - Alarm Input: 1 TTL, NC/NO Programmable, 4.3V(NC) or 0.3V(NO) threshold, 5V DC

  - Alarm Output: 1 TTL open collector, 30mA @ 5 VDC

- Trigger Events: Motion Detection, Alarm in, Audio detection, Tampering, Trip Zone

- Event Notification: Remote S/W, Email (with Image)

  - Encryption Type: SSL, TLS

Environmental Specification

- Operating Temperature: -40°C ~ +60°C (-40°F ~ +140°F)

- Operating Humidity: 0% to 90% non-condensing

- Vandal-proof Enclosure: Yes (IK10)

- Outdoor Ready: IP67, Heater

Electrical Specification

- Power Source: 12VDC, PoE(IEEE 802.3af class 3)

- Power Consumption: 12W

- Regulatory Approvals: FCC, CE, KC

Mechanical Specification

- Dimensions (Ø x H): Ø160mm x 128.5mm (Ø6.30" x 5.06")

- Unit Weight: 1.21Kg (2.67lb)

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.0 | Ray Sun | Jun. 05, 2018 | Initial Release |
| 1.1 | Everett Lee | Jun. 15, 2020 | Modifying Power Consumption |
| 1.2 | TS Team | Aug. 29, 2022 | Spec Update |