DC-D4216WRX 2.8mm

Architectural and Engineering Specifications

Version 1.2

(Feb. 25, 2025)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Level 1 - 28.20.00 – Video Surveillance

Level 2 - 28.21.00 – Surveillance Cameras

Level 3 - 28.21.13 – IP Cameras

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 463-400, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

General

Product Description

DC-D4216WRX 2.8mm is Outdoor Dome type IP Camera designed and manufactured by IDIS. This camera provides Full HD (1920 x 1080) resolution at 30ips (images per second) with H.265, H.264, and M-JPEG compression. This camera is equipped with Fixed-focal lens, IR LEDs, True Day/Night, PoE (IEEE 802.3af Class 2), Audio I/O, Alarm I/O, microSD/SDHC/SDXC card backup(up to 512GB), Vandal-proof dome enclosure design, IP67 rated with Heater/ IK10 and provides continuous operation in subzero temperature..

General Specification

- The IP camera shall be equipped with 2 Megapixel 1/2.8” CMOS Sensor.

- The IP camera shall be equipped with 2.8mm fixed-focal lens, F2.0

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance.
The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have wide dynamic range compensation (True WDR) for improved video quality in high-contrast situations (120 dB).

- The IP camera shall have 4 IR LEDs looming 30m (98.4ft).

- The IP camera shall support Fixed-Iris.

- The IP camera shall utilize 2DNR/3DNR (Dynamic Noise Reduction) technology to reduce the bitrate and storage requirements by removing noise artifacts.

- The IP camera shall be Vandal-Proof(IK10) and IP rating 67 complied dome enclosure design

- The IP camera shall be equipped with 10/100 Base-T, auto-sensing, half/full duplex, RJ-45 Ethernet connection.

- The IP camera shall support industry standard Power over Ethernet (PoE) IEEE 802.3af, Class 2

- The IP camera shall have on board micro SD/SDHC/SDXC, Smart Failover (Up to 512GB) backup storage slot as a safeguard against data loss during network interruptions.

- Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.

- The IP camera shall deliver maximum video resolution of 1920 x 1080 at rates up to 30ips (images per second).

- The IP camera shall provide direct network connection using H.265, H.264 and M-JPEG compression.

- The IP camera shall support quadruple streams in DirectIP 2.0 protocol mode.

- The IP camera shall conform to the ONVIF Profile S/T.

- The IP camera shall be equipped with embedded web server (IDIS Web) which works independently using a Web Browser with ActiveX plug-in.

- The IP camera shall have Digest authentication, Password Protection, User access log, IP Filtering, IEEE802.1x(EAP-TLS), Chained Fingerprint, SSL Encryption and HTTPS Encryption for greater security.

- The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.

- The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Intelligent Video Analysis (VA): Motion Detection, Active Tampering Alarm and Trip Zone.

Protocol Specification: DirectIP 2.0

- The IP camera shall have DirectIP 2.0 mode.

- DirectIP 2.0 protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

- DirectIP 2.0 shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

- DirectIP 2.0 shall support camera can be linked to IDIS software solution such ad IDIS Center and IDIS Solution Suite, or 3rd party solution while it is being connected to a DirectIP NVR.

- DirectIP 2.0 camera shall be compatible with DirectIP 1.0 NVR as well as DirectIP 2.0 NVR.

- DirectIP 2.0 camera shall be unavailable for No-password login when connecting to DirectIP 2.0 NVR and IDIS Software Solutions.

- DirectIP 2.0 protocol shall provide Quadruple streams.

- DirectIP 2.0 protocol shall support H.264 and H.265 and M-JPEG compression.

Technical Specification

Video Specification

- Image Sensor: 1/2.8” CMOS

- Maximum Resolution: 1920 x 1080

- Scanning Mode: Progressive Scan

- Lens Type: Fixed-focal (f=2.8, F2.0)

- Iris Control: Fixed Iris

- Angular Field of View (H: Horizontal, V: Vertical, D:Diagonal): 111.6º(H), 61.45º(V), 127.3º(D)

- Pan/Tilt Range: Pan: -170° ~ 170°, Tilt: -5° ~ 67.5°, Rotate: -109° ~ 109°

- Minimum Illumination:

  - COLOR : 0.1 lux @ F2.0

  - B/W: 0 lux (IR LED ON)

- S/N Ratio: more than 45 dB

- Maximum Frame Rate: 30fps : 1920 x 1080(WDR)

- Video Resolution: 1920x1080, 1280x720, 640x360, 352x240

- Video Compression : H.265, H.264, M-JPEG

- Video Compression Level: 4 levels: Basic, Standard, High, Very High

- Multi-Video Streaming: Quadruple streams

- Dynamic Range: 120 dB

- True Day & Night: Yes (ICR)

- IR Distance (The number of LEDs, IR wavelength): 30m / 98.4ft (4ea)

- Intelligent Video Analytic: Video Motion Detection, Active Tampering Alarm and Trip Zone

Audio Specification

- Audio Compression Algorithm: ADPCM 16K, G.726, G.711 u-Law, G.711 a-Law

- Audio Input / Output: 1 / 1

- Two-way Audio Communication: Yes

- Pre-recorded Voice Alert: On/Off

Network Specification

- Port: RJ-45 10/100 Base-T 1 port

- Network Protocols: DirectIP 2.0 Protocol, IPv4, IPv6, RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP,RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, uPNP, SNMPv2

- Streaming Mode: Quadruple streaming

Security Specification

- SSL Encryption, Multi-user Authority, IEEE 802.1x IP Filtering, HTTPS

- Maximum User Access: 10 (Live), 1 (Recording), 1 (Search), 2 (Admin)

Alarm and Event Specification

- Alarm Input / Output: 1 / 1

  - Alarm Input: 1 TTL, NC/NO Programmable, 4.3V(NC) or 0.3V(NO) threshold, 5V DC

  - Alarm Output: 1 TTL open collector, 30mA @ 5 VDC

- Trigger Events: Motion Detection, Alarm in, Audio detection, Tampering and Trip Zone

- Event Notification: Remote S/W, Email (with Image)

Environmental Specification

- Operating Temperature: -20°C ~ +55°C (-4°F ~ +131°F)

* Starting up at above 0°C (32°F)

- Operating Humidity: 0% ~ 90%

- Vandal-proof Enclosure: IK10

- Outdoor Ready: IP67

Electrical Specification

- Power Source: PoE(IEEE 802.3af class 2)

- Power Consumption: 4.6W

- Regulatory Approvals: FCC, CE, KC

Mechanical Specification

- Dimensions (Ø x H): Ø120 x 92.9mm (Ø4.72” x 3.65”)

- Unit Weight: 0.465kg(1.02lb)

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.2 | Kelly Choi | Feb. 25. 2025 | Spec Update |
| 1.1 | TS Team | Aug. 26, 2022 | Spec Update |
| 1.0 | TS Team | Feb. 18. 2020 | Initial release |