DC-D4536RX

Architectural and Engineering Specifications

Version 1.1

(Jul. 08th, 2022)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Level 1 - 28.20.00 – Video Surveillance

Level 2 - 28.21.00 – Surveillance Cameras

Level 3 - 28.21.13 – IP Cameras

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 13493, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

General

Product Description

DC-D4536RX is a Dome type IP Camera followed by NDAA Compliance designed and manufactured by IDIS. This camera provides 5MP (2592 x 1944) resolution at 30ips (images per second) with H.265, H.264, and M-JPEG compression. This camera is equipped with Motorized Focus/Zoom lens, IR LEDs, True Day/Night, PoE (IEEE 802.3af Class 3), Alarm I/O and microSD/SDHC/SDXC card backup (up to 256GB).

General Specification

- The IP camera shall be equipped with 5 Megapixel 1/2.8” CMOS Sensor.

- The IP camera shall be equipped with 3.0mm – 13.5mm motorized vari-focal lens, F1.4 – F3.2.

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance.
The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have wide dynamic range compensation (True WDR) for improved video quality in high-contrast situations (120 dB).

- The IP camera shall be equipped with 6ea Infrared LED with range up to 30m (98.4ft).

- The IP camera shall support DC-Iris.

- The IP camera shall utilize 2DNR/3DNR (Dynamic Noise Reduction) technology to reduce the bitrate and storage requirements by removing noise artifacts.

- The IP camera shall be equipped with 10/100 Base-T, auto-sensing, half/full duplex, RJ-45 Ethernet connection.

- The IP camera shall support industry standard Power over Ethernet (PoE) IEEE 802.3af, Class 3 to supply power to the camera over the network and 12VDC input.

- Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.

- The IP camera shall deliver maximum video resolution of 2592x1944 at rates up to 30ips (images per second).

- The IP camera shall provide network connection using H.265, H.264 and MJPEG** compression.

- The IP camera shall support quadruple streams in DirectIP 2.0 protocol mode.

- The IP camera shall conform to the ONVIF ver.16.12.

- The IP camera shall be equipped with embedded web server (IDIS Web) which works independently using a Web Browser with ActiveX plug-in.

- The IP camera shall have IP filtering, HTTPS, SSL Encryption, IEEE 802.1X, and configurable user authority levels for greater security.

- The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.

- The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Intelligent Video Analysis (VA): Motion Detection, Alarm in, Tampering and Trip Zone.

Protocol Specification: DirectIP 2.0

- The IP camera shall have DirectIP 2.0 mode.

- DirectIP 2.0 protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

- DirectIP 2.0 shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

- DirectIP 2.0 shall support camera can be linked to IDIS software solution such ad IDIS Center and IDIS Solution Suite, or 3rd party solution while it is being connected to a DirectIP NVR.

- DirectIP 2.0 camera shall be compatible with DirectIP 1.0 NVR as well as DirectIP 2.0 NVR.

- DirectIP 2.0 camera shall be unavailable for No-password login when connecting to DirectIP 2.0 NVR and IDIS Software Solutions.

- DirectIP 2.0 protocol shall provide Quadruple streams.

- DirectIP 2.0 protocol shall support H.264 and H.265 and MJPEG compression.

Technical Specification

Video Specification

- Image Sensor: 1/2.8” CMOS

- Maximum Resolution: 2592 x 1944

- Scanning Mode: Progressive Scan

- Lens Type: Motorized Vari-focal (f=3.0 – 13.5mm, F1.4 – 3.2)

- Iris Control: DC-Iris

- Angular Field of View (H: Horizontal, V: Vertical, D:Diagonal):

  - Wide: 91º(H) 66º(V) 120º(D)

  - Tele: 31º(H) 23º(V) 38(D)

- Pan/Tilt/Rotate Range : Pan: -175° ~ 175°, Tilt: 0° ~ 67°, Rotate: -177° ~ 177°

- Minimum Illumination:

  - COLOR: 0.15 lux @ F1.4

  - B/W : 0 lux  (IR LED ON)

- S/N Ratio: More than 45 dB

- Maximum Frame Rate:

  - 4:3 mode : 30ips : 2592 x 1944(WDR)

  - 16:9 mode : 30ips : 2592 x 1456(WDR)

- Video Resolution:

  - 4:3 : 2592x1944, 1920x1440, 1280x960x 640x480

  - 16:9 : 2592x1456, 1920x1080, 1280x720, 640x360

- Video Compression : H.265, H.264, MJPEG

- Video Compression Level: 4 levels: Basic, Standard, High, Very High

- Intelligent Codec is supported.

- Multi-Video Streaming: Quadruple streams

- Dynamic Range: 120dB

- True Day & Night: Yes (ICR)

- IR Distance (The number of LEDs, IR wavelength): 30m (98.4ft.) (6ea)

- Intelligent Video Analytic: Motion Detection, Alarm in, Tampering and Trip Zone

Audio Specification

- Audio Compression Algorithm: ADPCM 16K, G.726, G.711 u-Law, G.711 a-Law

- Audio Input / Output: 1(Built Mic) + 1 / 1

- Two-way Audio Communication: Yes

Network Specification

- Port: RJ-45 10/100 Base-T 1 port

- Network Protocols: DirectIP 2.0 Protocol, RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP, RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, uPNP

- Streaming Mode: Quadruple Streaming

Security Specification

- SSL Encryption, Multi-User Authority, IEEE 802.1x, IP Filtering, HTTPS

- Maximum User Access: 10 (Live + Admin), 1 (Recording), 1 (Search)

Alarm and Event Specification

- Alarm Input / Output: 1 / 1

  - Alarm Input: TTL, NC/NO Programmable, 4.3V(NC) or 0.3V(NO) threshold, 5V DC

  - Mechanical or electrical switches can be wired to the Alarm-In and GND connectors. The maximum voltage should not exceed 5V.

  - Alarm Output: 1 TTL open collector, 30mA @ 5 VDC

- Trigger Events: Motion Detection, Alarm in, Tampering and Trip Zone

- Event Notification: Remote S/W, Email (with Image)

  - Encryption Type: SSL

Environmental Specification

- Operating Temperature: -10°C ~ +50°C (+14°F ~ +122°F)

*Starting up at above 0°C (32°F)

- Operating Humidity: 0% to 90% non-condensing

Electrical Specification

- Power Source: PoE (IEEE 802.3af class 3), 12VDC

- Power Consumption:

    - PoE, IEEE 802.3af Class 3, 9.5W

    - 12V=, 1A, 12W

- Regulatory Approvals: FCC, CE, KC

Mechanical Specification

- Dimensions (Ø x H): Ø122.4 x 103.4mm

- Unit Weight: 0.4Kg (0.88lb)

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.1 | Tech support | Mar. 11, 2022 | Initial Release |