DC-T1445WRA

Architectural and Engineering Specification

Version 1.1

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Level 1 - 28.20.00 – Video Surveillance

Level 2 - 28.21.00 – Surveillance Cameras

Level 3 - 28.21.13 – IP Cameras

## Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 13493, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

## General

### Product Description

DC-T1445WRA is a bullet-type IP camera that provides 4MP resolution at 30 fps with H.265/H.264/MJPEG. This LPR(License Plate Recognition) camera captures real-time license plates with high accuracy, even in extreme glare or darkness, ensuring reliable license plate recognition.

### Key Features

- Superior Image Quality (Starlight Sensor, Smart IR II Technology)

- LPR Smart Search (Relevant videos can be found quickly by searching for only a portion of the target suspect vehicle’s features, such as vehicle type & color, plate color, plate number, direction, etc.)

- Two models are available, offering 850nm and 740nm IR illumination to enhance regional license plate recognition performance.

- NDAA compliant

- ONVIF compatible

- IDIS Solution Suite compatible: Live Monitoring, Event Notification, Event Recording, and Event Search

Caution 1: Only the vehicle detection time and license plate number are supported as event data in the IDIS Solution Suite. Other camera features are not compatible with the IDIS Solution Suite.

Caution 2: It is compatible with the following IDIS Solution Suite versions and add-on software. If compatibility with other IDIS Solution Suites is required, please discuss with IDIS technical staff.

  - IDIS Solution Suite: 5.0.0 version

  - Add-On: ISS_Add_On_for_IDIS_VAI_and_DC-T1445WRA_LPR_v5.1.0_20250520.zip

- DirectIP NVR not compatible

## Technical Specification

### Video Specification

- Image Sensor: 1/1.8″ Progressive Scan CMOS

- Maximum Resolution: 2688 x 1520

- Scanning Mode: Progressive Scan

- Lens Type: Motorized Vari-focal, Auto-Focus

- Focal Length: 8 ~ 32mm

- Aperture: F1.6 ~ F1.7

- IRIS Control: P-Iris

- Angular Field of View (H: Horizontal, V: Vertical, D:Diagonal):

  - Wide: 44º(H) 25º(V) 51º(D)

  - Tele: 13º(H) 7º(V) 15º(D)

- Minimum Illumination:

  - Color: 0.005Lux@F1.6

  - B/W: 0 lux (IR LED ON)

- Dynamic Range: 120dB (True WDR)

- Electronic Shutter Speed: 1/100000s ~ 1s

- Day and Night: Yes (Day/Night/Auto/Customize/Schedule)

- IR Distance: Up to 590ft (180m)

- IR Wavelength: 850nm (Default), 740nm (Optional)

- Image Setting: Exposure, White Balance, Self-Adjusting Video Mode(Sharpness, Contrast, Colors, Brightness Control), AGC, Anti-flicker, Deblur, Watermark

- Backlight Compensation: Yes

- Highlight Compensation: Yes

- Mirroring (Flip): Horizontal / Vertical

- Pivot (Corridor Mode): Yes

- Image Stabilizer: DIS

- De-fog: Yes

- HLC: Yes

- DNR: Yes (2D / 3D)

- Privacy Masking: Up to 8 zones (Mask Area and Mosaic Area are optional)

- Intelligent Video: Black/WhiteList, Attributes Event, Motion Detection

- S/N Ratio: More than 55dB

- ROI: Up to 8 areas

### Network Specification

- Port: RJ-45 10/100 Base-T 1 Ethernet port

- Video Compression: H.265 / H.264 / MJPEG

- Intelligent Codec: No

- Bitrate Control: 16Kbps ~ 16Mbps (CBR/VBR Adjustable)

- IP Camera Protocol: API, HTTP, TCP, ONVIF compatible (Profile S, T, G, and M)

  - Note) The Profile G and M are not compatible with IDIS Solution Suite.

- Multi-video Streaming

  - Primary Stream

    - 60Hz: 60fps@(2688x1520, 2304x1296, 1920x1080, 1280x720)

    - 50Hz: 50fps@(2688x1520, 2304x1296, 1920x1080, 1280x720)

  - Secondary Stream

    - 60Hz: 60fps@(704x576, 640x480, 640x360, 352x288, 320x240)

    - 50Hz: 50fps@(704x576, 640x480, 640x360, 352x288, 320x240)

  - Tertiary Stream

    - 60Hz: 30fps@(1280x720, 704x576, 640x480, 640x360, 320x240)

    - 50Hz: 25fps@(1280x720, 704x576, 640x480, 640x360, 320x240)

- Network Protocols

  - IPv4/IPv6, ARP, TCP, UDP, RTCP, RTP, RTSP, RTMP, HTTP, HTTPS, DNS, DDNS, DHCP, FTP, NTP, SMTP, SNMP, UPnP, Bonjour, SIP, PPPoE, VLAN, 802.1x, QoS, IGMP, ICMP, SSL

- Recording Session Buffer (NLTSrec): No

- Edge Storage: Support microSD/SDHC/SDXC Card Local Storage, up to 1TB

  - Note) This feature is not compatible with IDIS Solution Suite.

### Audio Specification

- Audio In / Out: 1 / 1

- Audio Compression: G.711 / AAC / G.722 / G.726

- Audio Sampling Rate: 8/16/32/44.1/48KHz

- Audio Bit Rate: 16 ~ 256kbps

- Two-way Audio: Yes

- SIP/VOIP support: Yes, Voice & Video-over-IP

### Alarm and Event Specification

- Alarm Input / Output: 1 / 1

- Trigger Event: Black/White List, Attributes Event, Motion Detection, Network Disconnection, External Input, Audio Alarm

- Event Action: FTP Upload, SMTP Upload, SD Card Record, External Output, SIP Phone, HTTP Notification

### Vehicle License Recognition Specification

- Max. Vehicle Speed: 120 km/h (75 mile/h)

  - Recommended: 90 km/h (56 mile/h) or less

- Recognition Accuracy: Daytime > 95%, Nighttime > 90% (depending on the camera installation environment)

- Coverage: 2 lanes

- ANPR Distance:

  - Up to 95m (295ft) during the day

  - Up to 45m (147.5ft) during the night with illumination

- Country Region Supported: More than 80 countries and regions

  - Recommended Illuminator Wavelengths for LPR Across the U.S.

    - 850nm: Alaska, Arizona, California, Colorado, Connecticut, Delaware, Georgia, Hawaii, Idaho, Kansas, Kentucky, Maine, Maryland, Michigan, Minnesota, Missouri, Nebraska, Nevada, New Jersey, New York, North Dakota, Ohio, Oklahoma, Oregon, Pennsylvania, South Carolina, South Dakota, Texas, Utah, Virginia, Washington, West Virginia, Wisconsin, Wyoming

    - 740nm: Alabama, Arkansas, Florida, Illinois, Indiana, Iowa, Louisiana, Massachusetts, Mississippi, Montana, North Carolina, New Hampshire, New Mexico, Rhode Island, Tennessee, Vermont

- Attributes Identification: Plate Color, Vehicle Type, Vehicle Color, Vehicle Brand, No-plate Vehicle Capture, Vehicle Direction Detection

- Smart Traffic Function: Evidence Camera Linkage, Specific Lane Management

- White/Black List: Up to 20,000 records inside the camera

- LPR Smart Search: Yes

### Environmental Specification

- Vandal-proof Enclosure: IK10-rated Metal Housing

- Outdoor Ready: IP67

- Operating Temperature: -40°C ~ +60°C (-40°F ~ +140°F)

- Operating Humidity: 5 ~ 90% (Non-condensing)

- Surge Protection: 4KV

- Warranty: 3 Years

### Electrical Specification

- Power Source: DC 12VDC ± 10%, PoE (IEEE 802.3at)

- Power Consumption: Max. 8W (with IR off), Max. 13W (with IR on)

- Regulatory Approvals: FCC, CE

### Mechanical Specification

- Dimensions: 320mm X 134mm X 171mm (12.60" x 5.28" x 6.73")

- Unit Weight: 1.623kg (3.578lb)

# Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.1 | IDIS AM SE | Jul. 09. 2025 | Add mile and ft units Add the recommended Illuminator Wavelengths for LPR Across the U.S. |
| 1.0 | IDIS AM SE | Mar. 24. 2025 | Initial Release |