DC-T4831HRX-A

Architectural and Engineering Specifications

Version 1.0

(May. 21, 2021)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Level 1 - 28.20.00 – Video Surveillance

Level 2 - 28.21.00 – Surveillance Cameras

Level 3 - 28.21.13 – IP Cameras

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 13493, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

General

Product Description

DC-T4831HRX-A is a Bullet type IP Camera designed and manufactured by IDIS. This camera provides 8M (3840 x 2160) resolution at 30ips (images per second) with H.265/H.264/M-JPEG compression. This camera is equipped with Motorized Vari-focal lens, True Day/Night, PoE (IEEE 802.3af Class 3), IR LED, Alarm I/O, microSD/SDHC/SDXC Smart Failover (up to 256GB), Vandal-proof bullet enclosure design, IP67 rated and heater provides continuous operation in subzero temperature.

General Specification

- The IP camera shall be equipped with 8 Megapixel 1/2.8” CMOS Sensor.

- The IP camera shall be equipped with 2.7mm – 13.5mm Motorized Vari-focal lens, F1.8 – 3.32.

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance. The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have Wide Dynamic Range compensation (Digital WDR) for improved video quality in high-contrast situations (120dB).

- The IP camera shall be equipped with 6ea Infrared LED with range up to 40m (131.2ft.).

- The IP camera shall utilize configurable 2DNR/3DNR (Dynamic Noise Reduction) technology to reduce the bitrate and storage requirements by removing noise artifacts.

- The IP camera shall be Vandal-Proof casing IK10, IP rating 67 and heater.

- IP camera shall be equipped with 10/100 Base-T, auto-sensing, half/full duplex, RJ45 Ethernet connection.

- The IP camera shall support industry standard Power over Ethernet (PoE) IEEE 802.3af, Class 3 to supply power to the camera over the network and 12VDC input.

- The IP camera shall have built-in heater for continued use in subzero temperature conditions.

- Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.

- The IP camera shall deliver maximum video resolution of 3840 x 2160 at rates up to 30ips (Images per second).

- The IP camera shall provide direct network connection using H.265, H.264 and M-JPEG compression.

- The IP camera shall support Quadruple Streams.

- The IP camera shall conform to the ONVIF Profile S Ver. 2.4.0 standard.

- The IP camera shall be equipped with embedded web server which works independently using a Web Browser with ActivX plug-in.

- The IP camera shall have SSL Encryption, Multi-user Authority, IEEE 802.1x IP Filtering and HTTPS for greater security.

- The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.

- The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Intelligent Video Analysis (VA): Video Motion Detection, Active Tampering Alarm and Trip Zone.

Protocol Specification: DirectIP 2.0

- The IP camera shall have DirectIP 2.0 mode.

- DirectIP 2.0 protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

- DirectIP 2.0 shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

- DirectIP 2.0 shall support camera can be linked to IDIS software solution such ad IDIS Center and IDIS Solution Suite, or 3rd party solution while it is being connected to a DirectIP NVR.

- DirectIP 2.0 camera shall be compatible with DirectIP 1.0 NVR as well as DirectIP 2.0 NVR.

- DirectIP 2.0 camera shall be unavailable for No-password login when connecting to DirectIP 2.0 NVR and IDIS Software Solutions.

- DirectIP 2.0 protocol shall provide Quadruple streams.

- DirectIP 2.0 protocol shall support H.264 and H.265 and MJPEG compression.

Technical Specification

Video Specification

- Image Sensor: 1/2.8" CMOS

- Maximum Resolution: 3840 x 2160

- Scanning Mode: Progressive Scan

- Lens Type: Motorized Vari-focal (f= 2.7 – 13.5mm, F1.8 – 3.32)

- IRIS Control: DC Iris

- Angular Field of View (H: Horizontal, V: Vertical, D:Diagonal):

  - Wide : 110.6º(H) 57.3º(V) 134.1º(D)

  - Tele : 32.6º(H) 18.3º(V) 37.4(D)

- Minimum Illumination:

  - COLOR : 0.2 lux @ F1.8

  - B/W : 0 lux (IR LED ON)

- S/N Ratio: More than 45dB

- Maximum Frame Rate:

  - 30ips : 3840 x 2160

  - 15ips : 3840 x 2160(WDR)

- Video Resolution: 3840 x 2160, 1920 x 1080, 1280 x 720, 640 x 360

- Video Compression : H.265, H.264, M-JPEG

- Intelligent Codec is supported.

- Video Compression Level: Basic, Standard, High, Very High

- Multi-Video Streaming: Quadruple streams

- Dynamic Range: 120dB

- True Day & Night: Yes (ICR)

- IR Distance (The number of LEDs): 40m (131.2ft.) (6ea)

- Intelligent Video Analytic: Video Motion Detection, Active Tampering Alarm, Trip Zone

Network Specification

- Port: RJ-45 10/100 Base-T 1 port

- Network Protocols: Network Protocols: DirectIP 2.0 Protocol, RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP, RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, Upnp

- Streaming Mode: Quadruple streaming

Security Specification

- SSL Encryption, Multi-User Authority, IEEE 802.1x, IP Filtering, HTTPS

- Maximum User Access: 10 (Live), 1 (Recording), 1 (Search), 2 (Admin)

Alarm and Event Specification

- Alarm Input / Output 1/1

  - Alarm Input: 1 TTL, NC/NO Programmable, 4.3V(NC) or 0.3V(NO) threshold, 5V DC,

  - Mechanical or electrical switches can be wired to the Alarm-In and GND connectors. The maximum voltage should not exceed 5V.

  - Alarm Output: 1 TTL open collector, 30mA @ 5 VDC

- Trigger Events: Motion Detection, Alarm in, Tampering, Trip Zone

- Event Notification: Remote Software, Email (with Image)

Environmental Specification

- Operating Temperature: -40°C ~ +55°C (-40°F ~ +131°F)

  - Starting up at above -20°C (-4°F)

- Operating Humidity: 0% ~ 90%

- Vandal-proof Enclosure: IK10

- Outdoor Ready: IP67, Heater

Electrical Specification

- Power Source: 12VDC, PoE(IEEE 802.3af class 3)

- Power Consumption:

  - 12V=, 1.1A, 13.2W

  - PoE, IEEE 802.3af(Class 3), 11.7W

- Regulatory Approvals: FCC, CE, KC

Mechanical Specification

- Dimensions (W x H): 80mm X 75mm X 237mm(3.15" x 2.95" x 9.33")

- Unit Weight: 1.006 kg (2.22 lb)

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.0 | TS Team | May. 21, 2021 | Initial Release |