DC-Y6516X

Architectural and Engineering Specifications

Version 1.3

(Feb. 14, 2025)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Level 1 - 28.20.00 – Video Surveillance

Level 2 - 28.21.00 – Surveillance Cameras

Level 3 - 28.21.13 – IP Cameras

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 13493, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

General

Product Description

DC-Y6516X is a Network Camera (IP Camera) designed and manufactured by IDIS. This camera provides 5MP (2560x2048) resolution at 30ips (images per second) with H.265/H.264/M-JPEG compression. This camera is equipped with Fixed-focal lens, IR LEDs, True Day/Night, PoE (IEEE 802.3af Class 2), Alarm I/O, Audio I/O, microSD/SDHC/SDXC card backup (Smart Failover up to 512GB)

General Specification

- The IP camera shall be equipped with 5 Megapixel 1/1.8” CMOS Sensor.

- The IP camera shall be equipped with 1.5mm Fixed-focal lens, F1.8.

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance.
The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have wide dynamic range compensation (Digital WDR) for improved video quality in high-contrast situations (120dB).

- The IP camera shall support fixed Iris.

- The IP camera shall utilize configurable 3DNR (Dynamic Noise Reduction) technology to reduce the bitrate and storage requirements by removing noise artifacts.

- The IP camera shall be equipped with 10/100 Base-T, auto-sensing, half/full duplex, RJ-45 Ethernet connection.

- The IP camera shall support industry standard Power over Ethernet (PoE) IEEE 802.3af, Class 2 to supply power to the camera over the network

- The IP camera shall have on board microSD/SDHC/SDXC card backup storage slot as a safeguard against data loss during network interruptions (Smart Failover up to 512GB).

- Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.

- The IP camera shall deliver maximum video resolution of 2560x2048 at rates up to 30ips (images per second).

- The IP camera shall provide direct network connection using H.265, H.264 and M-JPEG compression.

- The IP camera shall support Quadruple Streams.

- The IP camera shall conform to the ONVIF Profile S/T.

- The IP camera shall be equipped with embedded web server which works independently using a Web Browser with ActiveX plug-in.

- The IP camera shall have IP filtering, HTTPS, SSL, IEEE 802.1X, and configurable user authority levels for greater security.

- The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.

- The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Intelligent Video Analysis (VA): Video Motion Detection, Active Tampering Alarm, Trip Zone and Heatmap.

- The IP camera shall support De-fog function

Protocol Specification: DirectIP 2.0

- The IP camera shall have DirectIP 2.0 mode.

- DirectIP 2.0 protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

- DirectIP 2.0 shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

- DirectIP 2.0 shall support camera can be linked to IDIS software solution such ad IDIS Center and IDIS Solution Suite, or 3rd party solution while it is being connected to a DirectIP NVR.

- DirectIP 2.0 camera shall be compatible with DirectIP 1.0 NVR as well as DirectIP 2.0 NVR.

- DirectIP 2.0 camera shall be unavailable for No-password login when connecting to DirectIP 2.0 NVR and IDIS Software Solutions.

- DirectIP 2.0 protocol shall provide Quadruple streams.

- DirectIP 2.0 protocol shall support H.264 and H.265 and MJPEG compression.

Technical Specification

Video Specification

- Image Sensor: 1/1.8” CMOS

- Maximum Resolution: 2560x2048

- Scanning Mode: Progressive Scan

- Lens Type: Fixed-focal (f=1.5mm, F1.8)

- Iris Control: Fixed Iris

- Angular Field of View: 180º(Horizontal), 180º(Vertical), 180º(Diagonal)

- Minimum Illumination:

  - Color: 0.15 lux @ F1.8

  - B/W: 0.015 lux @ F1.8

- S/N Ratio: 45dB

- Maximum Frame Rate: 30ips @ 2560x2048 (WDR)

- Video Resolution: 2560x2048, 640x512

- Video Compression: H.265, H.264 and M-JPEG

- Video Compression Level: Basic, Standard, High, Very High

- Multi-Video Streaming: Quadruple streams

- Dynamic Range: 120dB (Ture WDR)

- True Day & Night: Yes (ICR)

- Intelligent Video Analytic: Video Motion Detection, Active Tampering Alarm, Trip Zone and Heatmap

- De-fog : Yes

Network Specification

- Port: RJ-45 10/100 Base-T 1 port

- Network Protocols: DirectIP 2.0 Protocol, IPv4, IPv6, RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP,RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, uPNP, SNMPv2

- Streaming Mode: Quadruple streaming

Security Specification

- SSL Encryption, Multi-User Authority, IEEE 802.1x, IP Filtering, HTTPS

Audio Specification

- Audio Compression Algorithm: ADPCM 16K, G.726, G.711 u-Law, G.711 a-Law

- Audio Input / Output: Line-in 1ea(Built in MIC) / Line-out 1ea

- Two-way Audio Communication: Yes

Alarm and Event Specification

- Alarm Input / Output: 1 / 1

  - Alarm Input: 1 TTL, NC/NO Programmable, 4.3V(NC) or 0.3V(NO) threshold, 5V DC

  - Mechanical or electrical switches can be wired to the Alarm-In and GND connectors. The maximum voltage should not exceed 5V.

  - Alarm Output: 1 TTL open collector, 30mA @ 5 VDC

- Trigger Events: Motion Detection, Alarm in, Tampering and Trip Zone

- Event Notification: Remote S/W, Email (with Image)

Environmental Specification

- Operating Temperature: -10°C ~ +50°C (-22°F ~ +122°F)

- Operating Humidity: 0% to 90% non-condensing

Electrical Specification

- Power Source: PoE(IEEE 802.3af class 2)

- Power Consumption:

  - PoE (IEEE 802.3af Class 2), 5.6W

- Regulatory Approvals: FCC, CE, KC

Mechanical Specification

- Dimensions (Ø x H): Ø122mm x 35.8mm (Ø4.80" x 1.40")

- Unit Weight: 0.28kg (0.62lb)

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.0 | TS Team | May. 18, 2021 | Initial Release |
| 1.1 | TS Team | Aug. 31, 2022 | Spec Update |
| 1,2 | Roy Lee | Mar. 26. 2024 | Added Onvif T, De-fog function |
| 1.3 | Suny kwon | Feb. 14. 2025 | Editted data(smart failover and etc) |