DH-2010P

Architectural and Engineering Specifications

Version 1.2

(Aug. 26, 2022)

Table of Contents

1	Manufacturer	3

2	General	3

2.1	Product Description	3

2.2	General Specification	3

3	Technical Specification	3

3.1	Hardware Specification	3

3.2	Network Specifications	3

3.3	Power Over Ethernet	3

3.4	Electrical Specifications	4

3.5	Mechanical Specifications	4

3.6	Environmental Specifications	4

Version History	5

Manufacturer

IDIS Co., Ltd.

IDIS Tower, 344 Pangyo-ro, Bundang-gu, Seongnam-si, Gyeonggi-do, 13493, Korea

Tel: +82 31 723 5400

Fax: +82 31 723 5100

General

Product Description

DH-2010P is a PoE switch designed and manufactured by IDIS. The PoE switch provides data transmission and power at the same time to the connected network cameras through Ethernet cables by using PoE-enabled ports without need of a separate power outlet for each network camera and it will reduce the installation costs. Two of service ports are RJ-45 connectors that support 10/100/1000 BASE-T without PoE (Power over Ethernet) function. The other ports are RJ-45 connectors that support 10/100 BASE-T with PoE function.

General Specification

- The switch shall support 8 RJ45 connectors at 10/100 BASE-T for service ports.

- The switch shall support 2 RJ45 connectors at 10/100/1000 BASE-T for Uplink ports.

- The switch shall support IEEE 802.3af-2003 and IEEE 802.3at-2009 PoE standard.

- The switch shall support up to 8 IEEE 802.3af PDs(Powered Devices) or 2 IEEE 802.3at PDs.

- The switch shall support 70W for PSE Power Budget.

Technical Specification

Hardware Specification

- CPU: Embedded MIPS

- System Memory: 256MB SDRAM, 32M Flash

- Switching Capacity: 5.6Gbps

Network Specifications

- Copper Service Port: 8 RJ-45 Connectors(10/100 BASE-T ports)

- Copper Uplink Port: 2 RJ-45 Connectors(10/100/1000 BASE-T ports)

- Optical Uplink Port: N/A

Power Over Ethernet

- IEEE 802.3af/at compliant

- Power to application over Ethernet cabling

- Central power control for greater network utility

- PoE alternate B power supply

- Power feeding over 4/5 & 7/8 data twisted.

- PoE-supported Network Camera Connection

  - Up to 8 IEEE 802.3af cameras or 2 IEEE802.3at cameras

Electrical Specifications

- Power Supply Equipment power budget: 70W

- Power Consumption: 90W

- Power Input: AC100-240V, 2.0~1.0A, 50/60Hz

- Regulatory Approvals:

  - Electrical: FCC,CE,KC,UL,CB

Mechanical Specifications

- Unit Dimensions (W x H x D): 280mm x 44mm x 180mm (11.0" x 1.7" x 7.1")

- Unit Weight: 1.35Kg (2.97 lbs.)

Environmental Specifications

- Operating Temperature: 0°C ~ 40°C (32℉~104℉)

- Operating Humidity: 10~90%

- Storage Temperature: -30°C ~ 60°C (-22°F ~ 140°F)

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.2 | Tei Jeong | Aug. 26. 2022 | Spec Updated |
| 1.1 | Daniel Lee | Jan. 28. 2021 | L2 feature unsupported |
| 1.0 | Glen Oh | May. 26. 2018 | Initial Release |