DH-2328PF-A

Architectural and Engineering Specifications

Version 1.0

(Apr. 13, 2023)

Table of Contents

1	Manufacturer	3

2	General	3

2.1	Product Description	3

2.2	General Specification	3

3	Technical Specification	3

3.1	Network features	3

3.2	Power Over Ethernet	3

3.3	Electrical Specifications	4

3.4	Mechanical Specifications	4

3.5	Environmental Specifications	4

Version History	5

Manufacturer

IDIS Co., Ltd.

IDIS Tower, 344 Pangyo-ro, Bundang-gu, Seongnam-si, Gyeonggi-do, 13493, Korea

Tel: 	+82 31 723 5400

Fax: 	+82 31 723 5100

General

Product Description

DH-2328PF-A is a PoE switch designed and manufactured by IDIS. The PoE switch provides data transmission and power at the same time to the connected network cameras through Ethernet cables by using PoE-enabled ports without need of a separate power outlet for each network camera and it will reduce the installation costs. 24 ports of service ports are RJ-45 connectors that support 10/100/1000 BASE-T with PoE(Power over Ethernet). The other 4 ports are SFP slots that support 1000BASE-X without PoE function.

General Specification

- The switch shall support 24 RJ45 connectors at 10/100/1000 BASE-T for Service port.

- The switch shall support 4 SFP slots at 1000 BASE-X.

- The switch shall support IEEE 802.3af-2003 and IEEE 802.3at-2009 PoE standard.

- The switch shall support up to 24 PDs(Powered Device) at IEEE 802.3af or 12 PDs at IEEE 802.3at.

- The switch shall support 370W for PSE Power Budget.

Technical Specification

Network features

- Copper Service Port: 24 RJ-45 Connectors(10/100/1000 BASE-T ports)

- Optical Uplink Port: 4 SFP Connectors(1000BASE-X)

Power Over Ethernet

- IEEE 802.3af-2003 and IEEE 802.3at-2009 compliant

- Power to application over Ethernet cabling

- Central power control for greater network utility

- PoE alternate A power supply

- PoE-supported Network Camera Connection

  - Up to 24 IEEE 802.3af PDs or 12 IEEE 802.3at PDs

Electrical Specifications

- Power Supply Equipment power budget: 370W

- Power Consumption: 400W

- Power Input: 100-240V, 2.0~1.0A, 50/60Hz, 160W

- Regulatory Approvals:

- Electrical: FCC, CE, KC

Mechanical Specifications

- Unit Dimensions (W x H x D): 440mm x 44mm x 208mm (17.3" x 1.7" x 8.2")

- Unit Weight: 2.97kg (6.54lb)

Environmental Specifications

- Operating Temperature: 0°C ~ 45°C (32℉~113℉)

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.1 | Global Tech Team | Dec, 12, 2023 | Unit Weight modification |
| 1.0 | Global Tech Team | Apr, 13, 2023 | Initial Release |