GUIDE SPECIFICATION

Section 28 23 29

[image: image1.png]



Digital Watchdog® is a leading manufacturer of security and surveillance solutions, offering stunning image quality, advanced hardware capabilities, reliable customer support and lowest total cost of deployment to the analog & IP megapixel surveillance markets. Located in Cerritos, CA with manufacturing facilities in Seoul, Korea, Digital Watchdog® is committed to delivering powerful security solutions to its customers worldwide.
For additional information, contact: 

Digital Watchdog®

16220 Bloomfield Avenue, 

Cerritos, California 90703 USA


Phone: +1 888 446-3593


Web: www.digital-watchdog.com


E-mail: dw-tech@dwcc.tv
COMPRESSOR™ 4K HD ANALOG TO IP ONVIF ENCODER
DIVISION 28 – ELECTRONIC SAFETY AND SECURITY
28 20 00 
Video Surveillance
28 23 00 
Video Management System
28 23 29 
Video Surveillance Remote Devices and Sensors
Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.
2.  Explanatory notes and comments are presented in colored text.

3. Include related sections as appropriate if embedded digital video recorder system is integrated to other systems 

4. CSI Master Format 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although the adoption of Master Format 2016 is encouraged. The following is a guide to the Master Format numbers relevant to the product referenced in this specification.

4. MasterFormat 2014 Specification Category: 

28 23 29 - Video Surveillance Remote Devices and Sensors
COMPRESSOR™ 4K HD ANALOG TO IP ONVIF ENCODER
PART 1   GENERAL

1.01 SECTION INCLUDES
A. Product - an H.265, H.264 16-channel 4K analog to IP encoder system for video surveillance, including design, supply, installation, and commissioning.
B. Related Requirements

1. Section 26 05 00: Common Work Results for Electrical, for interface and coordination with building electrical systems and distribution.

2. Section 28 05 13: Conductors and Cables for Electronic Safety and Security, for cabling between system servers, panels, and remote devices. 

3. Section 28 05 28: Pathways for Electronic Safety and Security, for conduit and raceway requirements.

4. Section 28 23 13: Video Surveillance Control and Management Systems.

5. Section 28 23 16: Video Surveillance Monitoring and Supervisory Interfaces.

6. Section 28 23 23: Video Surveillance Systems Infrastructure.

7. Section 28 23 29: Video Surveillance Remote Devices and Sensors.

1.02 REFERENCES
A. Reference Standards: Provide systems that meet or exceed the requirements of the following publications and organizations as applicable to the work of this Section.

1. Conformity for Europe (CE).

2. Electronic Industry Association (EIA).

3. Federal Communications Commission (FCC).

4. Restriction of the Use of Certain Hazardous Substances (RoHS).

5. Underwriters Laboratories Inc. (UL)
1.03 SYSTEM DESCRIPTION
A. The 16-channel 4K analog to IP encoder system shall provide an entry-level and work with up to 16 channel analog cameras to encode and transmit video over the network. The encoder shall support traditional analog or 960H cameras, and HD-Analog and HD-TVI cameras, streaming at up to 4K resolution. The encoder shall offer true HD, VGA and digital spot local outputs for local configuration.
B. The encoder shall provide an analog connection to allow users to integrate analog camera products into IP applications.

1.04 SUBMITTALS
A. General: Submittals shall be made per the Conditions of the Contract and Submittal Procedure Section. 

B. Manufacturer’s Product Data: Submit manufacturer’s data sheets indicating systems and components proposed for use, including instruction manuals.

C. Shop Drawings: Submit installation drawings, including connection diagrams for interfacing equipment, a list of connected equipment, and locations for major equipment components. Shop drawings shall indicate surrounding construction as provided for the Project.

D. Project Record Drawings: Indicate the location of equipment and wiring on project record drawings. Submit an electronic version of the project record drawings not later than Substantial Completion of the Project.

E. Operation and Maintenance Data: Submit manufacturer’s operation and maintenance data customized to the system installed. Include operator manuals.

F. Field Tests: Submit results of field testing of every device, including date, testing personnel, retesting date (if applicable), and confirmation that every device passed field testing.

1. Maintenance Service Agreement: Submit a sample copy of the manufacturer’s maintenance service agreement, including cost and services for one year for the Owner’s review. Maintenance shall include, but not be limited to, labor and materials to repair the system, tests and adjustments, and regular inspections.
1.05 QUALITY ASSURANCE

A. Qualifications: Manufacturers shall have a minimum of 10 years of full-time experience in manufacturing and maintaining digital video recorder systems. The manufacturer shall provide toll-free technical assistance and support available Monday thru Friday, 8:00 AM to 8:00 PM EST. Installers shall have a minimum of 2 years of experience installing similar systems and shall be acceptable to the manufacturer of the digital video recorder system.

B. Regulatory Requirements: 

1. Emissions: FCC, Part 15, Class A; CE (EN 55022).

2. Immunity: CE (EN 50130-4).

3. Safety: UL/CSA 60950-1; CE (EN 60950-1).

C. Power Requirements: Input voltage shall be 12 V DC, 2 A.

1.06 DELIVERY, STORAGE, AND HANDLING

A. Packing and Shipping: Deliver products in the manufacturer’s labeled packages.

B. Storage and Protection: Store and handle products following manufacturer’s requirements in a facility where environmental conditions are within recommended limits.

1.07 PROJECT CONDITIONS

A. Environmental Requirements: Comply with environmental requirements and recommendations of the manufacturer for the proper installation of products.

B. Temperature Criteria: Do not install digital video recorder system unless the temperature is between 32°F (0°C) to 104°F (40°C).
1.08 WARRANTY

A. Manufacturer’s Guarantee: two years for parts and labor from the manufacture date code under normal use and service for the encoder system.

END OF SECTION

PART  2    PRODUCTS

2.01 EQUIPMENT
A. Manufacturer: 
Digital Watchdog, Inc.




16220 Bloomfield Avenue. Cerritos,

California USA 90703 USA




Phone: (866) 446-3595




Web: www.digital-watchdog.com




E-mail:  dw-tech@dwcc.tv
B. Models: DW-EN4K16 
C. Alternates: None

2.02 SYSTEM COMPONENTS

A. Specified Product: Universal HD over Coax® series 4K analog to IP encoder system. 
2.02 OPERATIONAL REQUIREMENTS

A. The analog to IP encoder (encoder) shall meet or exceed the following design and performance specifications:

1. The encoder shall provide 16 analog inputs for analog encoding.
2. The encoder shall support the following analog signals: 
a. Analog coaxial signal up to 960H resolution

b. HD-Analog signal up to 4K
c. HD-TVI signal up to 4K
3. The encoder shall provide the following audio connections: 

a. 4 RCA audio input channels. 
b. 1 RCA audio output channel.

c. 1 HD audio output channel.
4. The encoder shall provide the following local outputs: 
a. True HD: 4K, 1920x1080, 1280x720, 1024x768
b. VGA: 4K, 1920x1080, 1280x720, 1024x768

c. Spot: 1.0 Vp-p (75 ohm, Composite).

5. The encoder shall use H.265 and H.264 image compression.

6. The encoder shall offer the following streaming resolutions:
a. Main stream: 4K @ 112fps 
b. Main stream: 5MP @ 192fps 
c. Main stream: 4MP @ 240fps 
d. Main stream: 2.1MP/1080p @ 480fps 
e. Sub-stream: VGA @ 480fps
7. The encoder shall support operation using the encoder’s web interface.

8. The encoder front panel shall include LED status indicators for power, hard disk drive activity, and network activity.

9. The encoder rear panel shall include the following items:

a. BNC video inputs

b. Four (4) audio inputs and one (1) audio output.

c. Two (2) RS-485 PTZ control interface.
d. Four (4) alarm inputs and one (1) relay outputs, NO/NC.

e. VGA monitor output.

f. True HD monitor output
g. Digital spot monitor output
h. Low voltage DC power supply jack.
10. The encoder shall be preconfigured with a DHCP-enabled IP address and subnet mask for quick integration within existing IT structures.

11. The encoder shall include a bandwidth throttle to ensure that images and system messages are delivered as quickly as possible within network bandwidth limits.

12. The encoder shall include playback controls via the encoder’s web interface that allows the user to playback recorded video forward or backward at multiple speeds.

13. The encoder shall support multicast streaming to deliver source traffic to multiple receivers using the least amount of network bandwidth.  
14. The encoder shall provide PTZ camera control—including multiple pan, tilt, zoom, and focus speeds, iris control (including return to auto-focus), programming presets, and viewing presets—through the RS-485 port or Up the Coax (UTC) protocol.

15. The encoder shall support alarm sensor in and relay out functions, motion detection, and video loss detection, and shall include alarm monitoring software.  

16. The encoder shall include alarm notifications to an e-mail with an attached JPEG image. 
a. The notifications shall include: 

1.) Motion detection

2.) Sensor and alarm activation

3.) Video loss

4.) System start

5.) User login

17. The encoder shall include a system log that records and displays information relating to alarm events, reboots, and other system information. The user shall have the ability to export the log information.

18. The encoder shall adjust for Daylight Saving Time changes, with no loss of video when the clock advances forward one hour. When the clock is adjusted backward when Daylight Saving Time ends, the encoder shall record both hours, allowing the user to select which hour to playback.

19. The encoder shall include a user management console that allows the administrator to create, edit, and delete user accounts.

20. The encoder shall provide remote operation and configuration through remote viewing software, a web client, and mobile device applications (Apple and Android).

21. The encoder shall allow the users to configure, upgrade and view the following information: 

a. View live video

b. View logs of the system

c. Configure the system’s settings, including:

1.) Network settings

2.) Local display settings
3.) Users settings

4.) Date and time settings

5.) Text overlay and camera titles
6.) Two-way audio and alarm settings 

d. Configure camera settings and user settings

e. Control PTZ cameras

f. View the system’s information. 

B. Connections: 

1. I/O ports: 

a. 4-audio input

b. 1-audio output

c. 4-alarm inputs
d. 1-alarm output
e. 2-RS485 inputs for PTZ camera control
2.03 SYSTEM HARDWARE

A. The digital video recorder shall have the following mechanical specifications: 

1. Unit dimensions: 
14.76" x 1.76" x 12.52" (375 x 44.9 x 318.2 mm)

2. Unit weight:

5.5 lbs (2.5 kg)
3. Construction:

a. Housing: Steel chassis.

b. Finish: Black matte finish.

B. The digital video recorder shall have the following electrical specifications:

1. Voltage: 12 V DC, 2A.

2. Power Consumption: Max 15.6W
C. The encoder shall be designed to meet the following environmental conditions:

1. Operating Less than 90%, non-condensing.

2.04 OTHER CERTIFICATIONS

A. The encoder shall include the following certifications: 

1. FCC

2.05 MANUFACTURER SUPPORT

A. The manufacturer shall provide customer service, pre-sales application assistance, after-sales technical assistance, access to online technical support, and online training using Web conferencing. 

B. The manufacturer shall provide technical assistance and support through a toll-free telephone number at no extra charge Monday thru Friday, 8:00 AM to 8:00 PM EST.

END OF SECTION

PART  3    EXECUTION

3.02 INSTALLERS
A. Contractor personnel shall comply with all applicable state and local licensing requirements.

3.03 PREPARATION

A. The network design and configuration shall be verified for compatibility and performance with the camera(s). 

B. Network configuration shall be tested and qualified by the Contractor before camera installation.

3.04 INSTALLATION

A. The contractor shall follow all Manufacturer published installation procedures and guidelines.

B. Before permanent installation of the system, the system shall be factory tested in conditions simulating the final installed environment

1. A report indicating successful test results shall be produced.

3.05 STORAGE

A. The 4K analog to IP encoder system shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.
END OF SECTION
DW-EN4K16

Compressor™ 16-channel 4K analog to IP video encoder
May 2020

28 23 29 - 7

