GUIDE SPECIFICATION

Section 28 23 29

[image: image1.jpg]LW





Digital Watchdog is a leading manufacturer of security and surveillance solutions, offering stunning image quality, advanced hardware capabilities, reliable customer support and lowest total cost of deployment to the analog & IP megapixel surveillance markets. Located in Tampa, FL with manufacturing facilities in Seoul, Korea, Digital Watchdog is committed to delivering powerful security solutions to its customers worldwide.
For additional information, contact: 

Digital Watchdog, Inc.


16220 Bloomfield Avenue. Cerritos,

California USA 90703 USA


Phone: (866) 446-3595


Web: www.digital-watchdog.com


E-mail:  dw-tech@dwcc.tv
MEGApix® 3MP 36X PTZ IP CAMERA
DIVISION 28 – ELECTRONIC SAFETY AND SECURITY
2820 00 
Electronic Surveillance
28 23 00 
Video Surveillance

28 23 29
Video Surveillance Remote Devices and Sensors

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.
2.  Explanatory notes and comments are presented in colored text.

MEGApix® 3MP 36X PTZ IP CAMERA
PART 1 GENERAL

1.01 SUMMARY
A. The section includes a 3MP high definition IP Pan, Tilt, Zoom (PTZ) video camera with vandal-resistant IP-66 rated, IK10 impact-resistant high-speed PTZ housing.
B. Product - A high definition IP PTZ video camera, based on H.265, H.264 and MJPEG compression, capable of quad streaming 30fps at resolutions up to 2065 x 1565, with color in near-total darkness technology and a 4.6 ~ 165.6mm motorized zoom lens and built-in IR illuminator in an indoor/outdoor high-speed PTZ housing.
C. Related Requirements

1. 27 20 00 Data Communications 

2. 28 23 13 Video Surveillance Control and Management Systems

3. 28 23 16 Video Surveillance Monitoring and Supervisory Interfaces

4. 28 23 19 Digital Video Recorders and Analog Recording Devices

5. 28 23 23 Video Surveillance Systems Infrastructure
1.02 REFERENCES
A. Abbreviations
1. AGC - Automatic Gain Control
2. ARP – Address Resolution Protocol
3. AWB  - Automatic White Balance
4. BLC – Backlight compression

5. DHCP - Dynamic Host Configuration Protocol
6. DNR – Digital Noise Reduction
7. DNS - Domain Name Server
8. DDNS –Dynamic Domain Name Server
9. fps - frames per second
10. FTP - File Transfer Protocol
11. GUI – Graphical User Interface
12. HLC – Highlight Compensation
13. HTTP - Hypertext Transfer Protocol
14. ICMP – Internet Control Message Protocol
15. IGMP - Internet Group Management Protocol
16. IP - Internet Protocol
17. JPEG - Joint Photographic Experts Group
18. MJPEG - Motion JPEG
19. MPEG - Moving Pictures Experts Group
20. NTP - Network Time Protocol
21. PoE - Power over Ethernet
22. PPPoE - Point-to-Point Protocol over Ethernet
23. PTZ – Pan, Tilt, Zoom
24. QoS – Quality of Service
25. RARP – Reverse Address Resolution Protocol
26. RTP - Real-Time Transport Protocol
27. RTSP - Real-Time Streaming Protocol
28. SMTP - Simple Mail Transfer Protocol
29. SNMP – Simple Network Management Protocol
30. TCP - Transmission Control Protocol
31. UDP - User Datagram Protocol
32. UPnP – Universal Plug and Play
33. VMS - Video Management System
34. WDR – Wide Dynamic Range
B. Reference Standards
1. Network

a. IEEE - 802.3 Ethernet Standards
2. Video

a. ISO / IEC 14496 – MPEG-4

1) ISO / IEC 14496–10, MPEG-4 Part 10 ( ITU H.264)

b. ISO / IEC 10918 – JPEG 
c. ONVIF–Profile S
3. Emissions
a. FCC-47 CFR Part 15 Class B
4. Environmental

a. ANSI / IEC60529 – Degrees of Protection Provided by Enclosures

b. International Electrotechnical Commission (IEC) – Ingress Protection Rating IP66

c. European standard EN 62262 — equivalent to International Electrotechnical Commission (IEC) – Impact Protection Rating IK10.
1.03 SUBMITTALS
A. Product Data
1. Manufacturer’s printed or electronic data sheets
2. Manufacturer’s installation and operation manuals
3. Warranty documentation

1.04 QUALIFICATIONS
A. The manufacturer shall have a minimum of two years’ experience in producing IP video equipment.
B. Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.
1.05 DELIVERY, STORAGE, AND HANDLING

A. Deliver the camera in the manufacturer’s original, unopened, and undamaged container with identification labels intact.

B. Store the camera in a temperature environment of -22°F ~ 131°F (-30°C ~ 55°C ), protected from mechanical and environmental conditions as designated by the manufacturer.

1.06 WARRANTY AND SUPPORT

A. The manufacturer shall provide a limited 5-year warranty for the product to be free of defects in material and workmanship.

END OF SECTION
PART 2 PRODUCTS

1.01 EQUIPMENT

A. Manufacturer: 
Digital Watchdog, Inc.




16220 Bloomfield Avenue. Cerritos,

California USA 90703 USA




Phone: (866) 446-3595




Web: www.digital-watchdog.com




E-mail:  dw-tech@dwcc.tv
B. Model

DWC-MPTZ336XW
C. Alternates: None

2.04 GENERAL DESCRIPTION

A. The high definition IP PTZ Dome Camera (“PTZ dome camera”) shall provide video performance capable of providing selectable resolutions up to 2065 x 1565 pixels at 30 frames per second (fps) with color in near-total darkness technology, contained within an IP-66 rated, IK10 impact-resistant high-speed PTZ dome housing.
B. The PTZ dome camera shall possess the following characteristics:
1. H.265, H.264 and MJPEG compression

2. 3MP
3. Star-Light Plus™ color in near-total darkness technology

4. Up to four (4) independent IP video streams (quad streaming)

5. day/night operation with IR cut filter
6. 36 x optical zoom
7. Integral IR LED illuminator providing 1148-foot distance

8. 16 privacy masking regions
9. integral motion detection

10. True Wide Dynamic Range
11. 3D digital noise reduction
12. two-way audio

13. alarm input and output

14. PoE capable
15. multicast or unicast capable

16. local storage via Micro SD, SDXC or SDHC card
17. Dynamic DNS (DDNS) support

18. IP-66 rated 

19. IK-10 impact resistant
2.05 VIDEO
A. Imager
1. Sensor:
1/2.8" CMOS
2. Minimum illumination

a. Color mode: 

0.35 lux 
b. Black & white mode:
0.0 lux 
3. Scanning - Progressive
4. Image Control Settings shall be available for:
a. Automatic white balance (AWB)
b. Exposure modes: 

automatic or manual
1) In manual exposure mode, configurable settings for:

a) brightness

b) shutter mode: 
automatic 1/10000 
c) digital slow shutter up to 8x to decrease the shutter speed in low light
d) Automatic Gain Control (AGC)
e) Backlight compensation (BLC)
f) WDR Level:

100 dB minimum
c. Day and night settings, to allow configuration for Day (color), Night (Black and White), or Automatic.

d. Frequency

e. Image sharpness

f. Chroma

g. Image mirror or flip

h. Digital noise reduction
i. Privacy zone definition, up to 16 zones
5. Lens:


a. Lens type: 4.6 ~ 165.5 mm, Motorized Zoom PTZ Lens
b. Optical zoom: 36x

c. Field of view (FOV): 58.1° ~ 1.9°
d. IR Distance: 1148ft range

e. Double shutter Wide Dynamic Range, 100dB
f.  Shutter speed: 
1/10000 
B. Video Streams
1. The PTZ dome camera shall support two configurable video streams, each of which may have the following properties:
a. Resolution:

1) 2048x1536

2) 1920x1080

3) 1440x1080

4) 1280x1024/720

5) 1024x768

6) 800x600/480
7) D1
8) 640x480
9) 400x240
10) CIF
b. Compression type:
1) H.265 (main profile)
2) H.264 (baseline, main, high profile)
3) MJPEG
c. Bit rate:
1.8 Mbps – 8 Mbps
d. Frame rate:
0 – 30 fps
C. Motion Detection – The PTZ dome camera shall have the ability to detect motion within user-defined areas of the video image.
1. Configuration settings shall be available for sensitivity and dwell time.
D. Storage and Recording
1. The PTZ dome control shall have onboard SD card storage.
a. Card Type:
Micro SD, SDHC and SDXC Class 10
2. Local recording on the SD card shall commence upon loss of network connectivity, based on a pre-programmed schedule.
3. The local SD storage shall have the ability to be backed up to alternate media without the removal of the SD card from the camera.
E. ONVIF – Video streams shall be capable of supporting ONVIF protocol, profile S.
F. Pan Tilt 

1. Pan Range:
360o
2. Tilt Range:
-10 ~ 190o
3. Presets:
256
4. Tours:

up to 8

5. Speed:

380o per second

G. Other
1. Single Images - The PTZ dome camera shall support jpg file image screenshot and export.
2. Discovery - The manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.
3. Emergency alarm – The PTZ dome camera shall allow the sending of a video clip of configurable duration to up to five server locations.
4. Access- The PTZ dome camera shall permit up to five users to simultaneously access the camera.
5. Alarm/Sensors – The PTZ dome camera shall have three alarm/sensor inputs and one alarm/control output.
2.06 AUDIO

A. The PTZ dome camera shall provide one line-level input and one line-level output.

B. Compression and Sampling Rate

1. G.711 (8 kHz)

2.07 NETWORK
A. Connectivity: 10/100 BASE-TX Ethernet with RJ-45 connector
B. Protocols supported

1. Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)
2. Configuration: Dynamic Host Configuration Protocol (DHCP)
3. Web services: Hypertext Transfer Protocol (HTTP)
4. Network services: Universal Plug and Play (UPnP)
5. Media: Real-Time Transport Protocol (RTP), Real-Time Streaming Protocol (RTSP)
6. Notifications: File Transfer Protocol (FTP)
C. Flow Control- The PTZ dome camera shall support TCP and Layer 3 QoS protocols to control network congestion.
D. DDNS – The PTZ dome camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.
E. Security

1. The PTZ dome camera shall support IP address filtering whereby users can enter a list of allowed or blocked IP addresses for viewing video and configuring camera settings
2. The PTZ dome camera shall provide three levels of user access with password protection.
2.08 ADDITIONAL FEATURES

A. Auxiliary Inputs and Outputs
1. Input: Alarm sensor (contact closure)
2. Output: Relay
B. System Information
1. The system settings of the PTZ dome camera shall be exportable as a separate file.
2. The PTZ dome camera shall maintain an access log of the system and motion-triggered events.

a. The log shall be exportable to an Excel spreadsheet file.
2.09 CAMERA SOFTWARE
A. The PTZ dome camera shall have a built-in web server that supports browser-based configuration using Internet Explorer, Google Chrome, Mozilla Firefox, and Apple Safari.
B. The software GUI shall allow access to camera information and all primary software functions, including:
1. Camera network address

2. Configuration

3. Stream control

4. Screen snapshot, print and export

5. Start and stop recording
C. The Manufacturer shall offer a video viewer and configuration to implement the following actions:
1. Camera discovery
2. Configuration 
a. video parameters
b. events and notifications

1) motion-related

2) emergency

c. camera network parameters

d. SD card storage recording management 

e. image capture, export, and print
3. Viewer - view video streams through the web browser
4. Image print and export
5. Instant record and playback
6. Alerts
a. e-mail setup
b. define web addresses for notifications
7. System
a. firmware upgrade
b. reset to factory default
c. set date, time, and NTP server synchronization
d. user access control
e. view and export camera settings
f. view system logs
2.010 ELECTRICAL

A. Power

1. Sources

a. 12 VDC

b. PoE (UPoE, Class 4)
2. Power Consumption:


a. PoE: 28W, 500mA

b. DC 12V: 28W, 2.3A
3. Ethernet: 


RJ-45 connector
4. External power (12 VDC):
2-conductor power to the terminal block
5. Audio:



individual 2 wire connections for in and out

6. Alarm In and Out:

up to 8 wire connection to the terminal block
2.011 MECHANICAL AND ENVIRONMENTAL

A. Housing Material: 

aluminum housing, polycarbonate dome
B. Configuration:

PTZ dome
C. Dimensions (D x H): 
7.94 in. x 12.51 in. (201.8mm x 31.78 mm)
D. Weight: 


11.24 lbs (5.1 Kg)

E. Temperature:

1. Operating:

-22°F ~ 131°F (-30°C ~ 55°C )
F. Humidity:


0 - 90%, non-condensing
G. Environmental Rating: 
IP-66
H. IK Rating: 


IK10

END OF SECTION
PART 3 EXECUTION

3.01 INSTALLERS
A. Contractor personnel shall comply with all applicable state and local licensing requirements.
3.02 PREPARATION

A. The network design and configuration shall be verified for compatibility and performance with the camera(s). Network configuration shall be tested and qualified by the Contractor before camera installation.
3.03 INSTALLATION
A. Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment

1. A report indicating successful test results shall be produced.
3.04 STORAGE

A. The PTZ dome camera hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.
END OF SECTION

DWC-MPTZ336XW

MEGApix® 3MP 36X PTZ IP CAMERA
March 2020

28 23 29 - 11

