GUIDE SPECIFICATION

Section 28 23 29

[image: image1.jpg]LW





Digital Watchdog is a leading manufacturer of security and surveillance solutions, offering stunning image quality, advanced hardware capabilities, reliable customer support and lowest total cost of deployment to the analog & IP megapixel surveillance markets. Located in Tampa, FL with manufacturing facilities in Seoul, Korea, Digital Watchdog is committed to delivering powerful security solutions to its customers worldwide.
For additional information, contact: 

Digital Watchdog, Inc.


16220 Bloomfield Avenue. Cerritos,

California USA 90703 USA


Phone: (866) 446-3595


Web: www.digital-watchdog.com


E-mail:  dw-tech@digital-watchdog.com
STAR-LIGHT PLUS™ 2.1MP/1080P UNIVERSAL HD OVER COAX® 20X PTZ CAMERA
DIVISION 28 – ELECTRONIC SAFETY AND SECURITY
2820 00 
Electronic Surveillance
28 23 00 
Video Surveillance

28 23 29
Video Surveillance Remote Devices and Sensors

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.
2.  Explanatory notes and comments are presented in colored text.

STAR-LIGHT PLUS™ 2.1MP/1080P UNIVERSAL HD OVER COAX® 20X PTZ CAMERA
PART 1    GENERAL

1.01 SUMMARY
A. The section includes at 2.1MP/1080p Universal HD over Coax® analog high definition IP Pan, Tilt, Zoom (PTZ) camera with vandal-resistant IP-66 rated, IK10 impact-resistant high-speed PTZ housing.
B. Product - A high definition PTZ video camera, capable of transmitting 30fps at up to 1945 x 1097 video with color in near-total darkness technology over analog connections, in a vandal-resistant IP66 environmental-rated water and dust-resistant bullet housing.
C. Related Requirements

1. 27 20 00 Data Communications 

2. 28 23 13 Video Surveillance Control and Management Systems

3. 28 23 16 Video Surveillance Monitoring and Supervisory Interfaces

4. 28 23 19 Digital Video Recorders and Analog Recording Devices

5. 28 23 23 Video Surveillance Systems Infrastructure
1.02 REFERENCES
A. Abbreviations
1. AGC - Automatic Gain Control
2. ARP – Address Resolution Protocol
3. AWB  - Automatic White Balance
4. BLC – Backlight compression

5. DHCP - Dynamic Host Configuration Protocol
6. DNR – Digital Noise Reduction
7. DNS - Domain Name Server
8. DDNS –Dynamic Domain Name Server
9. fps - frames per second
10. FTP - File Transfer Protocol
11. GUI – Graphical User Interface
12. HLC – Highlight Compensation
13. HTTP - Hypertext Transfer Protocol
14. ICMP – Internet Control Message Protocol
15. IGMP - Internet Group Management Protocol
16. IP - Internet Protocol
17. JPEG - Joint Photographic Experts Group
18. MJPEG - Motion JPEG
19. MPEG - Moving Pictures Experts Group
20. PTZ – pan, tilt, zoom
21. QoS – Quality of Service
22. RARP – Reverse Address Resolution Protocol
23. RTP - Real-Time Transport Protocol
24. RTSP - Real-Time Streaming Protocol
25. TCP - Transmission Control Protocol
26. UDP - User Datagram Protocol
27. UPnP – Universal Plug and Play
28. WDR – Wide Dynamic Range
B. Reference Standards
1. Network

a. IEEE - 802.3 Ethernet Standards
2. Video

a. ISO / IEC 14496 – MPEG-4

1) ISO / IEC 14496–10, MPEG-4 Part 10 ( ITU H.264)

b. ISO / IEC 10918 – JPEG 
c. ONVIF–Profile S
3. Emissions
a. FCC-47 CFR Part 15 Class B
4. Environmental

a. ANSI / IEC60529 – Degrees of Protection Provided by Enclosures

b. International Electrotechnical Commission (IEC) – Ingress Protection Rating IP66

c. European standard EN 62262 — equivalent to International Electrotechnical Commission (IEC) – Impact Protection Rating IK10

1.03 SUBMITTALS
A. Product Data
1. Manufacturer’s printed or electronic data sheets
2. Manufacturer’s installation and operation manuals
3. Warranty documentation

1.04 QUALIFICATIONS
A. The manufacturer shall have a minimum of two years experience in producing IP video equipment.
B. Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.
1.05 DELIVERY, STORAGE AND HANDLING

A. Deliver the camera in the manufacturer’s original, unopened and undamaged container with identification labels intact.

B. Store the camera in a temperature environment of -4°F to 122°F (-20°C to 50°C), protected from mechanical and environmental conditions as designated by the manufacturer.

END OF SECTION

PART 2    PRODUCTS

1.01 EQUIPMENT

A. Manufacturer: 
Digital Watchdog, Inc.




16220 Bloomfield Avenue. Cerritos,

California USA 90703 USA




Phone: (866) 446-3595




Web: www.digital-watchdog.com




E-mail:  dw-tech@digital-watchdog.com
B. Model

DWC-PTZ220XW
C. Alternates: None

1.02 GENERAL DESCRIPTION

D. The high definition IP PTZ Dome Camera (“PTZ dome camera”) shall provide video performance capable of capturing and transmitting 2.1MP/1080p video at 30fps over an analog connection, to allow users to integrate it with a digital video recorder or digital video encoder products. The camera shall include color in near-total darkness technology contained within an IP-66 rated, IK10 impact-resistant high-speed PTZ dome housing.
E. The PTZ dome camera shall possess the following characteristics:
1. 2.1MP/1080p resolution
2. Star-Light Plus™ color in near-total darkness technology

3. Day/night operation with IR cut filter
4. 16 x digital zoom, 20 x optical zoom
5. Integral IR LED illuminator providing 1148-foot distance

6. 16 privacy masking regions

7. Integral motion detection

8. True Wide Dynamic Range
9. 3D digital noise reduction
10. 4 alarm inputs and 1 alarm output

11. Multicast or unicast capable
12. IP-66 rated 

13. IK-10 impact resistant
1.03 VIDEO
F. Connection
1. The PTZ dome camera shall include the following connectors:

a. Video Output: 1 BNC (HD-Analog, HD-TVI, CVBS selectable) 

1) HD-TVI: 2.1MP/1080p at 30fps,720p at 30/60fps

2) HD-Analog: 2.1MP/1080p at 30fps

3) CVBS: 720x480 at 30fps
b. Bit rate:
1.8 Mbps – 8 Mbps
c. Frame rate:
0 – 30 fps
G. Imager
1. Sensor:
1/2.8" CMOS
2. Minimum illumination

a. Color mode: 

0.35 lux (50IRE)

b. Black & white mode:
0.013 lux (50IRE)
3. Scanning - Progressive
4. Image Control Settings shall be available for:
a. Automatic white balance (AWB)
b. Exposure modes: 

automatic or manual
1) In manual exposure mode, configurable settings for:

a) brightness

b) shutter mode: 
automatic 1/30000 at 30fps, manual

    





automatic 1/50000 at 60fps, manual 

c) digital slow shutter up to 8x to decrease the shutter speed in low light

d) Backlight compensation (BLC)

e) Automatic Gain Control (AGC)

f) WDR Level:

100 dB minimum
c. Day and night settings, to allow configuration for Day (color), Night (Black and White), or Automatic.

d. Frequency

e. Image sharpness

f. Chroma

g. Image mirror or flip

h. Digital noise reduction
i. Privacy zone definition, up to 16 zones
5. Lens:
a. Lens type: 4.7 ~ 94mm, Remote Auto Focus Lens
b. Optical zoom: 20x
c. Digital zoom: 16x
d. Field of view (FOV): 55.5o ~ 3o
e. IR Distance: 1148ft
f. Double shutter Wide Dynamic Range
g.  Shutter speed: 
1/30000 at 30fps
1/50000 at 60fps
H. Motion Detection – The PTZ dome camera shall have the ability to detect motion within user-defined areas of the video image.
1. Configuration settings shall be available for sensitivity and dwell time.
2.04 PROTOCOL

A. Remote control interface: Coaxial control (UTC)

B. Protocols: COAX: NTSC / PAL

2.05 PTZ 

1. Pan Range:
360o
2. Tilt Range:
-10 ~ 190o
3. Presets:
240
4. Tours:

up to 8

a. entries per tour:
20

5. Patterns: 
up to 8

6. Home function: 
Supported

7. Speed:

380o per second

8. Protocol:
Pelco-P, Pelco-D
B. Other
1. Alarm/Sensors – The PTZ dome camera shall have four alarm/sensor inputs and one alarm/control output.
2.06 ADDITIONAL FEATURES

A. Auxiliary Inputs and Outputs
1. Input: 4 Alarm sensors (contact closure)
2. Output: 1 Relay output
2.07 ELECTRICAL

A. Power

1. Sources

a. 24VAC, 12VDC
2. Power Consumption:


a. Max, 1A (12W)
3. External power (12 VDC):
2-conductor power to terminal block
4. Alarm In and Out:

up to 6 wire connection to terminal block
2.08 MECHANICAL AND ENVIRONMENTAL

A. Housing Material: 

aluminum housing, polycarbonate dome
B. Configuration:

PTZ dome
C. Dimensions (D x H): 
6.06 in. x 5.9 in. (154mm x 150 mm)
D. Temperature:

E. Operating:

-10° C to 50° C (14° F to 122° F)
F. Humidity:


0 - 90%, non-condensing
G. Environmental Rating: 
IP-66
H. IK Rating: 


IK10

END OF SECTION
PART 3    EXECUTION

3.01 INSTALLERS
A. Contractor personnel shall comply with all applicable state and local licensing requirements.
3.02 PREPARATION

A. The network design and configuration shall be verified for compatibility and performance with the camera(s). Network configuration shall be tested and qualified by the Contractor before camera installation.
3.03 INSTALLATION
A. Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment

1. A report indicating successful test results shall be produced.
3.04 STORAGE

A. The PTZ dome camera hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.
END OF SECTION

DWC-PTZ220XW

STAR-LIGHT PLUS™ UHDoC® 20X PTZ CAMERA
April 2021

28 23 29 - 1

