GUIDE SPECIFICATION

Section 28 23 29

[image: image1.jpg]LW




Digital Watchdog is a leading manufacturer of security and surveillance solutions, offering stunning image quality, advanced hardware capabilities, reliable customer support and lowest total cost of deployment to the analog & IP megapixel surveillance markets. Located in Tampa, FL with manufacturing facilities in Seoul, Korea, Digital Watchdog is committed to delivering powerful security solutions to its customers worldwide.
For additional information, contact: 

DW, Inc., Inc.

16220 Bloomfield Avenue, 

Cerritos, California 90703 USA

Phone: +1 888 446-3593

Web: www.digital-watchdog.com

E-mail: dw-tech@dwcc.tv
HIGH DEFINITION ANALOG DOME CAMERA
DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 
Electronic Surveillance
28 23 00 
Video Surveillance


28 23 29
Video Surveillance Remote Devices and Sensors

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.
2.  Explanatory notes and comments are presented in colored text.

3. CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

4. MasterFormat 2014 Specification Category: 

28 23 29 - Video Surveillance Remote Devices and Sensors

HIGH DEFINITION DOME CAMERA

PART 1   GENERAL

1.01 SUMMARY

A.  Section includes a 1080p high definition indoor dome camera. 
B.  Product - A high definition indoor dome video camera, capable of transmitting 1944 x 1092 video over analog connections.
C. Related Requirements
1. 27 20 00 Data Communications 

2. 28 23 13 Video Surveillance Control and Management Systems

3. 28 23 16 Video Surveillance Monitoring and Supervisory Interfaces

4. 28 23 19 Digital Video Recorders and Analog Recording Devices

5. 28 23 23 Video Surveillance Systems Infrastructure

A. REFERENCES
B. Abbreviations
1. AGC – Automatic Gain Control
2. ARP – Address Resolution Protocol

3. AWB – Automatic White Balance
4. BLC – Back light compression

5. DNR – Digital Noise Reduction
6. FPS – frames per second
7. HLC – Highlight Compensation
8. VMS – Video Management System
9. WDR – Wide Dynamic Range
C. Reference Standards
1. Emissions
a. FCC-47 CFR Part 15 Class B
1.02 SUBMITTALS
A. Product Data
1. Manufacturer’s printed or electronic data sheets
2. Manufacturer’s installation and operation manuals
3. Warranty documentation

1.03 QUALIFICATIONS
A. Manufacturer shall have a minimum of five years’ experience in producing IP video equipment.
B. Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.
1.04 DELIVERY, STORAGE, AND HANDLING

A. Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

B. Store the camera in a temperature environment of -40° F to 122° F (-40° C to 50° C), protected from mechanical and environmental conditions as designated by the manufacturer.

1.05 WARRANTY AND SUPPORT
A. Manufacturer shall provide a limited 5-year warranty for the product to be free of defects in material and workmanship.
END OF SECTION

PART  2    PRODUCTS

2.01 EQUIPMENT
A. Manufacturer: 
Digital Watchdog, Inc.




16220 Bloomfield Avenue. Cerritos,

California USA 90703 USA




Phone: (866) 446-3595




Web: www.digital-watchdog.com




E-mail:  dw-tech@dwcc.tv
B. Models DWC-D4283WD, DWC-D4283WTIR Series

Digital Watchdog model differences:


DWC-D4283WD
Electronic Day/Night
DWC- D4283WTIR
IR Illumination, Smart IR™
C. Alternates: None

2.02 GENERAL DESCRIPTION

A. The indoor dome camera system shall capture and transmit 1080p, 2.1MP video over an analog connection, for the purpose of allowing users to integrate it with digital video recorder or digital video encoder products.

B. The indoor dome camera system shall include IR illumination with a fixed or varifocal lens and electronic day/night functions. 

C. The camera shall possess the following characteristics:
1. 2.1 megapixels

2. day/night operation with IR cut filter

3. 4.2x optical zoom

4. Motorized zoom

5. option for Integral IR LED illuminator providing 100-foot distance

6. 16 privacy masking zones

7. integral motion detection

8. Double shutter Wide Dynamic Range

9. 3D digital noise reduction
10. Built-in microphone
2.03 VIDEO

A. Connection
1. The indoor dome camera shall include the following connectors:

a. Video Output: 1 BNC (HD-Analog, HD-TVI, HD-CVI, CVBS selectable) 

b. Resolution: 1920x 1080, 1280 x 720

c. Maximum Framerate: 30fps at 1080p, 30fps at 720p

B. Imager
1. Sensor:
 1/3" Sony CMOS

a. Pixels per sensor: 1944 (H) x 1092 (V) total
2. Minimum illumination

a. Color mode: 

0.3 lux (F1.6)

b. Black & white mode: 
<0 lux (F1.6)>
Note: Options listed in bold apply to the DWC-D4283WTIR models only.

3. Scanning - Progressive
4. SN Ratio: 52dB

5. Image Control Settings shall be available for:

a. On Screen Display; 

b. Camera Title: 

c. Gain control

d. Automatic white balance (AWB)

e. Exposure modes: 

automatic or manual

1.) In manual exposure mode, configurable settings for:

a.) brightness

b.) shutter mode: 
automatic or manual - 1/30 to 1/30000 

c.) digital slow shutter up to 8x to decrease shutter speed in low light

d.) Backlight compensation (BLC)

e.) Auto gain control (AGC)

f.) WDR level:

100 dB minimum

f. Day and night settings, to allow configuration for Day (color), Night (Black and White), or Automatic.

g. Frequency

h. Image sharpness

i. Chroma

j. Image mirror or flip

k. Digital noise reduction

l. Privacy zone definition, up to 16 zones

6. Lens:
a. Lens type: 2.8~12mm P-Iris Lens, remote auto-focus
b. Field of view (FOV): 91o – 28o
c. Double shutter Wide Dynamic Range
d. Shutter speed:  1/30 sec to 1/30000
C. Video Transmission Distance: 1640.42ft (500m) (75-5 coaxial cable)
2.04 PROTOCOL

A. Remote control interface: Coaxial control (UTC)

B. Protocols: COAX: NTSC / PAL

2.05 AUDIO

A. The indoor dome camera shall provide one-line level input.

B. Audio sensitivity:  -38dB ± 2dB
C. Audio Frequency Response: 50Hz ~ 10KHz
2.06 ADDITIONAL FEATURES

A. Auxiliary Inputs and Outputs
1. Output: Relay
2.07 ELECTRICAL

A. Power

1. Sources

a. Dual 24 VAC & 12 VDC
2. Power Consumption

a. DWC-D4283WD
1.) DC12V:

Max:3.0W
2.) AC24V:

Max:2.3W

b. DWC-D4283WTIR
1.) DC12V


a.) Led off:3.0W


b.) Led on:5.2W
2.) AC24V



a.) Led off:2.3W


b.) Led on:4.6W
B. Connectors

a. Power Connector


2-pin terminal block with screw terminals

b. Video Connector


BNC
c. Audio: 
2.08 MECHANICAL AND ENVIRONMENTAL

A. Housing Material: 

Plastic housing, polycarbonate dome
B. Configuration: 

Dome
C. Dimensions (D x H):  
4.24 in. x 3.9 in. (107.8 mm x 99.1 mm)
D. Temperature:

1. Operating: 

-40° F to 122° F (-40° C to 50° C)  


E. Humidity:


10 - 90% RH, non-condensing


END OF SECTION
PART  3    EXECUTION

3.02 INSTALLERS
A. Contractor personnel shall comply with all applicable state and local licensing requirements.

3.03 PREPARATION

A. The network design and configuration shall be verified for compatibility and performance with the camera(s). 

B. Network configuration shall be tested and qualified by the Contractor prior to camera installation.

3.04 INSTALLATION

A. Contractor shall follow all Manufacturer published installation procedures and guidelines.

B. Before permanent installation of the system, the system shall be factory tested in conditions simulating the final installed environment

1. A report indicating successful test results shall be produced.
3.05 STORAGE

A. The IP camera hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.
END OF SECTION
SnapIt DWC-D4283WD, DWC-D4283WTIR 
HD OVER COAX DOME CAMERA

October 2018

28 23 29 - 7

