# EKI-K41T44 EKI-K82T46 EKI-K164T412

4K Value Express Kits

## Cameras

- 1/3" (4 MP) Progressive Scan CMOS

- 2560 x 1440 (4 MP) Resolution

- 2.8 mm Fixed Lens

- EXIR 2.0 with 100 ft (30 m) IR Range

- H.265+/H.265/H.264+/H.264/MJPEG

- IP67 Protection

- Durable Metal Housing

- 120 dB WDR for Clear Images

## NVR

- Up to 8 MP Resolution Recording

- H.265+/H.265/H.264+/H.264/MPEG

- Plug-and-Play with 4, 8, or 16 PoE Cameras

- Up to 2 SATA Interfaces

Hikvision EKI-KxxxT4xx Series 4K Value Express Kits are cost-effective solutions for value-oriented users.

Each kit is supplied with four, six, or twelve network cameras with 4 MP image sensors and a 4-, 8-, or 16-channel NVR in the same box.

Kits are plug-n-play and can be set up quickly.

Available models:

EKI-K41T44: (1) ERI-K104-P4 NVR + 1TB HDD + (4) ECI-T24F2 Cameras

EKI-K82T46: (1) ERI-K108-P8 NVR + 2TB HDD + (6) ECI-T24F2 Cameras

EKI-K164T412: (1) ERI-K216-P16 NVR + 4TB HDD + (12) ECI-T24F2 Cameras

- Specification - for ECI-T24F2(C)

| Camera |  |
| --- | --- |
| Image Sensor | 1/3" progressive scan CMOS |
| Max. Resolution | 2560 × 1440 |
| Min. Illumination | Color: 0.01 Lux @ (F2.0, AGC ON), B/W: 0 Lux with IR |
| Shutter Time | 1/3 s to 1/100,000 s |
| Day ＆ Night | IR cut filter |
| Angle Adjustment | Pan: 0° to 360°, tilt: 0° to 75°, rotate: 0° to 360° |
| Lens |  |
| Lens Type | Fixed focal length, 2.8 and 4 mm optional |
| Focal Length ＆ FOV | 2.8 mm, horizontal FOV 98.0°, vertical FOV 53.1°, diagonal FOV 114.7° 4 mm, horizontal FOV 78.7°, vertical FOV 43.1°, diagonal FOV 92.3° |
| Lens Mount | M12 |
| Iris Type | Fixed |
| Aperture | F2.0 |
| Illuminator |  |
| Supplement Light Type | IR |
| Supplement Light Range | Up to 30 m |
| Smart Supplement Light | Yes |
| IR Wavelength | 850 nm |
| Video |  |
| Main Stream | 50 Hz: 20 fps (2560 × 1440), 25 fps (2304 × 1296, 1920 × 1080, 1280 × 720) 60 Hz: 20 fps (2560 × 1440), 30 fps (2304 × 1296, 1920 × 1080, 1280 × 720) |
| Sub-Stream | 50 Hz: 25 fps (640 × 480, 640 × 360) 60 Hz: 30 fps (640 × 480, 640 × 360) |
| Video Compression | Main stream: H.265/H.264/H.264+/H.265+ Sub-stream: H.265/H.264/MJPEG |
| Video Bit Rate | 32 Kbps to 8 Mbps |
| H.264 Type | Baseline Profile/Main Profile/High Profile |
| H.265 Type | Main Profile |
| Region of Interest (ROI) | 1 fixed region for main stream |
| Audio |  |
| Audio Compression | G.711ulaw/G.711alaw/G.722.1/G.726/MP2L2/PCM/AAC |
| Audio Bit Rate | 64 Kbps (G.711)/16 Kbps (G.722.1)/16 Kbps (G.726)/32 to 160 Kbps (MP2L2)/16 to 64 Kbps (AAC) |
| Audio Sampling Rate | 8 kHz/16 kHz |
| Environment Noise Filtering | Yes |
| Network |  |
| Protocols | TCP/IP, ICMP, HTTP, HTTPS, FTP, DHCP, DNS, DDNS, RTP, RTSP, RTCP, PPPoE, NTP, UPnP™, SMTP, SNMP, IGMP, 802.1X, QoS, IPv6, Bonjour, IPv4, UDP, SSL/TLS |
| Simultaneous Live View | Up to 6 channels |
| API | Open Network Video Interface, ISAPI |
| User/Host | Up to 32 users. 3 levels: administrator, operator and user |
| Network Storage | NAS (NFS, SMB/CIFS), ANR |

| Client | iVMS-4200, Hik-Connect |
| --- | --- |
| Web Browser | Plug-in required live view: IE 10+ Plug-in free live view: Chrome 57.0+, Firefox 52.0+ Local service: Chrome 57.0+, Firefox 52.0+ |
| Image |  |
| Image Settings | Saturation, brightness, contrast, sharpness, AGC, white balance adjustable by client software or web browser |
| Day/Night Switch | Auto, Schedule, Day, Night |
| Wide Dynamic Range (WDR) | 120 dB |
| Image Enhancement | BLC, 3D DNR |
| Interface |  |
| Ethernet Interface | 1 RJ45 10 M/100 M self-adaptive Ethernet port |
| On-Board Storage | Built-in memory card slot, support microSD card, up to 256 GB |
| Built-in Microphone | Yes |
| Reset Key | Yes |
| Event |  |
| Basic Event | Motion detection, video tampering, exception |
| Linkage | Upload to FTP, notify surveillance center, send email, upload to memory card, trigger recording, trigger capture |
| General |  |
| Power | 12 VDC ± 25%, 0.4 A, max. 5 W, Ø5.5 mm coaxial power plug PoE: 802.3af, Class 3, 36 V to 57 V, 0.2 A to 0.15 A, max. 6.5 W |
| Material | Metal ＆ Plastic |
| Dimension | Ø127.3 mm × 95.9 mm (Ø 5.0" × 3.7") |
| Package Dimension | 150 mm × 150 mm × 141 mm (5.9" × 5.9" × 5.6") |
| Weight | Approx. 469 g (1.03 lb.) |
| With Package Weight | Approx. 705 g (1.55 lb.) |
| Storage Conditions | -30 °C to 60 °C (-22 °F to 140 °F). Humidity 95% or less (non-condensing) |
| Startup and Operating Conditions | -30 °C to 60 °C (-22 °F to 140 °F). Humidity 95% or less (non-condensing) |
| General Function | Anti-flicker, heartbeat, mirror, password protection, privacy mask, watermark, IP address filter |
| Approval |  |
| EMC | FCC SDoC (47 CFR Part 15, Subpart B); IC VoC (ICES-003: Issue 6, 2016) |
| Protection | IP67 (IEC 60529-2013) |

- Dimension

Accessory

- Optional

| CPM | CBM | CB130T | PM | PC130T |
| --- | --- | --- | --- | --- |
| WML | ABM | CM |  |  |

Specification

| Model | ERI-K104-P4(C) |
| --- | --- |
| Video and Audio |  |
| IP video input | 4-ch |
|  | Up to 8 MP resolution |
| Incoming bandwidth | 40 Mbps |
| Outgoing bandwidth | 80 Mbps |
| HDMI output | 1-ch, 4K (4096 x 2160)/30 Hz, 4K (3840 x 2160)/30 Hz, 2K (2560 x 1440)/60 Hz, 1920 x 1080/60 Hz, 1600 x 1200/60 Hz, 1280 x 1024/60 Hz, 1280 x 720/60 Hz |
| VGA output | 1-ch, 1920 × 1080/60 Hz, 1280 × 1024/60 Hz, 1280 × 720/60 Hz |
| Video output mode | HDMI/VGA simultaneous output |
| CVBS output | N/A |
| Audio output | 1-ch, RCA (Linear, 1 KΩ) |
| Two-way audio input | 1-ch, RCA (2.0 Vp-p, 1 KΩ, using the audio input) |
| Decoding |  |
| Decoding format | H.265+/H.265/H.264+/H.264 |
| Recording resolution | 8 MP/6 MP/5 MP/4 MP/3 MP/1080p/UXGA/720p/VGA /4CIF/DCIF/2CIF/CIF/QCIF |
| Synchronous playback | 4-ch |
| Capability | 1-ch@8 MP (30 fps)/5-ch@1080p (30 fps) |
| Stream type | Video, Video & Audio |
| Audio compression | G.711ulaw/G.711alaw/G.722/G.726 |
| Network |  |
| Remote connection | 16 |
| Network protocol | TCP/IP, DHCP, IPv4, IPv6, DNS, DDNS, NTP, RTSP, SADP, SMTP, SNMP, NFS, iSCSI, ISUP, UPnP™, HTTP, HTTPS |
| Network interface | 1, RJ-45 10/100 Mbps self-adaptive Ethernet interface |
| PoE |  |
| Interface | 4, RJ-45 10/100 Mbps self-adaptive Ethernet interface |
| Power | ≤ 50 W |
| Supported standard | IEEE 802.3 af/at |
| Auxiliary Interface |  |
| SATA | 1 SATA interface |
| Capacity | Up to 8 TB capacity for each disk |
| Alarm in/out¹ | N/A (4/1 is optional) |
| USB interface | Front panel: 1 × USB 2.0; Rear panel: 1 × USB 2.0 |
| General |  |
| Power supply | 48 VDC, 1.35 A |
| Consumption (without HDD and PoE off) | ≤ 10 W |
| Working temperature | -10 °C to 55 °C (14 °F to 131 °F) |
| Working humidity | 10% to 90% |
| Dimension (W × D × H) | 320 × 240 × 48 mm (12.4" × 9.4" × 1.9") |
| Weight (without HDD) | ≤ 1 kg (2.2 lb) |

Specification

| Model | ERI-K108-P8(C) |
| --- | --- |
| Video and Audio |  |
| IP video input | 8-ch |
|  | Up to 8 MP resolution |
| Incoming bandwidth | 80 Mbps |
| Outgoing bandwidth | 80 Mbps |
| HDMI output | 1-ch, 4K (4096 x 2160)/30 Hz, 4K (3840 x 2160)/30 Hz, 2K (2560 x 1440)/60 Hz, 1920 x 1080/60 Hz, 1600 x 1200/60 Hz, 1280 x 1024/60 Hz, 1280 x 720/60 Hz |
| VGA output | 1-ch, 1920 × 1080/60 Hz, 1280 × 1024/60 Hz, 1280 × 720/60 Hz |
| Video output mode | HDMI/VGA simultaneous output |
| CVBS output | N/A |
| Audio output | 1-ch, RCA (Linear, 1 KΩ) |
| Two-way audio input | 1-ch, RCA (2.0 Vp-p, 1 KΩ, using the audio input) |
| Decoding |  |
| Decoding format | H.265+/H.265/H.264+/H.264 |
| Recording resolution | 8 MP/6 MP/5 MP/4 MP/3 MP/1080p/UXGA/720p/VGA /4CIF/DCIF/2CIF/CIF/QCIF |
| Synchronous playback | 8-ch |
| Capability | 1-ch@8 MP (30 fps)/5-ch@1080p (30 fps) |
| Stream type | Video, Video & Audio |
| Audio compression | G.711ulaw/G.711alaw/G.722/G.726 |
| Network |  |
| Remote connection | 64 |
| Network protocol | TCP/IP, DHCP, IPv4, IPv6, DNS, DDNS, NTP, RTSP, SADP, SMTP, SNMP, NFS, iSCSI, ISUP, UPnP™, HTTP, HTTPS |
| Network interface | 1, RJ-45 10/100 Mbps self-adaptive Ethernet interface |
| PoE |  |
| Interface | 8, RJ-45 10/100 Mbps self-adaptive Ethernet interface |
| Power | ≤ 75 W |
| Supported standard | IEEE 802.3 af/at |
| Auxiliary Interface |  |
| SATA | 1 SATA interface |
| Capacity | Up to 8 TB capacity for each disk |
| USB interface | Front panel: 1 × USB 2.0; Rear panel: 1 × USB 2.0 |
| General |  |
| Power supply | 48 VDC, 1.88 A |
| Consumption (without HDD and PoE off) | ≤ 10 W |
| Working temperature | -10 °C to 55 °C (14 °F to 131 °F) |
| Working humidity | 10% to 90% |
| Dimension (W × D × H) | 320 × 240 × 48 mm (12.4" × 9.4" × 1.9") |
| Weight (without HDD) | ≤ 1 kg (2.2 lb) |

Specification

| Model | ERI-K216-P16(C) |
| --- | --- |
| Video and Audio |  |
| IP video input | 16-ch |
|  | Up to 8 MP resolution |
| Incoming bandwidth | 160 Mbps |
| Outgoing bandwidth | 80 Mbps |
| HDMI output | 1-ch, 4K (4096 x 2160)/30 Hz, 4K (3840 x 2160)/30 Hz, 2K (2560 x 1440)/60 Hz, 1920 x 1080/60 Hz, 1600 x 1200/60 Hz, 1280 x 1024/60 Hz, 1280 x 720/60 Hz |
| VGA output | 1-ch, 1920 × 1080/60 Hz, 1280 × 1024/60 Hz, 1280 × 720/60 Hz |
| Video output mode | HDMI/VGA simultaneous output |
| CVBS output | N/A |
| Audio output | 1-ch, RCA (Linear, 1 KΩ) |
| Two-way audio input | 1-ch, RCA (2.0 Vp-p, 1 KΩ, using the audio input) |
| Decoding |  |
| Decoding format | H.265+/H.265/H.264+/H.264 |
| Recording resolution | 8 MP/6 MP/5 MP/4 MP/3 MP/1080p/UXGA/720p/VGA /4CIF/DCIF/2CIF/CIF/QCIF |
| Synchronous playback | 16-ch |
| Capability | 1-ch@8 MP (30 fps)/5-ch@1080p (30 fps) |
| Stream type | Video, Video & Audio |
| Audio compression | G.711ulaw/G.711alaw/G.722/G.726 |
| Network |  |
| Remote connection | 64 |
| Network protocol | TCP/IP, DHCP, IPv4, IPv6, DNS, DDNS, NTP, RTSP, SADP, SMTP, SNMP, NFS, iSCSI, ISUP, UPnP™, HTTP, HTTPS |
| Network interface | 1, RJ-45 10/100/1000 Mbps self-adaptive Ethernet interface |
| PoE |  |
| Interface | 16, RJ-45 10/100 Mbps self-adaptive Ethernet interface |
| Power | ≤ 150 W |
| Supported standard | IEEE 802.3 af/at |
| Auxiliary Interface |  |
| SATA | 2 SATA interfaces |
| Capacity | Up to 8 TB capacity for each disk |
| USB interface | Front panel: 1 × USB 2.0; Rear panel: 1 × USB 2.0 |
| General |  |
| Power supply | 100 to 240 VAC, 50 to 60 Hz |
| Consumption (without HDD and PoE off) | ≤ 15 W |
| Working temperature | -10 °C to 55 °C (14 °F to 131 °F) |
| Working humidity | 10% to 90% |
| Dimension (W × D × H) | 385 × 315 × 52 mm (15.2"× 12.4" × 2.0") |
| Weight (without HDD) | ≤ 2.6 kg (5.7 lb) |