GUIDE SPECIFICATION

MasterFormat 2014: Section 28 23 29

MasterFormat 2016: Section 28 2x xx

[image: image1.png]WiseNeT





Hanwha Techwin is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://www.hanwha-security.com/ 

2MP ANALOG BOX CAMERA
DIVISION 28 – ELECTRONIC SAFETY AND SECURITY
Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.
2.  Explanatory notes and comments are presented in colored text.

Important:  See further notes on the following page.

Important Note to Security Systems Specifiers

CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

Primary Specification Area:

MasterFormat 2014:

28 20 00 
Electronic Surveillance
28 23 00 
Video Surveillance


28 23 29
Video Surveillance Remote Devices and Sensors

MasterFormat 2016:

28 20 00
Video Surveillance


28 2x xx
Surveillance Cameras


28 2x xx
IP Cameras

Related Requirements:

MasterFormat 2014:


27 20 00 
Data Communications 


28 23 13 
Video Surveillance Control and Management Systems


28 23 16 
Video Surveillance Monitoring and Supervisory Interfaces


28 23 19 
Digital Video Recorders and Analog Recording Devices


28 23 23 
Video Surveillance Systems Infrastructure
MasterFormat 2016


27 15 01.xx
Video Surveillance Communications Conductors and Cables 

27 20 00 
Data Communications 

28 05 xx.xx
PoE Power Sources for Electronic Safety and Security 

28 05 xx
Storage Appliances for Electronic Safety and Security 

28 05 xx.xx
Network Video Recorders 

28 05 xx
Cyber Requirements for Electronic Safety and Security


28 05 xx
Safety and Security Network Communications Equipment 

28 2x 00
Video Management System
2MP ANALOG BOX CAMERA

PART  1   GENERAL

1.01 SUMMARY
A. Section includes 2MP analog camer. 
B. Product - A 1080p analog camera in a box type body.
C. Related Requirements
Refer to MasterFormat notes at the beginning of this document to select requirements specific to the MasterFormat version being used in the specification.

1.02 REFERENCES
A. Abbreviations
1. AGC

Auto Gain Control

2. AES

Advanced Encryption Standard

3. API

Application Programming Interface
4. ARP

Address Resolution Protocol

5. AWB

Auto White Balance

6. BLC

Back light compensation

7. CBR

Constant Bit Rate

8. CVBS

Composite Video Blanking and Sync

9. DHCP

Dynamic Host Configuration Protocol
10. DNR

Digital Noise Reduction
11. DNS

Domain Name Server
12. DDNS

Dynamic Domain Name Server
13. DSCP

Differentiated Services Code Point
14. fps

frames per second
15. FTP

File Transfer Protocol
16. GOV

Group of Video
17. GUI

Graphical User Interface
18. HD

High Definition
19. HTTP

Hypertext Transfer Protocol
20. HTTPS

Secure HTTP
21. ICMP

Internet Control Message Protocol
22. IGMP

Internet Group Management Protocol
23. IP

Internet Protocol
24. IR

Infrared
25. JPEG

Joint Photographic Experts Group
26. LAN 

Local Area Network
27. LED

Light Emitting Diode
28. LDC

Lens Distortion Correction
29. LPR

License Plate Recognition
30. MJPEG

Motion JPEG
31. MP

Megapixel
32. MPEG

Moving Pictures Experts Group
33. NAS

Network Attached Storage
34. NTP

Network Time Protocol
35. NVR

Network Video Recorder
36. PIM-SM
Protocol Independent Multicast-Sparse Mode

37. PoE

Power over Ethernet
38. PPPoE

Point to Point Protocol over Ethernet
39. QoS

Quality of Service
40. RTP

Real-Time Transport Protocol
41. RTCP

Real-Time Control Protocol
42. RTSP

Real-Time Streaming Protocol
43. SDK

Software Development Kit
44. SFP

Small Form factor Pluggable
45. SMTP

Simple Mail Transfer Protocol
46. SNMP

Simple Network Management Protocol
47. SSDR

Super Smart Dynamic Range
48. SSNR

Super Smart Noise Reduction
49. SSL

Secure Sockets Layer
50. TCP

Transmission Control Protocol
51. UDP

User Datagram Protocol
52. UPnP

Universal Plug and Play
53. VBR

Variable Bit Rate
54. VMS

Video Management System
55. WDR

Wide Dynamic Range
B. Reference Standards
1. Emissions
a. FCC-47 CFR Part 15 Subpart B ANSI C63.4-2014, Class A
b. CE EN 55022:2010
c. IC Regulation ICES-003:2016 CAN/CSA CISPR 22-10 Class A
d. AS/NZS CISPR 32:2013 Class A
e. VCCI V-3/2015.04 Class A
2. Immunity - CE

a. EN 55032:2012/AC:2013 Class A

b. EN 50130-4:2011 +A1:2014
3. Safety

a. UL listed

1.03 SUBMITTALS
A. Product Data
1. Manufacturer’s printed or electronic data sheets
2. Manufacturer’s installation and operation manuals
3. Warranty documentation

1.04 QUALIFICATIONS
A. Manufacturer shall have a minimum of five years’ experience in producing IP video equipment.
B. Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.
1.05 DELIVERY, STORAGE AND HANDLING

A. Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact. 
B. Store the camera in a temperature environment specified in section 2.03 Detailed Specification, protected from mechanical and environmental conditions as designated by the manufacturer.
1.06 WARRANTY, LICENSING AND SUPPORT
A. Manufacturer shall provide at least a limited 3 year warranty for the product to be free of defects in material and workmanship.
B. Manufacturer shall provide video analytics embedded in the camera free of license charges.
END OF SECTION

PART  2  PRODUCTS

2.01 EQUIPMENT
A. Manufacturer: 
Hanwha Techwin 
http://www.hanwha-security.com/
B. Model

HCB-6000
C. Alternates: 

None

2.02 GENERAL DESCRIPTION

A. Video – 2MP (1920x1080) resolution at 30 fps(N), 25 fps(P)
B. Camera – The 2MP analog camera device shall have the following physical and performance properties:
1. Auto day/night operation with removable IR cut filter
a. Low light level operation to 0.09 lux (color) and 0.009 lux (black and white)

2. Digital Wide Dynamic Range, providing a wide range between dark and light areas visible at the same time
3. Digital noise reduction

4. Digital zoom

5. 8 privacy masking regions utilizing a 4 point polygon
6. Digital image stabilization
C. Intelligence and Analytics – The camera shall have a suite of integral intelligent operations and analytic functions to include:

1. Motion detection with four definable detection areas, minimum / maximum object size definition and a learning algorithm that ignores false alarms such as trees and waves on water.
2.03 DETAILED SPECIFICATIONS

A. Video
1. Imager
a. Sensor:


1/2.8" 2 MP CMOS
1.) pixels per sensor:
1,945 (H) x 1,109 (V) total; 1,945 (H) x 1,097 (V) effective
2.) scanning
:
progressive
b. Minimum illumination

1.) Color mode: 

0.09Lux (F1.2, 1/30sec)
2.) Black & white mode: 
0.009Lux (F1.2, 1/30sec)
c. Scanning System:

Progressive Scan
d. Signal: Noise ratio:

52 dB
e. Video Output:

BNC(AHD, CVBS, TVI, CVI selectable)
f. Resolution


1920 x 1080
g. Maximum Frame rate
30fps at 1920x1080
h. The following features with control settings shall be available:

1.) On Screen Display


English, Spanish, French,  

Portuguese, German, Italian, Russian, Polish, Czech, Romanian, Serbian, Swedish, Danish, Turkish
2.) Camera Title: 



On / Off (up to 15 characters)
3.) Day / Night



Auto (ICR) / Color / BW
4.) Backlight compensation (BLC):

Off / User BLC / HLC
5.) Wide Dynamic Range


120dB
6.) Digital Noise Reduction (DNR):

On / Off (SSNR IV)
7.) Motion detection
a.) 4 zones with 4 sided polygon

b.) selectable for On/Off
8.) Privacy masking



a.)
8 zones with 4 sided polygon

b.)
selectable for On/Off
9.) Gain control



4 levels or Off
10.) White Balance



ATW / Outdoor / Indoor / Manual / AWC 
(1,800K° ~ 10,500K°)
11.) Electronic shutter speed: 

NTSC:1 - 1/12,000 sec
12.) Reverse:



Off / H-Rev / V-Rev / HV-Rev
i. Lens:




Not included
1.) Mount Type



C / CS
2. Video Transmission Distance: 
500m (1640.42 ft.) (5C2V coaxial cable)
B. Protocol
1. Remote Control Interface: 
Coaxial control
2. Protocols:


AHD:ACP(AHD Coax Protocol), CVBS:Pelco-C(Coaxitron)
                                                    CCP(CVI Coax Protocol), TCP(TVI Coax Protocol)
3. Serial Interface


RS-485(Samsung-T, Pelco-D/P, Panasonic, Bosch, AD, GE, 
                                                    Vicon, Honeywell)
C. Alarm



(MD) output 1
D. Electrical
a. Input Voltage and Current
Dual (24V AC & 12V DC +-10%)
b. Power Consumption
Maximum 3.5W
E. Mechanical And Environmental
1. Material: 
a. Housing:


Plastic
2. Dimensions (W x H x D):
68.4 x 59.6 x 92.7 mm (2.69 x 2.35 x 3.65 in.)
3. Weight:



186g (0.41 lb.)
4. Operating Temperature: 
-10° C to 55° C (14° F to 131° F)
5. Humidity:


0 - 90%, non-condensing 

END OF SECTION
PART  3    EXECUTION
3.01 INSTALLERS
A. Contractor personnel shall comply with all applicable state and local licensing requirements.
3.02 PREPARATION

A. The design and configuration shall be verified for compatibility and performance with the camera(s). 
B. Configuration shall be tested and qualified by the Contractor prior to camera installation.

C. All firmware found in products shall be the latest and most up-to-date provided by the manufacturer, or of a version as specified by the provider of the Video Management Application (VMA) or Digital Video Recorder (DVR). 

3.03 INSTALLATION
A. The Contractor shall carefully follow instructions in documentation provided by the manufacturer to insure all steps have been taken to provide a reliable, easy-to-operate system.

B. All equipment shall be tested and configured in accordance with instructions provided by the manufacturer prior to installation.

C. Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment.
3.04 STORAGE

A. The camera hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.
END OF SECTION

HCB-6000

2MP ANALOG BOX TYPE CAMERA
Aug 2020

Page 8

