NK-1200

Architectural and Engineering Specifications

Version 1.0

(Jul. 2, 2018)

PART 2 - PRODCUTS

Division 28 - Electric Safety and Security

Level 1 - 28.05.00 - Common Work Result for Electronic Safety and Security

Level 2 - 28.05.31 - Communications Equipment for Electronic Safety and Security

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 463-400, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

General

Product Description

NK-1200 is a Network Keyboard designed and manufactured by IDIS. This product can remote control IDIS Recorder / Network Camera / VMS. It communicates with the equipment via network connection or Serial port and supports two-way audio communication.

Firmware upgrade is available through USB and network, and supports PoE (IEEE 802.3af class 0).

General Specification

- The NK-1200 shall support Remote control of IDIS Recorder/Network Camera/VMS.

- The NK-1200 shall support USB mouse control.

- The NK-1200 shall support Two-way audio communication

- The NK-1200 shall support Convenient firmware upgrades via USB port, Ethernet.

- The NK-1200 shall support Firmware duplication and auto-recovery functions to enhance system stability.

- The NK-1200 shall support Management of multiple network keyboards via network connection.

- Programmable by using the LCD screen.

- Support PoE (IEEE 802.3af class 0), 10/100/1000 Ethernet (RJ45)

Technical Specification

  - Hardware Specification

Hardware Specifications

- Interface :

  - 2 x RS485(Terminal block), 10/100/1000 Ethernet(PoE IEEE 802.3af)

- Baud Rate : 300bps ~ 115,200bps

- Display : LCD (2 line)

- USB : 1 x USB 2.0, 1 X USB 3.0

- Audio in/Out : 1(3.5mm) / 1(3.5mm)

- Component : Joystick + Main Control Board

- S/W Upgrade : USB, Network

- Setup Program : IDIS Discovery

  - Other Specifications

- Dimension (W X H X D) : 376.9mm x 95mm x 177.6mm (15.0" x 3.7" x 7.0")

- Unit Weight(Kg) : '1.2kg (2.64lb)

- Operating Temperature : '0°C  ~ 40°C ('32℉ ~ 104℉)

- Power Input : DC 48V, 1A, PoE(IEEE 802.3af class 0)

- Approvals : FCC, CE, KC

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.0 | Roy Lee | July, 02, 2018 | Initial Release |