GUIDE SPECIFICATION

MasterFormat 2014: Section 28 23 29

MasterFormat 2016: Section 28 2x xx

[image: image1.png]WiseNeT





Hanwha Techwin is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit  www.hanwha-security.com/
49 CH NETWORK VIDEO DECODER
DIVISION 28 – ELECTRONIC SAFETY AND SECURITY
Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.
2.  Explanatory notes and comments are presented in colored text.

Important:  See further notes on the following page.

Important Note to Security Systems Specifiers

CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

Primary Specification Area:

MasterFormat 2014:

28 23 19 
Electronic Surveillance

28 23 00 
Video Surveillance




28 23 19 Digital Video Recorders and Analog Recording Devices

MasterFormat 2016:

28 05 00
Common Work Results


28 05 xx
Storage Appliances for Electronic Safety and Security 



28 05 xx.xx
Network Video Recorders 
Related Requirements:

MasterFormat 2014:


27 20 00 
Data Communications 


28 23 13 
Video Surveillance Control and Management Systems


28 23 16 
Video Surveillance Monitoring and Supervisory Interfaces

28 23 19 
Video Surveillance Remote Devices and Sensors

28 23 23 
Video Surveillance Systems Infrastructure
MasterFormat 2016


27 15 01.xx
Video Surveillance Communications Conductors and Cables 

27 20 00 
Data Communications 

28 05 xx.xx
PoE Power Sources for Electronic Safety and Security 

28 05 xx
Cyber Requirements for Electronic Safety and Security


28 05 xx
Safety and Security Network Communications Equipment 

28 2x xx
IP Cameras


28 2x 00
Video Management System
49 CH NETWORK VIDEO DECODER
PART  1   GENERAL

1.01 SUMMARY
A. Section includes a 49 channel network video decoder. 
B. Product – A video decoder capable of displaying up to 49 cameras across HDMI, VGA, & BNC monitor outputs. 
C. Related Requirements

Refer to MasterFormat notes at the beginning of this document to select requirements specific to the MasterFormat version being used in the specification.

1.02 REFERENCES
A. Abbreviations
1. ARP – Address Resolution Protocol
2. DHCP - Dynamic Host Configuration Protocol
3. DNR – Digital Noise Reduction
4. DDNS – Dynamic Domain Name Server
5. fps - frames per second
6. GUI – Graphical User Interface
7. HDD – Hard Disk Drive
8. HTTP - Hypertext Transfer Protocol
9. ICMP – Internet Control Message Protocol
10. IGMP - Internet Group Management Protocol
11. IP - Internet Protocol
12. iSCSI – Internet Small Computer System Interface
13. JBOD – Just a Bunch of Disks
14. JPEG - Joint Photographic Experts Group
15. MJPEG - Motion JPEG
16. MP - Megapixel
17. MPEG - Moving Pictures Experts Group
18. NAS – Network Attached Storage
19. NTP - Network Time Protocol
20. POS – Point of Sale
21. PPPoE – Pont to Point Protocol over Ethernet
22. RAID – Redundant Array of Independent Disks (Drives)
23. RTP - Real-Time Transport Protocol
24. RTCP – Real-Time Control Protocol
25. RTSP - Real-Time Streaming Protocol
26. SMTP - Simple Mail Transfer Protocol
27. SNMP – Simple Network Management Protocol
28. SSL – Secure Sockets Layer
29. TCP - Transmission Control Protocol
30. UDP - User Datagram Protocol
31. UPnP – Universal Plug and Play
32. VMS - Video Management System
33. PoS – Point of Sales
34. VA – Video Analytics
35. PnP – Plug and Play
36. ARB – Auto Recovery Backup
37. NVR – Network Video Recorder
38. RAID – Redundant Array of Independent Disks
B. Reference Standards
1. Network - IEEE
a. 802.3 Ethernet Standards
2. Video

a. ISO / IEC 23008-2:2013, MPEG-H Part2 (ITU H.265, HEVC)

b. ISO / IEC 14496–10, MPEG-4 Part 10 ( ITU H.264)

c. ISO / IEC 10918 – JPEG 
3. Emissions
a. FCC-47 CFR Part 15 Subpart B Class A
b. EN 55022:2010, Class A
4. Immunity
a. EN 50130-4:2011 

b. EN 61000-3-2:2006+A2:2010

c. EN 61000-3-3:2008
5. Safety
a. UL listed

b. CE EN 50581:2012 (hazardous substances)

C. Definitions

1. JBOD - a collection of hard disks that have not been configured to act as a redundant array of independent disks (RAID) array.
2. GOV (Group of Video object planes) - A set of video frames for H.264 and H.265 compression, indicating a collection of frames from the initial I-Frame (key frame) to the next I-Frame. GOV consists of 2 kinds of frames: I-Frame and P-Frame.
3. WiseStream – Smart Codec that controls quantization parameter in H.265 and H.264 to efficiently manage bitrate of the video stream and reduce the storage required. 
4. Dynamic GOV – Dynamic assignment of GOV length based on the complexity of the scene to efficiently manage bitrate of the video stream and reduce the storage required. 
5. Dynamic fps - Dynamic assignment of frames per second based on the complexity of the scene to efficiently manage bitrate of the video stream and reduce the storage required.
6. ARB (Auto Recovery Backup) – Automatic backup mechanism that enables WiseNet cameras to store videos on to SD card during failures and stream it to the storage device after recovery.  
7. Failover – A feature that automatically switches to a redundant or standby device upon failure or unexpected shutdown of an active device. 
1.03 SUBMITTALS
A. Product Data
1. Manufacturer’s printed or electronic data sheets
2. Manufacturer’s installation and operation manuals
3. Warranty documentation

1.04 QUALIFICATIONS
A. Manufacturer shall have a minimum of five years’ experience in producing IP video equipment.
B. Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.
1.05 DELIVERY, STORAGE AND HANDLING

A. Deliver the product in the manufacturer’s original, unopened, undamaged container with identification labels intact.
B. Store the product in a temperature environment specified in section 2.04 Detailed Specification, protected from mechanical and environmental conditions as designated by the manufacturer.
1.06 WARRANTY, LICENSING AND SUPPORT
A. Manufacturer shall provide a limited 3 year warranty for the product to be free of defects in material and workmanship.
END OF SECTION
PART  2  PRODUCTS

2.01 EQUIPMENT
A. Manufacturer: 
Hanwha Techwin 

http://www.hanwha-security.com/

B. Model

SPD-150
C. Alternates: 

None

2.02 GENERAL DESCRIPTION

A. The 49 channel video decoder shall be able to display video from up to 64 network cameras. The available decoding CODEC formats are as below. 
1. H.265
2. H.264
3. MJPEG
B. The decoder shall provide the following ports for simultaneous display.
1. HDMI
2. VGA
3. CVBS 
C. The maximum resolution of video from HDMI port shall be up to 4K and VGA up to 1080p. The available resolutions are the following. 
1. 12MP
2. 8MP
3. 5MP
4. 3MP
5. 1080p
6. 720p
7. 4CIF
D. The decoder shall provide hallway view modes for hallway view cameras. 
E. The decoder shall allow set up and display of live video channels in the following layouts or sequence configurations.
1. HDMI:
1 / 4 / 6 / 8 / 9 / 13 / 16 / 25 / 32 / 36 / 2V / 3V

2. VGA:
1 / 4 / 6 / 8 / 9 / 13 / 16
F. The decoder shall provide AAC audio codec for capturing sound in high frequency that is suitable for sound classifications. 
G. The decoder device provides a graphical user interface available locally for all configuration.
H. The device shall support 12v DC & PoE power input sources.
2.03 DETAILED SPECIFICATIONS

A. Operating System:
Linux
B. Output
1. HDMI


1ea (up to 4K resolution)
2. VGA


1ea (DB15)
3. CVBS


1ea (BNC)
C. External
1. Ethernet

1ea (10/100/1000 BASE-T)
2. USB 


USB 2.0 2ea

3. Audio out 

1 out



D. Power


DC 12V, PoE
E. Temperature 

+0°C ~ +40°C / 0% ~ 90% RH

F. Video

1. Codec 


H.265 / H.264 / MJPEG

2. Max. Channel 

49@D1

G. Max. Decoding

30fps@8MP, up-to 12MP

H. Division 
1. HDMI


1 / 4 / 6 / 8 / 9 / 13 / 16 / 25 / 36 (32) / 2V / 3V

2. VGA 


1 / 4 / 6 / 8 / 9 / 13 / 16

3. Layout 


Up-to 20

4. Audio Codec 

G.711 / G.726 / AAC

I. Management 

Local GUI

J. Time Setting 

NTP / Manual

END OF SECTION
PART  3    EXECUTION
3.01 INSTALLERS
A. Contractor personnel shall comply with all applicable state and local licensing requirements.
3.02 PREPARATION

A. The network design and configuration shall be verified for compatibility and performance with the camera(s). 
B. Network configuration shall be tested and qualified by the Contractor prior to camera installation.

C. All equipment shall be tested and configured in accordance with instructions provided by the manufacturer prior to installation
D. All firmware found in products shall be the latest and most up-to-date provided by the manufacturer, or of a version as specified by the provider of the Video Management Application (VMA).
E. All equipment requiring users to log on using a password shall be configured with user/site-specific password/passwords. No system/product default passwords shall be allowed.
3.03 INSTALLATION
A. The Contractor shall carefully follow instructions in documentation provided by the manufacturer to insure all steps have been taken to provide a reliable, easy-to-operate system.

B. Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment. 

3.04 STORAGE

A. The hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.
END OF SECTION

SPD-150

49 CH NETWORK VIDEO DECODER
October 2019

Page 6

