TC-D5532WRX

Architectural and Engineering Specifications

Version v1.0

(Jul. 27. 2022)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Section 28 23 29 – Video Surveillance Remote Devices and Sensors

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 463-400, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

General

Product Description

TC-D5532WRX is a HD-TVI Dome Camera designed and manufactured by IDIS. This camera provides 5M (2592 X 1944) resolution. Also, this camera is equipped with Motorized Vari-focal lens. In addition, supports for progressive scan CMOS, privacy masking and motion detection are provided. This camera supports standard communication protocols and it provides OSD which is available in various languages. This camera is equipped with Day&Night, IR LED and 12VDC, Vandal-proof dome enclosure design (IK10) and IP67 rated.

General Specification

- The HD-TVI camera shall be equipped with 1/2.8” CMOS Sensor.

- The HD-TVI camera shall be equipped with 3.0mm~13.5mm Motorized Vari-Focal lens, F1.4 – 3.2.

- The HD-TVI camera shall be a Day&Night camera with a mechanical filter for low light performance. The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The HD-TVI camera shall have WDR(Wide Dynamic Range) compensation for improved video quality in high-contrast situations. So even too light images can be seen to make it dimmed. This camera shall support True WDR.

- The HD-TVI camera shall be equipped with 4ea Infrared LEDs with range up to 30m (98.4ft).

- The HD-TVI camera shall utilize configurable Dynamic Noise Reduction technology to reduce the bitrate and storage requirements by removing noise artifacts.

- The HD-TVI camera shall be vandal proof (IK10) and IP rating 67 complied dome enclosure design.

- The HD-TVI camera shall be equipped with Coaxial cable connection.

- The HD-TVI camera shall use 12VDC / 24VAC power input to supply power to the camera.

- The HD-TVI camera shall have 1 HD BNC (Signal Switchable) video out feature.

- The HD-TVI camera shall support De-fog feature mode. It is the image compensation feature for the clear video in the mist or fog environment.

- The HD-TVI camera shall support Motion Detection functionality (4 Zone).

- The HD-TVI camera shall support OSD(On-Screen Display) Control over Coax. The control UI can be seen On-Screen Display and it goes through the coaxial cable connection.

- The HD-TVI camera shall support HLC with 1~20 level adjustable. It makes stable video quality with preventing specific back light source from images.

Technical Specification

Video Specification

- Image Sensor: 1/2.8” CMOS Sensor

- Effective Pixels: 2616(H) X 1964(V)

- Maximum Resolution: 2592 x 1944

- Scanning Mode: Progressive Scan

- Frame Rate

  - TVI : 20ips@5MP, 12.5ips@5MP, 30ips@4MP, 30ips@2MP

  - AHD : 20ips@5MP, 12.5ips@5MP, 30ips@4MP, 30ips@2MP

  - CVBS : NTSC / PAL(for previewing video)

- Lens Type: Motorized Vari-focal (f=3.0mm~13.5mm)

- Iris Control: DC IRIS

- Angular Field of View

  - Wide : 91°(H), 66°(V), 120°(D)

  - Tele : 31°(H), 23°(V),38°(D)

- Pan / Tilt Range :

  - Pan : -175° ~ 175°

  - Tilt : 0° ~ 67°

  - Rotate: -177° ~ 177°

- Minimum Illumination:

  - Color: 0.13 Lux @ F1.4

  - B/W: 0 Lux (IR LED On)

- S/N Ratio: more than 50dB

- Electronics Shutter Speed: Auto, Manual (1/30, 1/25 Sec ~ 1/30000 Sec) and Flicker Mode

- Day & Night: Yes (ICR)

- IR Distance (LEDs): 30m / 98.4ft. (4ea)

- Video Output: 1 HD BNC (Signal Switchable)

Functional Specification

- AGC(Automatic Gain Control): 0~10 Level Adjustable

- Brightness: 0~20 Level Adjustable

- Sens-Up: Off / x2 ~ x32

- WDR : Selectable Options are OFF / FRAME / LINE (LOW, MIDDLE, HIGH) /ROI Mode Available

- BLC(Back Light Compensation ): Off / On

- HLC: 1~20 Level Adjustable (All Day / Night Only)

- Motion Detection: Off / On (4 Zone)

- Privacy Masking: Off / On (24 Zone _ BOX: 16 & POLYGON: 8)

- Day&Night: Color/B&W/EXTERN

- Color: Set as during day time

- B&W: Set as during night time

- Extern: Day&Night mode changed according to external intensity sensor of illumination

- Smart-IR (IR Optimizer): 0~20 Level Adjustable

- White Balance: AUTO / AUTOext / AWC-SET / MANUAL (3,000K, 5,000K, 8,000K)

- DNR: configurable OFF/LOW/MIDDLE/HIGH

- Mirror: Mirror / Flip

- Digital Zoom(E-Zoom) : x1 ~ x16

- Sharpness: 0~10 Level Adjustable

- De-fog: Off / On

- GAMMA Correction: r = 0.45 ~ 0.75

Environmental Specification

- Out-door Ready: IP67

- Vandal-proof Enclosure: IK10

- Operating Environment

  - Temperature: -10°C ~ +55°C (+14°F ~ +131°F)

  - Humidity: 0% to 90%

- Storage Environment

  - Temperature: -20°C ~ +60°C (-4°F ~ +140°F)

  - Humidity: 0% to 95%

Electrical Specification

- Power Source: DC 12V, AC 24V

- Power Consumption: DC 12V/8W, AC 24V/8.3W

Mechanical Specification

- Dimensions (W x H x D): Ø155mm x 105mm (Ø6.10" x 4.13")

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.0 | TS Team | Jul. 27. 2022 | Initial release |

Image processing technology to prevent the blurred image in low light conditions. It allows that user can select digital slow shutter for gaining extra light to the camera and get higher sensitivity in dark conditions.