Hanwha Techwin is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://www.hanwha-security.com/

2M 32x NETWORK POSITIONING CAMERA

# DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented in colored text.

Important:  See further notes on the following page.

# Important Note to Security Systems Specifiers

# CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

# Primary Specification Area:

# MasterFormat 2014:

28 20 00 	Electronic Surveillance

28 23 00 	Video Surveillance

28 23 29	Video Surveillance Remote Devices and Sensors

# MasterFormat 2016:

# 28 20 00	Video Surveillance

# 28 2x xx	Surveillance Cameras

# 28 2x xx	IP Cameras

# Related Requirements:

# MasterFormat 2014:

# 27 20 00 Data Communications

# 28 23 13 Video Surveillance Control and Management Systems

# 28 23 16 Video Surveillance Monitoring and Supervisory Interfaces

# 28 23 19 Digital Video Recorders and Analog Recording Devices

# 28 23 23 Video Surveillance Systems Infrastructure

# MasterFormat 2016

# 27 15 01.xx	Video Surveillance Communications Conductors and Cables

# 27 20 00 	Data Communications

# 28 05 xx.xx	PoE Power Sources for Electronic Safety and Security

# 28 05 xx	Storage Appliances for Electronic Safety and Security

# 28 05 xx.xx	Network Video Recorders

# 28 05 xx	Cyber Requirements for Electronic Safety and Security

# 28 05 xx	Safety and Security Network Communications Equipment

28 2x 00	Video Management System

2M 32x NETWORK POSITIONING CAMERA

- GENERAL

  - SUMMARY

    - Section includes a 2M IP camera with positioning system.

    - Product - A 2M IP camera, with multi-streaming (H.264 and MJPEG) capability in IP66 housing.

    - Related Requirements

Refer to MasterFormat notes at the beginning of this document to select requirements specific to the MasterFormat version being used in the specification.

  - REFERENCES

    - Abbreviations

      - AGC	Auto Gain Control

      - ARP	Address Resolution Protocol

      - AWB	Auto White Balance

      - BLC	Back light compensation

      - CBR	Constant Bit Rate

      - CVBS	Composite Video Blanking Sync

      - DHCP	Dynamic Host Configuration Protocol

      - DNR	Digital Noise Reduction

      - DNS	Domain Name Server

      - DDNS	Dynamic Domain Name Server

      - DSCP	Differentiated Services Code Point

      - fps	frames per second

      - FTP	File Transfer Protocol

      - GOV	Group of Video

      - GUI	Graphical User Interface

      - HD	High Definition

      - HTTP	Hyper Text Transfer Protocol

      - ICMP	Internet Control Message Protocol

      - IGMP	Internet Group Management Protocol

      - IP	Internet Protocol

      - JPEG	Joint Photographic Experts Group

      - MJPEG	Motion JPEG

      - MP	Mega Pixel

      - MPEG	Moving Pictures Experts Group

      - NAS	Network Attached Storage

      - NTP	Network Time Protocol

      - PIM-SM Protocol Independent Multicast-Sparse Mode

      - PoE	Power over Ethernet

      - PPPoE	Point to Point Protocol over Ethernet

      - RTP	Real-time Transport Protocol

      - RTCP	Real-Time Control Protocol

      - RTSP	Real-Time Streaming Protocol

      - SDK	Software Development Kit

      - SMTP	Simple Mail Transfer Protocol

      - SNMP	Simple Network Management Protocol

      - SSL	Secure Sockets Layer

      - TCP	Transmission Control Protocol

      - UDP	User Datagram Protocol

      - UPnP	Universal Plug and Play

      - VBR	Variable Bit Rate

      - VMS	Video Management System

      - WDR	Wide Dynamic Range

      - LDC	Lens Distortion Correction

    - Reference Standards

      - Network - IEEE

        - 802.3 Ethernet Standards

        - 802.1x  Port-based Network Access Control

      - Video

        - ISO / IEC 23008-2:2013, MPEG-H Part2 (ITU H.265, HEVC)

        - ISO / IEC 14496–10, MPEG-4 Part 10 ( ITU H.264)

        - ISO / IEC 10918 – JPEG

        - ONVIF – Profiles S

      - Emissions

        - FCC-47 CFR Part 15 Subpart B Class B

        - CE EN 55022:2010

      - Immunity - CE

        - EN 50130-4:2011

        - EN 61000-3-3:2014

        - EN 61000-4-2:2009

        - EN 61000-4-3:2006+A2:2010

        - EN 61000-4-4:2012

        - EN 61000-4-5:2014

        - EN 61000-4-6:2009

      - Safety

        - UL listed

        - CE EN 50581:2012 (hazardous substances)

    - Definitions

      - GOV (Group of Video object planes) - A set of video frames for H.264 compression, indicating a collection of frames from the initial I-Frame (key frame) to the next I-Frame. GOV consists of two kinds of frames in video surveillance setup: I-Frame and P-Frame.

      - Dynamic GOV – Dynamic assignment of GOV length based on the complexity of the scene to efficiently manage bitrate of the video stream and reduce the storage required.

      - Multi-exposure wide dynamic range - Operation which automatically adjusts shutter speed to provide a wide range between dark and light areas visible at the same time, preventing backlighting issues. Long exposure is used for bright areas and a short exposure is used in dark areas.

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic data sheets

      - Manufacturer’s installation and operation manuals

      - Warranty documentation

  - QUALIFICATIONS

    - Manufacturer shall have a minimum of five years’ experience in producing IP video equipment.

    - Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

    - Store the camera in a temperature environment indicated in 2.04 Detailed Specification, protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY, LICENSING AND SUPPORT

    - Manufacturer shall provide a limited 3 year warranty for the product to be free of defects in material and workmanship.

    - Manufacturer shall provide embedded camera video analytics free of license charges.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Hanwha Techwin

http://www.hanwha-security.com/

    - Model		TNU-6320

    - Alternates: 		None

  - GENERAL DESCRIPTION

    - The positioning camera shall have a metal housing. The camera shall also have PTZ capability.

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously

        - H.264 –60 fps at all resolutions

        - MJPEG – 1920x1080 / 1280x1024 /1280x960 / 1280x720 / 1024x768 : Max. 15 fps

800x600 / 800x450 / 720x576 / 640x480 / 640x360 / 320x240 / 320x180 : Max. 30fps

      - The camera shall be able to configure up to 10 independent video stream profiles with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - The camera shall be able to stream up to three different profiles.

      - Resolution selections

        - 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768,

800 x 600, 800 x 450, 720 x 576, 640 x 480, 640 x 360, 320 x 240, 320 x 180

      - Simultaneous unicast access by up to 10 users

      - Multicast or unicast capable

      - Dynamic DNS (DDNS) support.

    - Camera – The camera device shall have the following physical and performance properties:

      - Low light level operation to 0.3 lux (F1.6, color) and 0.03 lux (F1.6, black and white)

      - 32x(4.44~142.6mm) optical and 16x digital zoom

      - Defog feature to remove fogginess of scene.

      - Dust proof and water proof (IP66 rated)

      - Auto day/night operation with removable IR cut filter

      - 2D and 3D digital noise reduction

      - True WDR up to 120dB

    - Intelligence and Analytics – The camera shall have a suite of integral intelligent operations and analytic functions to include:

      - Motion detection with four definable detection areas, minimum / maximum object size definition and a learning algorithm that ignores false alarms such as trees and waves on water.

      - Detection of logical events of specified conditions from the camera’s video input

        - camera tamper (scene change)

        - face detection

        - virtual line

        - enter/exit

        - appear/disappear

    - Interoperability – The camera shall be ONVIF Profile S compliant.

    - The camera shall possess the following further characteristics:

      - Built-in web server, accessed via standard browsers including Internet Explorer, Firefox, Chrome & Safari

      - Micro SD/SDHC/SDXC memory card with configurable pre-alarm and post-alarm recording intervals

      - NAS recording option with configurable pre-alarm and post-alarm recording intervals

      - Alarms and notifications

        - alarm notification triggers:

          - alarm input

          - motion detection

          - video analytics

          - network disconnect

          - tampering

          - face detection

        - available notification means upon trigger:

          - file upload via FTP and e-mail

          - notification via e-mail

          - Local storage(SD/SDHC/SDXC)

          - NAS recording at event triggers

          - external output

  - CAMERA SOFTWARE

    - The camera shall have a built in web server which supports browser-based configuration using Internet Explorer, Google Chrome, Mozilla Firefox, and Apple Safari, for which web viewer plug-ins are available, from a PC or Mac.

    - The web viewer shall provide a monitoring screen which displays live camera video and simultaneously provides same-screen access to the following functions:

      - Live view window size

      - Resolution setting

      - Image (snapshot) capture

      - Manual recording to SD or NAS

      - Access Playback and Setup menus

    - The web viewer shall provide a playback screen which provides access to the following functions:

      - Search date and time range

      - Search event type

      - Play an event video

      - Set resolution

      - Play audio if present

      - Generate a backup copy of saved video data

    - The web viewer shall provide a setup screen which provides access to the following configuration settings and functions in the camera:

      - Digital video profile to include compression type, maximum or target bit rate, frame rate, multicast parameters, crop encoding area

      - User profile to include password, access level, authentication

      - Date and time

      - Network settings and IP version

        - DDNS

        - SSL, including certificate management

        - 802.1x authentication

        - Quality of Service settings

        - SNMP to include version selection and settings

      - Auto configuration

      - Camera settings to include image preset, sensor frame capture, dynamic range, white balance, back light, exposure, day/night operation, on-screen display, sharpness, contrast, and color level.

      - Event detection setup to include notification parameters, recording rules, time schedule, tamper protection, motion detection, event triggers

      - System function to include reboot, upgrade, check system and event logs, application (SDK) management

      - View profile information

    - Client requirements

      - Acceptable Operating Systems:	Windows 7 / 8, MAC OS X 10.7

      - Acceptable browsers: 	Microsoft Internet Explorer (Ver. 8 ~ 11), Mozilla Firefox (Ver. 9 ~ 19),

Google Chrome (Ver. 15 ~ 25),

Apple Safari (Ver. 6.0.2(Mac OS X 10.8, 10.7 only), 5.1.7)

  - DETAILED SPECIFICATIONS

    - Video

      - Imager

        - Sensor:			1/2.8" 2.38M CMOS

          - pixels per sensor:	1,952(H) x 1,116(V)  total; 1,944(H) x 1,104(V) effective

          - scanning	:	progressive

        - Minimum illumination

Color: 0.3Lux (1/30sec, F1.6, 50IRE, AGC high), 0.005Lux (2sec, F1.6, 50IRE, AGC high)

B/W: 0.03Lux (1/30sec, F1.6, 50IRE, AGC high), 0.0005Lux (2sec, F1.6, 50IRE, AGC high)

Color: 0.15Lux (1/30sec, F1.6, 30IRE), 0.0025Lux (2sec, F1.6, 30IRE)

B/W: 0.015Lux (1/30sec, F1.6, 30IRE), 0.00025Lux (2sec, F1.6, 30IRE)

        - S/N Ratio			50dB

        - Video out(installation)	CVBS: 1.0 Vp-p / 75Ω composite, 720 x 480(N), 702 x 576(P)

        - The following features with control settings shall be available:

          - Camera Title 				Off / On (Displayed up to 45 characters)

          - Day/night setting:			Auto (ICR) / Color / B/W

          - Backlight compensation (BLC):		Off / BLC / WDR

          - WDR					120dB

          - Contrast Enhancement 			Off / On (Samsung Super Dynamic Range)

          - Digital Noise Reduction (DNR):		Off / On (2D+3D noise filter)

          - Defog					Off / Auto / Manual

          - Motion Detection			Off / On (4ea polygonal zones)

          - Auto Gain Control			Off / Low / Middle / High

          - White Balance 				ATW / AWC / Manual / Indoor / Outdoor

(Include mercury)

          - Electronic shutter speed:

            - settings:				Minimum / Maximum / Anti flicker

(2 ~ 1/12,000sec)

          - Digital Zoom				16x

          - Video Analytics				Tampering, Virtual line, Enter / Exit, (Dis)Appear,

Face detection

          - Alarm I/O				Input 1 / Output 1 (relay type)

          - Alarm Triggers				Motion detection, Tampering, Face detection,

Video analytics, Alarm input, Network disconnection

          - Alarm Events				File upload via FTP and E-mail, Notification via

E-mail, recording at event triggers, External output

        - Lens:			4.44 ~ 142.6 mm (optical 32x)

          - Max. Aperture Ratio	F1.6 (Wide) / F4.4 (Tele)

          - Angle of view:		H: 62.8˚(Wide) ~ 2.23˚(Tele)  / V: 36.8˚(Wide) ~ 1.26˚(Tele)

          - Min. Object Distance:	Wide: 1.5 m (4.92ft), Tele: 1.5m (4.92ft)

          - Focus Control 		Auto Focus, One-shot Auto Focus, Manual

          - Lens Type		DC Auto Iris

          - Mount Type		Board-in Type

        - Pan, Tile

          - Pan			360˚ Endless

          - Tilt			130˚ (-90˚ ~ 40˚)

          - Pan/tilt speed		0.025˚ ~ 120˚/sec (Pan), 0.025˚ ~ 40˚/sec (Tilt)

          - Sequence		Preset (255ea), Swing, Group (6ea), Trace, Tour (1ea),

Auto run, Schedule

          - Preset accuracy		0.3˚

          - Azimuth		Yes (E / W / S / N / NE / NW / SE / SW OSD)

          - IR illuminator(Optional)	Compatible with Varifocal IR illuminator

          - Wiper			support

      - Video Streams

        - The camera shall be able to produce 10 video profiles, each of which may have the following properties:

          - Encoding type:

            - H.264

            - MJPEG

          - Resolution:

            - 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768,

800 x 600, 800 x 450, 720 x 576, 640 x 480, 640 x 360, 320 x 240, 320 x 180

          - Maximum frame rate:

            - H.265 and H.264:	60 fps at all resolutions

            - MJPEG: 		Max. 15fps at 1920 x 1080, 1280 x 1024, 1280 x 960,

1280 x 720, 1024 x 768

Max. 30fps at 800 x 600, 800 x 450, 720 x 576,

640 x 480, 640 x 360, 320 x 240, 320 x 180

          - Smart Codec		Manual mode (Area-based : 5ea), face detection mode

          - Bit rate control method:

            - H.264

              - target bitrate level control

              - constant bit rate (CBR) or variable bit rate (VBR)

            - MJPEG

              - quality level control

              - variable bit rate (VBR)

      - Number of multi-streaming profiles:	10 maximum

      - Simultaneous users (total):		10 maximum (unicast)

      - Storage and Recording

        - The camera shall have onboard SD card storage

          - Card type:	Micro SD/SDHC/SDXC

          - Capacity:	128 GB

          - Image content on the card shall have the ability to be downloaded to a selected destination.

        - NAS

      - Interoperability - Video streams shall be capable of supporting ONVIF protocol, profiles S.

      - Single Images - The camera shall support image screenshot and export in jpg format.

    - Audio

      - Audio communication			Bi-directional

      - Audio compression format		G.711 u-law/G.726 selectable

G.726(ADPCM) 8KHz, G.711 8KHz

G.726: 16Kbps, 24Kbps, 32Kbps, 40Kbps

      - Audio In				Selectable(Mic in/Line in),

Supply voltage: 2.5VDC(4mA), Input impedance: approx.. 2K Ohm

      - Audio Out				Line out(Mono), Max output level:1 Vrms

    - Network

      - Connectivity: 	10/100 Base-T Ethernet via RJ-45 connector

      - Protocols supported:

        - Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

        - Configuration: Dynamic Host Configuration Protocol (DHCP)

        - Web services: Hypertext Transfer Protocol (HTTP), Secure HTTP (HTTPS)

        - Network services: Address Resolution Protocol (ARP), Bonjour, Domain Name System (DNS), Internet Control Message Protocol (ICMP), Network Time Protocol (NTP), Protocol Independent Multicast-Sparse Mode (PIM-SM), Simple Network Management Protocol (SNMP v1/2c/3 – MIB-2), Universal Plug and Play (UPnP)

        - Media: Real-Time Transport Protocol (RTP), Real-Time Control Protocol, Real-Time Streaming Protocol (RTSP)

        - Multicast: Internet Group Management Protocol (IGMP)

        - Notifications: File Transfer Protocol (FTP), Simple Mail Transfer Protocol (SMTP)

        - Remote Access: Point-to-Point Protocol over Ethernet) (PPPoE)

      - DDNS – The camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

      - Quality of Service (QoS) – Layer 3 DSCP

      - Security features:

        - user password protection

        - IP address filtering - list of allowed or blocked IP addresses

        - HTTPS(SSL) login authentication

        - HTTPS(SSL) secured communications

        - Digest login authentication

        - User access log

        - 802.1x authentication

      - Discovery - Manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.

    - Electrical

      - Power

        - Input Voltage / Current	24V AC / 6A (Max.)

        - Power Consumption:	Max. 100W (Heater on), Max. 38W (Heater off)

    - Mechanical And Environmental

        - Material: 			Aluminum

        - Color			Ivory

      - Dimensions (W x H x D):	219 x 528 x 460 mm (8.62 x 20.79 x 18.11 inch)

      - Weight 				14kg (30.86 lb.)/camera main body

      - Temperature:

        - Operating: 			-40° C to 55° C (-40° F to 131° F)

        - Storage:  			-45° C to 60° C (-49° F to 140° F)

      - Humidity:			0 - 90%, non-condensing

      - Environmental Rating:

        - Ingress Protection		IP66

      - Certification

        - Safety 			UL, KC-SDOC

        - EMC			FCC, CE, VCCI, RCM, KC

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).

    - Network configuration shall be tested and qualified by the Contractor prior to camera installation.

    - All firmware found in products shall be the latest and most up-to-date provided by the manufacturer, or of a version as specified by the provider of the Video Management Application (VMA) or Network Video Recorder (NVR).

    - All equipment requiring users to log on using a password shall be configured with user/site-specific password/passwords. No system/product default passwords shall be allowed.

  - INSTALLATION

    - The Contractor shall carefully follow instructions in documentation provided by the manufacturer to insure all steps have been taken to provide a reliable, easy-to-operate system.

    - All equipment shall be tested and configured in accordance with instructions provided by the manufacturer prior to installation.

    - Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment.

  - STORAGE

    - The hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.

END OF SECTION

8